VIII-Fuji-10-ECO (2X3 IM) SPECIFICATIONS (FRA & NS FM-Z2727RGBA)

10.417mm Permanent Outdoor SMD LED Panel

| - Parameter | Unit | Value |
| --- | --- | --- |
| Brightness | nits | 6,000 |
| Colour temperature | deg. K | 6,500 |
| Viewing angle – Horizontal (50% brightness) | deg. | 140 (+70/-70) |
| Viewing angle – Vertical (50% brightness) | deg. | 100 (+45/-55) |
| Louver material | / | PC |
| Panel weight(max) | kg / lb | 25 / 55.1 |
| Weight (max) (sq.m) | kg / lb | 25 / 55.1 |
| Panel diagonal | mm/ft. | 1,414 / 4.64 |
| Panel width | mm/ft. | 1,000 / 3.28 |
| Panel height | mm/ft. | 1,000 / 3.28 |
| Panel depth | mm/ft. | 121/ 0.397 |
| Panel area | sq. m./ sq. ft. | 1 / 10.76 |
| Panel material |  | Aluminum & Steel |
| Panel color | / | Pantone Matte Black |
| Aspect ratio |  | 1:1 |
| Ingress protection (front/rear) | IP | IP65 / IP65 |
| Operating / storage temperature | deg. C | -20 to +50/-40 to+60 |
| Operating / storage humidity | % | ≤90 (No condensation) |
| Pixel type and configuration | R/G/B | SMD 3 in 1 |
| Pixel pitch | mm | 10.417 |
| Pixel matrix per panel |  | 96 x 96 |
| Pixels per panel |  | 9,216 |
| Pixels lines per meter |  | 96 |
| Pixels per sq. meter |  | 9,216 |
| Recommended minimum viewing distance | m | 10 |
| Colours |  | 281 trillion |
| Grey scale (linear) | Levels | 65,536 levels per colour |
| Brightness control | Levels | 100 |
| Contrast Ratio |  | 1,0000:1 |
| Processing depth | bit | 16 |
| System type |  | M4 |
| Video frame rate | Hertz | 50/60 |
| Display refresh rate | Hertz | ≥7,680 |
| Input voltage (nominal) | VAC | 100 to 240 |
| Input power frequency | Hertz | 50 to 60 |
| Power Supply |  | PFC |
| Input power (max) | W | 550 |
| Lifetime (50% brightness) | Hours | ≥100,000 |
| Panel input format |  | Ethernet |
| Data interconnection |  | CAT5e (Length≤100m) Optical link (100m<Length≤5km, optional) |
| Mounting system |  | Fixed with 4pcs M10 screw |
| Panel Maintenance |  | FRA |

(DI)

* Specification subjected to change without prior notice