|  | ENVIRONMENTAL HEALTH SUPPORTING COMMUNITIES |
| --- | --- |

| To: | Brendan Versluys, Planning Officer, Development Management, 5 Pancras Square |
| --- | --- |
| From: | Edward Davis (Environmental Health Officer) |
| Date: | 09 January 2025 |
| Re: | 22 Hanway Street London W1T 1UQ |
| Proposal: | Change of use of first floor from a bar (Sui Generis) to residential use (Class C3) to provide a studio flat. |
| Reference: | Planning application ref. 2024/5078/P |
| Key Points: | The proposals are acceptable in environmental health terms |

# ENVIRONEMTAL HEALTH OBERVATIONS

An acoustic assessment has been carried out to support the planning application for the change of use of first floor from a bar to residential use to provide a studio flat.

Appropriate noise guidelines have been followed within the report such as Noise Policy Statement for England, National Planning Policy Framework (NPPF), Planning Practice Guidance on Noise, BS 8233 Guidance on sound insulation and noise reduction for buildings, Camden Council’s Local Plan, version June 2017.

The plant noise criteria have been adequately predicted  taking into consideration distance losses, surface acoustic reflections and, where applicable, screening provided by the building.

The pre-completion sound insulation tests in the First Floor, 22 Hanway Street, London indicates that the proposed scheme should be capable of achieving the Camden’s environmental noise criteria for sound insulation acceptance.

I am satisfied that the submitted acoustic submission and assocoiated technical details meets our local plan guidelines and therefore acceptable in environmental health terms.  I offer no objections to the application and suggest the following should you be mindful to grant the application.

Suggested conditions:

The sound insulation of the floor/ ceiling/ walls separating the commercial part(s) of the premises from noise sensitive premises meet a standard of sound insulation value DnT,w and L’nT,w of at least 10dB above the Building Regulations value and, where necessary, additional mitigation measures will be implemented  to contain commercial noise within the commercial premises and to achieve the criteria of BS 8233:2014 Guidance on sound insulation and noise reduction for buildings within the noise sensitive premises.  Approved details shall be implemented prior to occupation of the development and thereafter be permanently retained.

Reason: To ensure that the amenity of occupiers of the development site/ adjacent dwellings/ noise sensitive premises is not adversely affected by noise

in accordance with the requirements of policies A1 and A4 of the London Borough of Camden Local Plan 2017

Regards

Edward Davis

Pollution Planning Officer

Supporting Communities

London Borough of Camden

Telephone:   02079745550/ Mobile: 07967 652382