|  | ENVIRONMENTAL HEALTH SUPPORTING COMMUNITIES |
| --- | --- |

| To: | Josh Lawlor, Planning Officer, Development Management, 5 Pancras Square |
| --- | --- |
| From: | Edward Davis (Environmental Health Officer) |
| Date: | 23 April 2025 |
| Re: | 14 Blackburn Road London NW6 1RZ |
| Proposal: | Demolition of the existing buildings at the Site and redevelopment of the Site for a mixed-use development comprising purpose-built student accommodation (Sui Generis), affordable housing (Use Class C3), lower ground and ground floor flexible commercial/business space comprising of showrooms, retail and ancillary offices (Use Class E/Sui Generis) and a cafe/PBSA amenity space (Use Class E/Sui Generis) and associated works including service yard, cycle parking, hard and soft landscaping, amenity spaces and plant. |
| Reference: | Planning application ref. 2025/1685/P |
| Key Points: | The proposals are acceptable in environmental health terms |

# ENVIRONEMTAL HEALTH OBERVATIONS

An assessment has been carried out to quantify the levels of noise and vibration affecting the Site. The results have been used to advise the design team and client on any measures that may be needed to control noise and vibration ingress.

The application seeks ‘Redevelopment of 14 Blackburn Road, London, NW6 1RZ (‘the site’) for a mixed-use development comprising student housing, affordable C3 self-contained housing and ground floor commercial space’.

The proposed development comprised of two distinct buildings that are linked at ground level. The C3 building will be 4-7 storeys including a taller ground floor and the PBSA building will be 10 storeys including a ground floor and amenity mezzanine level. There is a double height space spanning these lower two floors in the café at the base of the PBSA.

Appropriate noise guidelines have been followed within the report such as Noise Policy Statement for England, National Planning Policy Framework (NPPF), Planning Practice Guidance on Noise, BS 8233 Guidance on sound insulation and noise reduction for buildings, Camden Council’s Local Plan, version June 2017 and BS 4142:2014 “Methods for rating and assessing industrial and commercial sound”..

We would expect to see the following noise standards to be achieved internally for the residential units:

| Activity | Location | 07:00 to 23:00 | 23:00 to 07:00 |
| --- | --- | --- | --- |
| Resting | Living Room | 35 LAeq, 16hour | - |
| Dining | Dining Room / Area | 40 LAeq, 16hour | - |
| Sleeping (daytime resting) | Bedroom | 35 LAeq, 16hour | 30 LAeq, 8hour 45 LAfmax |

- These levels are derived in part from Table 4 of BS8233:2014 and also World Health Organization figures.  The figures from BS8233:2014 are themselves derived from World Health Organization values.

- The notes to Table 4 of BS8233:2014 apply to the interpretation of the above figures.

- It is also expected that to achieve an acceptable internal noise climate that individual noise events shall not exceed 45dB LAFmax on a frequent basis. The acceptability of the frequency of events will depend on the level of exceedance of the 45dB LAFmax criteria. Up to 10 events may be acceptable for small exceedances (<5dB) whilst for high exceedences (>=5 dB and <10 dB) less than 5 events will be acceptable. Events in excess of 10 dB above 45 dB are not permitted.

The maximum day time noise level in outdoor living areas exposed to external road traffic noise should not exceed 50dBA Leq 16 hour [free field].

Good acoustic design has been considered in relation to sound insulation of the building envelope to ensure residents are adequately protected from noise.

Facade design specification criteria have been discussed and assessed in any further acoustic submission and the applicant should ensure future occupants are protected against excessive internal and external noise transfer.

The plant noise criteria will have to be adequately predicted taking into consideration distance losses, surface acoustic reflections and, where applicable, screening provided by any buildings.

I am satisfied that the initial submitted acoustic submission meets our local plan guidelines and therefore acceptable in environmental health terms.  I would be offering no objections to a full application and would suggest the following should you be mindful to grant the application.

Suggested conditions:

The design and structure of the development shall be of such a standard that it will protect residents within it from existing external noise so that they are not exposed to levels indoors of more than 35 dB LAeq 16 hrs daytime and of more than 30 dB LAeq 8 hrs in bedrooms at night.

Reason: To ensure that the amenities of future occupiers are protected.

No vibration shall be transmitted into the proposed bulding structure as to cause a vibration dose value of greater than 0.4m/s (1.75) 16 hour day-time nor 0.26 m/s (1.75) 8 hour night-time as defined by BS 6472 (2008) in any part of a residential and other noise sensitive property.

terms.  I offer no objections to the application and suggest the following should you be mindful to grant the application.

The external noise levels emitted from plant/ machinery/ equipment shall ensure that the rating level of the noise emitted from the proposed installation located at the site shall not exceed the existing background level at any noise sensitive premises when measured and corrected in accordance with BS4142:2014 +A1:2019 “Methods for rating and assessing industrial and commercial sound.”

Reason: To ensure that the amenity of occupiers of the development site/ sur-rounding premises is not adversely affected by noise from mechanical installa-tions/ equipment in accordance with the requirements of policies A1 and A4 of the London Borough of Camden Local Plan 2017

Prior to use, machinery, plant or equipment at the development shall be mounted with proprietary anti-vibration isolators and fan motors shall be vibration isolated from the casing and adequately silenced and maintained as such.

Reason: To ensure that the amenity of occupiers of the development site and sur-rounding premises is not adversely affected by vibration in accordance with the requirements of policies A1 and A4 of the London Borough of Camden Local Plan 2017

Separation of  noise sensitive rooms in neighbouring flats

Prior to commencement of the development, details shall be submitted to and approved in writing by the Council, of an enhanced  sound insulation value DnT,w [and L’nT,w] of at least 5dB above the Building Regulations value, for the floor/ceiling /wall structures separating different types of rooms/ uses in adjoining dwellings, namely [eg. living room and kitchen above bedroom of separate dwelling]. Approved details shall be implemented prior to occupation of the development and thereafter be permanently retained.

Reason: To ensure that the amenity of occupiers of the development site is not adversely affected by noise

Separation of commercial and noise sensitive premises

Prior to commencement of the development, details shall be submitted to and approved in writing by the Council, of the sound insulation of the floor/ ceiling/ walls separating the commercial part(s) of the premises from noise sensitive premises.  Details shall demonstrate that the sound insulation value DnT,w  [and L’nT,w ]  is enhanced by at least 10dB above the Building Regulations value and, where necessary, additional mitigation measures are implemented  to contain commercial noise within the commercial premises and to achieve the ‘Good’ criteria of BS8233:1999 within the dwellings/ noise sensitive premises.  Approved details shall be implemented prior to occupation of the development and thereafter be permanently retained.

Reason: To ensure that the amenity of occupiers of the development site/ adjacent dwellings/ noise sensitive premises is not adversely affected by noise

Prior to commencement of the development hereby approved, a construction management plan shall be submitted to and approved in writing by the Council. Details shall include control measures for dust, noise, vibration, lighting, delivery locations, restriction of hours of work and all associated activities audible beyond the site boundary to 07:00 – 19:00 Monday to Friday daily, 08:00 – 13:00 SaturdaysNo works to be undertaken on Sundays or bank holidays, advance notification to neighbours and other interested parties of proposed works and public display of contact details including accessible phone contact to persons responsible for the site works for the duration of the works.  Approved details shall be implemented throughout the project period.

Reason: To ensure that the amenity of occupiers of surrounding premises is not adversely affected by noise, vibration, dust, lighting or other emissions from the building site.

Regards

Edward Davis

Pollution Planning Officer

Supporting Communities

London Borough of Camden

Telephone:   02079745550/ Mobile: 07967 652382