4 - Entrances

All entrances should:

a) Be illuminated

b) Have level access over the threshold; and

c) Have effective clear opening widths and nibs as specified below.

In addition, main entrances should also:

d) Have adequate weather protection*

e) Have a level external landing.*

The entrance to the property will have adequate lighting. In addition, there is adequate street lighting on Agar Grove to give background illumination.

As for level access, there is a step at the threshold and the pavement which makes compliance with the standards difficult. It is not possible to add a canopy over the main ground floor entrance.

5a – Communal Stairs

Principal access stairs should provide easy access, regardless of whether or not a lift is provided.

5b – Communal Lifts

Where a dwelling is reached by a lift, it should be fully accessible.

There is no communal stairs.

There is no provision for a communal lift system.

6. Internal doorways and hallways

Movement in hallways and through doorways should be as convenient to the widest range of people, including those using mobility aids or wheelchairs, and those moving furniture or other objects.

As a general principle, narrower hallways and landings will need wider doorways in their side walls.

The width of doorways and hallways should conform to the specification below.

Where altered, corridor width of 900mm will be achieved. The Ground Floor area is wheelchair accessible and therefore Part M standards compliant.

7. Circulation Space

There should be space for turning a wheelchair in dining areas and living rooms and basic circulation

space for wheelchair users elsewhere.

Wheelchair access and turning and turning area has been achieved within the Ground Floor.

8. Entrance level living space

A living room / living space should be provided on the entrance level of every dwelling.

A living room has been provided on the Ground Floor dwelling.

9. Potential for entrance level bed-space

In dwellings with two or more storeys, with no permanent bedroom on the entrance level, there should be space on the entrance level that could be used as a convenient temporary bed-space.

The Ground Floor dwelling meets the criteria.

10. Entrance level WC and shower drainage

Where an accessible bathroom, in accordance with Criterion 14, is not provided on the entrance level of a dwelling, the entrance level should have an accessible WC compartment, with potential for a shower to be installed.

Provision for wheelchair access is achievable.

11 – WC and bathroom walls

Walls in all bathrooms and WC compartments should be capable of firm fixing and support for adaptations such as grab rails.

Wall reinforcements will be used where new partitions are constructed for bathrooms and toilets. Where work is required to existing partition, this will also be carried out.

12 - Stairs and potential though-floor lift in dwellings

The design within a dwelling of two or more storeys should incorporate both:

a) Potential for stair lift installation; and,

b) A suitable identified space for a through-the–floor lift from the entrance level to a storey containing a main bedroom and a bathroom.

Given the limitations of the existing property and its layout, it is not possible to comply with the criteria.

13 – Potential for future fitting of hoists and bedroom / bathroom relationship

Structure above a main bedroom and bathroom ceilings should be capable of supporting ceiling hoists and the design should provide a reasonable route between this bedroom and the bathroom.

The proposed layout meets the criteria.

14 – Bathrooms

An accessible bathroom, providing ease of access in accordance with the specification below, should be provided in every dwelling on the same storey as a main bedroom.

There are no bathrooms.

15. Glazing and window handle heights

Windows in the principal living space (typically the living room), should allow people to see out when seated. In addition, at least one opening light in each habitable room should be approachable and usable by a wide range of people – including those with restricted movement and reach.

The proposal complies with the standards.

16. Location of service controls

Service controls should be within a height band of 450mm to1200mm from the floor and at least 300mm away from any internal room corner.

All switches and sockets will be positioned within the heights of 450mm and 1200mm from the floor.