Lifetime Homes Assessment for Proposed Residential Development at 11-13 Goodge Street

| Criteria | Comments |
| --- | --- |
| 1a) Where a dwelling has car parking within its plot boundary, at least one parking space length should be capable of enlargement to achieve a minimum width of 3300mm. | N/A, no car parking spaces are provided as part of the proposed development |
| 1b) Where parking is provided by communal or shared bays, spaces with a width of 3300mm, should be provided as required. | N/A, no car parking spaces are provided as part of the proposed development |
| 2) The distance from the car parking space to the dwelling entrance (or relevant block entrance or lift core), should be kept to a minimum and be level or gently sloping. The distance from visitors parking to relevant entrances should be as short as practicable and be level or gently sloping. | N/A, no car parking spaces are provided as part of the proposed development |
| 3) The approach to all entrances should preferably be level or gently sloping | The entrance is from the existing public highway which is predominantly level along the length of Goodge Street. |
| 4) All entrances should: - Be illuminated - Have level access over the threshold - Have effective clear opening width and nib - Have adequate weather protection - Have a level external landing. | - Yes, subject to detailed design - There is a small step up to the existing doorway that will be retained - Yes, subject to detailed design - There is an internal entrance lobby - N/A |
| 5a) Principal access stairs should provide easy access | The existing main stair core will be upgraded as part of the development. |
| 5b) Where a dwelling is reached by a lift, it should be fully accessible | N/A |
| 6) Movement in hallways and through doorways should be as convenient to the widest range of people, including those using mobility aids or wheelchairs, and those moving furniture or other objects. | All corridors and doorways will be designed to support access to the widest range of different individuals. |
| 7) There should be space for turning a wheelchair in dining areas and living rooms and basic circulation space for wheelchair users elsewhere. | Adequate turning space will be available within all living areas. |
| 8) A living room / living space should be provided on the entrance level of every dwelling. | All dwellings have living space at their entry level. |
| 9) In dwellings with two or more storeys, with no permanent bedroom on the entrance level, there should be space on the entrance level that could be used as a convenient temporary bed-space. | N/A |
| 10) Where an accessible bathroom, is not provided on the entrance level of a dwelling, the entrance level should have an accessible WC compartment, with potential for a shower to be installed. | All bathrooms are provided on the entrance level of each dwelling (single storey dwellings). |
| 11) Walls in all bathrooms and WC compartments should be capable of firm fixing and support for adaptations such as grab rails. | Yes |
| 12) The design within a dwelling of two or more storeys should incorporate: - Potential for stair lift installation - A suitable identified space for a through-the–floor lift from the entrance level to a storey containing a main bedroom and a bathroom | N/A |
| 13) Structure above a main bedroom and bathroom ceilings should be capable of supporting ceiling hoists and the design should provide a reasonable route between this bedroom and the bathroom. | This could be achieved subject to detail design. |
| 14) An accessible bathroom, providing ease of access should be provided in every dwelling on the same storey as a main bedroom. | Yes, subject to detailed design |
| 15) Windows in the principal living space (typically the living room), should allow people to see out when seated. In addition, at least one opening light in each habitable room should be approachable and usable by a wide range of people – including those with restricted movement and reach. | Window specifications will be to Lifetime Homes Standards subject to any constraints due to retaining the pattern and style of existing glazing units to meet Conservation Area objectives |
| 16) Location of service controls Service controls should be within a height band of 450mm to 1200mm from the floor and at least 300mm away from any internal room corner. | Yes, subject to detailed design |