### Date first listed: 30-Jun-1969

### Date of most recent amendment: Not applicable to this List entry.

# Legacy System Information

The contents of this record have been generated from a legacy data system.

### Legacy System: LBS

### UID: 476727

# Asset Groupings

This List entry does not comprise part of an Asset Grouping. Asset Groupings are not part of the official record but are added later for information.

# List Entry Description

### Summary of Building

Legacy Record - This information may be included in the List Entry Details.

### Reasons for Designation

Legacy Record - This information may be included in the List Entry Details.

### History

Legacy Record - This information may be included in the List Entry Details.

### Details

CAMDEN

TQ3081NW BLOOMSBURY SQUARE
798-1/100/101 (West side)
30/06/69 Nos.9-13 (Consecutive) 
White Hall Hotel (Nos.9-11) and 
attached railings 

GV II

5 terraced houses. c1662-5. Built by T Coxe. Many C18 & C19 
alterations. Nos 10, 11 & 13 refronted c1770-2; 3rd floor 
added 1841. Fronts stuccoed c1862 in Italianate style. 
Nos 9-11 form a symmetrical group with a recessed central, 
pedimented bay. Slated mansard roof with dormers. 4 storeys,
No.10 plus an attic storey, attics and basement. 3 windows 
each. Rusticated ground floor. Pilasters rise from 1st floor,
articulating each house, carrying entablature, with dentil 
cornice on outer bays. Square-headed doorcase with pilasters,
round-headed arch with keystone and voussoirs, paterae, and 
cornice; fanlight and 2 leaf panelled doors. Ground floor 
windows partly with hexagonal and small square leaded lights.
Architraved 2-pane sash windows, the 1st floor with cornices
and wrought-iron balconies, 2nd floor with cornices broken 
upwards over the keystones. Balustraded parapet to outer bays;
central bay with pediment over the attic storey with enriched
tympanum containing a cartouche with the words "White Hall".
INTERIOR: not inspected but noted to have been reconstructed
for use as a hotel 1909-11. Nos 12 and 13 continue the terrace
north in the same style as Nos 9 and 11 but without mansard 
roofs and dormers. Square-headed recessed doorways with 
fanlights and panelled doors. INTERIORS: not inspected but 
No.12 contains a panelled ground floor front room, formerly 
the office of the historian Sir Nikolaus Pevsner. 
SUBSIDIARY FEATURES: attached late C19 cast-iron railings to
areas. 
HISTORICAL NOTE: No.10 was the house of Dr Robert Willan, 
dermatologist, GLC plaque. 





Listing NGR: TQ3028181647

# Selected Sources

Legacy Record - This information may be included in the List Entry Details.

# Map

### National Grid Reference: TQ 30295 81638

The below map is for quick reference purposes only and may not be to scale. For a copy of the full scale map, please see the attached PDF - 1244507.pdf

© Crown Copyright and database right 2012. All rights reserved. Ordnance Survey Licence number 100019088.
© British Crown and SeaZone Solutions Limited 2011. All rights reserved. Licence number 102006.006.

This copy shows the entry on 28-Nov-2013 at 10:42:42.