Lifetime Homes (LTH) Revised Criteria – July 2010

Comments on criteria accompanying applications 2013/3982/invalid and 2013/4484/invalid insofar as these do not conflict with Listing

- Parking – n/a

- Approach from parking – n/a

- Approach to dwelling and communal entrances – minimum effective clear width 825mm plus 300mm nib on pull side of leading edge of door

- Entrances – Where new these will be to Required Specification and Good practice recommendations, being lit, having level access over thresholds, openings and widths as 4. level external landings and adequate weather protection.

- a) – Communal stairs – Existing listed communal stairs and balustrades retained as existing but with contrasting brightness to step nosings.

b) -  Communal lifts -  Existing single car lift well is to be enlarged to include thickness 	of external wall (retaining outer half brick skin only so as not to alter lightwell facing), 	creating space for 2 lifts (as recommended) with lift controls as specified. One new 	passenger lift car will be installed with minimum internal car dimensions 1100mm  x 	1400mm and a 1500mm x 1500mm landing. The second lift may not be to these 	dimensions and will then be a service lift. The proposed lift in the Carey Street stairwell 	will also be a service lift.

6.	Internal doorways and hallways -  Min width of hallways to be 900mm with 750mm at 	pinch points and with door widths and nibs as specified.

7.	Circulation Space -  Most bedrooms and living rooms are well in excess of min 	standards for wheelchair circulation.  The smallest rooms will comply.

8.	Entrance Level living space – the top dwelling (3.9/4/9) is on 3rd and 4th floor levels with 	lift access to 4th floor living room.

9.	Potential for entrance level bed-space. -  Within the 4th floor living room there is a 	recess for temporary bed-space.

10. Entrance level wc and shower drainage. – an accessible bathroom with shower is 	provided at 4th floor level as Criterion 14.

11.	WC and bathroom walls. – adequate fixing and support for grab rails will be provided.

12.	Stairs and potential through-floor lift. – Within the 3rd/4th floor dwelling the stair well 	provides space for a through-floor lift.  This is in addition to the communal lift.

13.	Potential for fitting of hoists and bedroom/bathroom relationship -  All bedrooms have 	bathrooms to Criterion 14 standards en-suite and ceiling structure over bedrooms and 	bathrooms will be capable of adapting to support a single point hoist above bed, bath 	and WC.

14,	Bathrooms - All master bedrooms have bathrooms to Criterion 14 standards en-suite.  	Other bathrooms will have as many facilities as are practical in addition to Criterion 11.

15.	Glazing and window handle heights – All windows facing Chancery Lane, Carey Street 	and Star Yard are listed and will remain unaltered.  Those facing lightwells are also 	listed but permission may be granted to adapt some windows to this criterion.

16.	Location of service controls -  New services will be provided with controls set within a 	height band of 450 to 1200mm above floor and at least 300mm from any internal 	corner.

95cl1013lifetmstatemt