HERITAGE STATEMENT.

Statement of Heritage impact for

86 Heath Street, London, NW3 1DP

This Statement forms part of the statement of the environmental effects for

The erection of a sage green external and Stone internal timber conservatory.

Date

25th June 2013

Prepared By,

Miss Charlotte Williams

For,

Mr Rudelhoff & Keys

Existing House

14/05/74 Nos.86, 88 AND 90 

Terrace of 3 houses is a grade II listed building. The building was first made listed 14th May 1974. Multi-coloured stock brick with slated roofs having projecting eaves. 3 storeys and basements. 2 windows each. Round-arched doorways with fanlights and half-glazed doors; C20 Georgian style doorcase with pilasters, open pediment and radial fanlight. Gauged brick flat arches to recessed 4-pane sashes, except ground floors with tripartite sashes.
INTERIORS: not inspected. 

Heritage entry number: 1378837

http://list.english-heritage.org.uk/resultsingle.aspx?uid=1378837

Overall Impact

The new conservatory would be well built to be in keeping to the existing property, It would bring a sense of the garden into the house. The proposed would not affect the setting, as it will be constructed of a traditional hardwood material and would be located at the rear elevation.

Proportion

The new conservatory will have a modest floor area of approximately 11.8m2. The proposal will be well screened from passing view. Please see proposed elevations for a detailed illustration of our proposal, including applicable measurements.

Height

The conservatory will be of Sage external and stone internal Woodgrain full height wall frames with double glazed sealed units.  The roof will also be glazed with a traditional lead flashing to join to the property. Please see proposed elevations for a detailed illustration of our proposal, including applicable measurements.

Massing/Bulk

As you can see from our plans, the timber frames will be of modest proportions, as such, clear glass will be the primary material feature of our proposed conservatory. Due to the thin timber wall frames, the overall transparency of the proposed unit will be substantial. This will effectively help to minimise the overall bulk of the proposed conservatory unit.

Use of Materials

As alluded to previously, the proposed conservatory unit will consist of sage green external and stone internal woodgrain wall frames, double glazed sealed units with a glass roof. Prior to proposing our unit, we carefully considered which materials would be best suited to this area and, more importantly, the property itself. Having conducted several site and area visits, we understand that timber wall frames are essential for any proposed conservatory extension. Proposing PVCu wall frames in such an area would be simply out of the question, as it would detract from the traditional values already present.

The wall frames will be inserted with clear glass, as it would not detract aesthetically from the area whilst helping to minimise the unit bulk, which is often caused by obscure glass inserts or unnecessarily large frames.

The roof will consist of glass roof sheets, with aluminium rafters with a lightweight

aluminium capping finished in sage green to match wall frames of unit.

Use

The applicant wishes to enjoy the garden from inside the home with their proposed new conservatory, this will be habitable living space.

Relationship with Adjacent Assets

Hampstead is a beautiful area, steeped in traditional values and architecture, we very much appreciate this and identify the need to maintain the traditional features where possible. With our proposed unit we have been sympathetic to this factor, opting for sage green external and stone internal woodgrain wall frames. We are therefore confident that our proposed unit will be a welcome addition to the property, very much consistent with the area and all other adjacent assets.

Additional information

Our proposed unit will not negatively affect the site and surrounding area by its presence. In fact, we are confident that the proposed unit will actually greatly enhance it. With the established boundaries there will be minimal disruption to the surrounding neighbours.