Jonathan Brunskill Associates

32 Dale Street

W4 2BL

Architecture - Building Conservation - Interior Design

Telephone 020 8995 5645  Mobile 07958 613340

Email  jb@jonathanbrunskill.co.uk

24th June 2013

Lifetime Home Statement -  85 Canfield Gardens NW6 3EA

_____________________________________________________________

- On plot parking – There is no provision for on site parking.

- Approach to dwelling from parking. There is no on site parking but the access from the street to the house is a level path.

- Approach to all entrances. The access is level from the street, with little or no gradient. The access path is 900mm minimum.

- Entrances. There will be lighting at the main entrance with a diffused luminaire. The external landing is level and the access threshold has a minimum 15mm rise. The clear width is minimum 800mm.

- Communal stairs and lifts. Not applicable..

- Internal doorways and hallways. Minimum width is 900mm, reducing to a minimum 750 at pinch points.  The clear opening widths of the doorayswithin the dwelling are no less than 750mm clear width.

- Circulation space.  There is sufficient space for a wheelchair to turn within the living areas, and the hallways are listed in Criterion 7. Due to the nature of the proposal there is no facility for wheelchair access to the first floor.

- Entrance level living space. The living space is sited on the entrance level of the flat.

- Potential for entrance level bed space. There is provision for an entrance level bed space on the entrance level if temporary accommodation was required.

- Entrance level WC and drainage. There is provision for an accessible entrance level WC and drainage, with a shower enclosure and basin.

- WC and bathroom walls. The walls are sufficient to provide fixings for grab rails, and are made of brick or studwork, with additional studding at appropriate positions for secure fixings.

- Stairs and potential through-floor lift in dwellings. The staircase is designed so that a stair lift can be installed if required. Due to the design there is space for a through floor lift if required. The stairs provide clear width of 900mm.

- Potential for fitting hoists and bedroom/bathroom relationship. The ceilings over the bedrooms is of timber construction and of sufficient strength to support a hoist. The design provides reasonable access to the bathroom adjacent.

- Bathrooms. The bathrooms provide ease of access sufifcnet to provide the clearance set out in clauses 14i), ii), iii).

- Glazing and window handle heights.  The windows within the principal living space are sliding box sash type and the cills are below 800mm above FFL. There is provision for a wheelchair user to approach a window in each habitable room. (750mm clear width approach route).

- Location of service controls. Service clntorls to the dwelling are located within a height band of 450mm to 1200mm above FFL and located minimum 300mm from a corner.