15 March 2013

DESIGN AND ACCESS STATEMENT FOR THE FLATS CONVERSION AT 127, KENTISH TOWN ROAD, LONDON, NW1 8PB.

The existing property is a two storey terraced building located in a busy commercial area consisting of a Ground floor retail shop used as a hair and beauty salon. There is no basement accommodation within this unit. The maisonette above has separate access directly from the street adjacent the retail shop.

There is a single storey rear extension at ground floor level which is used as part of the shop with a WC located at the rear. The unit narrows at the rear as the property at 129 gets wider at this section.

The existing building is in reasonable condition with white UPVC windows and white painted brick front elevation with the existing pitched roof finished with slates.

The proposed scheme consists of extending the First floor flat into a maisonette therefore increasing its accommodation at Second floor level within the width of the terrace plan which will be part of an overall scheme to add a new storey to the 5 units from 119-127 Kentish Town Road to standardise the street elevation and match the height of units either side of this section of the terrace.  The new enlarged flat will provide a two bedroom maisonette over the existing first floor and proposed second floor levels which will continue the pitched slate roof at the same height as the adjacent terrace at 129 Kentish Town Road.

The new elevation to the terrace will be uniformly rendered and painted white with white UPVC windows throughout. The rear elevation will be finished in facing brickwork in London Stock while the flat roof to the first floor rear extension will be finished in felt.

Nick Panayiotou RIBA ARB