Picus Sonic Tomographic Test Report

Test undertaken: 22nd January 2013 by Mr Kim Gifford. In the area of test and at the time of inspection.( in snowy 1 degree Celsius  weather conditions).

Method: using Picus software Q72std, 10 modules in accordance with manufacturers manual.

Brief: Provide Picus Tomographic Test and report regarding the structural health of the Black Poplar in the rear garden area of 12/18 Antrim Mansions.

Site : Rear of 12/18 Antrim Mansions Antrim Road London NW3.

Tests on the leaning Ivy clad Pollarded Black Poplar 16m high.

Test 1 @ level: 60 cms approximately from ground level. Test point 1 at north orientation..

Picus Test Analysis: The Tomographic image shows predominately altered or decayed timber depicted as blue and white coloured sonic representations. The measuring point readings give evidence of low or nil sound velocities results across the area of test.

Conclusion: The  Picus test result gave poor readings of decayed and structurally weak  timber in the area of test.

Recommendation:   This specimen is in a poor Arboricultural structural condition overall. It is therefore recommended that it is removed within the near future on the grounds of health and safety.

If you require any further information please do not hesitate to contact me.

Kind Regards

Kim Gifford

www.treetesting.com

07831488456