|  | ## SITE  INSPECTION  REPORT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | # MLM BUILDING CONTROL LTD Zetland House, 5-25 Scrutton Street, London EC2A 4HJ Tel: 020 7613 7500 Fax: 020 7613 5747 |  |  |  |  |  |
| Project New 2 bedroom dwelling adjacent |  |  |  |  |  | Job No: 8411321 |
| Address: Hodes Row |  |  |  |  |  |  |
| Surveyor: Stephen Percival |  |  | Time 4pm |  | Date 9.05.12 |  |
| Project Manager: Michael Wadood |  |  | Inspection Type: Foundation |  |  |  |
| Inspections carried out: 1 |  |  | Next Inspection: Foundation |  |  |  |
| Site Contact: Ray Bradley |  |  |  |  |  |  |
| Site Comments |  |  |  |  |  |  |
| Met Ray on site and noted the following issues : - New strip foundation dug 600mm wide x 1000mm deep, this is in line with engineers drawing HR/01, reinforcement will be added to foundation at a later date. Engineers drawings are approved. - Exposed gas pipe at end of foundation, to be boxed. - Clay at base of excavation some stones and rubble fallen into it will be cleared out. |  |  |  |  |  |  |
| Time Taken: 2 hours |  | Action required by project manager: No |  |  |  |  |
| Signed Site Representative: Date 9.05.12 |  |  |  |  |  |  |
| Signed Project Manager |  |  |  |  |  |  |