## 18 FERNCROFT AVENUE  NW3

Springs:

At the height of Ferncroft Avenue in Hampstead there is always the possibility of

springs occurring where there is sufficient sand and gravel in the soil. The trial bore holes indicated that the soil below ground level was a stiff silty clay becoming stiffer with depth. There was no significant water or water bearing strata down to a depth of 4m although in the rear borehole 1 there was water seepage at 1.9m and in borehole 2 at 3.3m. However on completion the water stood below 4m in both boreholes. The boreholes at the front of the property remained dry to a depth of 4m. There was no evidence of spring water in the rear garden and the space between nos. 18 & 20 will remain largely undisturbed allowing natural drainage in the soil from the rear to the front of the property. Nevertheless all retaining walls will be designed for possible water pressure and the inside surface will be lined with a self draining membrane to carry any moisture to a sump pit from which it is pumped to a ground level manhole.

.