Addendum to Arup Kidderpore Avenue BIA Appraisal

Page 14.

- Section 4 should read:

Assessment of ground movement is made based on CIRIA C580 case history data for wall installation and basement excavation stages of the development.  For wall installation the use of CIRIA C580 is industry standard for assessment of ground surface movements.  For basement excavation the combination of site specific ground stiffness and propping layout results in CIRIA C580 “design lines”  being acceptable for initial assessment of ground movement but crude when damage assessment of adjacent properties are being made (CIRIA C580 design lines remain as a sanity check).  The approach adopted in the GCG report in the use of CIRIA C580 data is considered appropriate for initial assessment but likely to be unreliable for detailed damage assessment due to the relatively linear nature of the movement profiles with distance from the basement excavation.  Further work will be required to demonstrate that the ground movements predicted (and resulting movement of adjacent structures) in the existing report are representative of more detailed calculations carried out later.