London Borough of Camden Planning Reference: CA/2007/ENQ/15165

Capita Percy Thomas Reference: CS/004729-01-02-17

Revision A   - 4th May 2007

Revision B   - 25th May 2007

Revision C   - 1st June 2007

RELOCATION OF THE BANDS AND MINOR UNITS – CENTRAL LONDON

SITE LOCATION: REGENTS PARK BARRACKS

DESIGN STATEMENT

INTRODUCTION

The design proposal is for new build accommodation for the Irish State and Welsh State Military Bands. The building is to provide rehearsal, ensemble and practice spaces for the bands, sectionals and individuals within the bands. The building will also provide associated administration, storage, toilet and changing Areas. The building is in part two-storey with the two main rehearsal rooms and the percussion practice rooms as double height spaces.

SITE

The proposed site for the building is on the existing Parade Ground of Regents Park Barracks, London. It is noted that the public are unable to see either the proposed building or Parade Ground from the road. The site is adjacent to and overlooked by the Officers’ Mess which is a Grade II Listed building. The land on the Parade Ground is understood to be classified as Contaminated Land, but a phase 1 survey has proved it otherwise.

This new building is located within the Parade Ground and would be a central feature for the Barracks. The existing buildings surrounding the site are one, two and three storey of brick construction with slate roofing. The proposed building does not exceed the overall heights of the higher of the existing buildings, as indicated on the Site Section.

REHEARSAL, PRACTICE ROOMS & ACOUSTICS

Acoustics are an important consideration for this building. Under the advice of the project acoustician, the musical rooms include at least one non-parallel surface at 15 degrees (from a minimum of 5 degrees for acoustic reasons). This has been used in the generation of the plan shapes. Room adjacencies have also been altered to include for acoustic considerations. The angle of the walls is a feature and this has developed into to the creation of the roof.

The rehearsal spaces are the key spaces to this whole facility where the whole band will rehearse together prior to public performances and the quality of sound created here is of vital importance. To produce this quality of sound the JSP scales ( Joint Service Publication for the Defence estates) require a minimum internal clear height of 5.5 metres. The scales are a tried and tested requirements of the defence estates and the musical requirements are better explained by the project acoustician in the enclosed letter. This height is required for the Percussion rooms as well. The ensemble rooms are required to be at a height of 3.7 metres. These criteria drive the height of the floors. With this requirement and those of the structural and building services engineers, there has been every effort to minimize the overall height of the building. This has included the lowering of the pitch of the roof by one degree to avoid increase in height of the fascia away from earlier drawings.

Other music rooms are the individual practice rooms which form part of the two-storey accommodation. A practice space would not be expected to have the same acoustic quality as a rehearsal space. These Practice Rooms have a ceiling height of 3m. The lower ceilings whilst not providing the same acoustics as the Rehearsal Space are sufficient for practice rooms. In plan, the smaller individual practice rooms can be defined either with the angled or parallel walls. Our preference is to have a non–parallel one that becomes a stronger architectural feature for the main entrance.

EXTERNAL MATERIALS

With regard to external materials, the intention is to use brick as the main cladding material. The brick is proposed is a London stock type brick and a blue engineering type brick is introduced for the plinth and feature banding to create variety and interest.

Fenestration would be powder coated aluminium windows. The Windows are treated as portrait or square openings in the masonry in the facades. The corner windows will also cope with the non right-angled corner detail for the main section of walls whilst breaking up the building mass.

The roof fascia would be an aluminium bull-nose type detail and the main roof as a ‘green’ roof falling to a central gutter with Sun pipe type roof lights for natural light.

The cladding which offsets the brick walls from the roof is envisaged to be in light coloured cladding to match the powder coated aluminium windows to provide a feeling of lightness to the whole building along with the inverted roof and the corner windows. Glazing was envisaged initially for this area. But, due to security reasons heavy density block work has to be used to build the interior skin of the external walls which go up all the way to the underside of the roof that will obscure the glazing and it will look no different than a panel. Also, it was

not cost effective and light weight panels which can achieve the same effect by matching them to the window mullions were chosen. Setting the panels back from the line of the bricks and providing the feature banding just below the panels will further break up the massing.

The building is served by a bio-mass boiler which is located adjacent to the plant room for ease of operation and to cut down costly service runs. This delivers in excess of ten percent savings in energy. The building by the nature of it’s operation has to be a completely sealed one and as such requires quite extensive mechanical systems to maintain the levels of ventilation required which in turn require a large plant room. So the boiler has to be separate but adjacent to the plant room. This bio-mass boiler is located opposite the listed building but cannot be seen from anywhere on site or in relation to the listed building. This has been necessary to achieve the main part of the proposed building being offset from the listed building and allowing for some of the listed frontage to be seen. In earlier drawings, the biomass boiler, to address the renewables requirement, included a 7metres high and 3metre diameter silo to store the fuel. As the design has developed, the team has sourced a fuel storage type that can be retained within the brick enclosure and obviating the need for the silo. However, the flues are integral to the renewables solution. This development with the omission of the silo is seen overall as an advantageous development of the design. The bio-mass boiler is quieter than a conventional gas boiler in operation and will not cause any acoustic disturbance to the site and surroundings.

TRANSPORT AND MOVEMENT

The new build will provide bandsmen practice accommodation. From here they would also march to attend to ceremonial duties. All members using the building would live off site. Please refer to separate Transport Statement to be submitted with the Planning Application.

External spaces: Mainly hard standing to fulfill the car parking and operational requirements of the Client and End User. The external layout includes Cycle racks for the Barracks, disabled parking close to the building and attendance on DDA Considerations.