# List Entry Summary

This building is listed under the Planning (Listed Buildings and Conservation Areas) Act 1990 as amended for its special architectural or historic interest.

### Name: NUMBERS 1-22 AND ATTACHED RAILINGS

### List Entry Number: 1330368

### Location

NUMBERS 1-22 AND ATTACHED RAILINGS, 1-22, REGENTS PARK TERRACE

The building may lie within the boundary of more than one authority.

County: Greater London Authority
District: Camden
District Type: London Borough
Parish:

### National Park: Not applicable to this List entry.

### Grade: II

### Date first listed: 14-May-1974

### Date of most recent amendment: Not applicable to this List entry.

# Legacy System Information

The contents of this record have been generated from a legacy data system.

### Legacy System: LBS

### UID: 477854

# Asset Groupings

This List entry does not comprise part of an Asset Grouping. Asset Groupings are not part of the official record but are added later for information.

# List Entry Description

### Summary of Building

Legacy Record - This information may be included in the List Entry Details.

### Reasons for Designation

Legacy Record - This information may be included in the List Entry Details.

### History

Legacy Record - This information may be included in the List Entry Details.

### Details

CAMDEN

TQ2883NE REGENT'S PARK TERRACE
798-1/76/1385 Nos.1-22 (Consecutive) 
14/05/74 and attached railings 

GV II

Terrace of 22 houses. c1840-50. Yellow stock brick with 
rusticated stucco ground floors. Nos 1-21 form a symmetrical
facade with slightly projecting end houses. 4 storeys and 
basements. 2 windows each. Square-headed doorways with 
cornice-heads, fanlights and panelled doors. Entrance to No.1
on right hand return with stucco portico having pilasters, 
cornice and parapet; round-arched door way. Architraved 
sashes; 1st floor with cornices and continuous cast-iron 
balconies, 2nd floor with cornices. Plain stucco sill bands to
2nd and 3rd floors. Stucco modillion cornice and blocking 
course. 
No.22: rusticated stucco. 2 storeys and basement. 1 window. 
Projecting stucco portico with balustraded parapet. Cornice 
with balustraded parapet. 
INTERIORS: not inspected. 
SUBSIDIARY FEATURES: attached cast-iron railings with foliated
finials to areas. 






Listing NGR: TQ2862883819

# Selected Sources

Legacy Record - This information may be included in the List Entry Details.

# Map

### National Grid Reference: TQ 28628 83819

The below map is for quick reference purposes only and may not be to scale. For a copy of the full scale map, please see the attached PDF - 1330368.pdf

© Crown Copyright and database right 2012. All rights reserved. Ordnance Survey Licence number 100019088.
© British Crown and SeaZone Solutions Limited 2011. All rights reserved. Licence number 102006.006.

This copy shows the entry on 24-Oct-2013 at 12:08:27.