UPPER MAISONETTE 130 CAMDEN STREET, CAMDEN TOWN, LONDON NW1 0HY:

DESIGN & ACCESS STATEMENT

The property is a three storey maisonette in the upper floors of this lower ground and four storey house built in c 1890. The property has been converted into three flats and the maisonette was originally owned by the Council.

The proposal is to construct new extension rooms on the existing rear extension to provide better living accommodation.

The proposal is to form a new extension off the half landings of the stairs. The existing entrance to the flat will be relocated on the ground floor which has been discussed / agreed with the freeholders.

On the first floor half level the new extension space will provide a utility and washroom; on the second floor the extension will provide a bathroom and built in cupboard.

The existing wash and bathrooms on the second floor will be reformed to provide greater storage and a new washroom.

The new extension is to be built out to the line of the extensions on adjoining properties. The scale of the new extension is in keeping with the scale of the adjoining extension on no 132.

The new extension will have a flat roof.

The flat roof will be finished in a grey ply roofing membrane. The new brickwork will be a stock brick matching the texture of the existing property; windows and new doors will be painted timber or steel double glazed elements.

Context

The new extension is sized to be a secondary element in relation to the existing property.

Use

This is a residential flat and the alterations continue the existing residential use for the current occupiers.

Amount

The new extension creates a new floor area of 4.7sq metres on each floor.

Layout

The extensions and internal alterations create better utility, wash and bathroom facilities for this family home.

Access & inclusion

The existing arrangement of rooms is difficult for current; the new extensions provide better useful spaces and improve the accommodation.

Scale

The size of the extension has been considered in relation to the overall property and in our opinion does not affect the overall appearance of the property.

Appearance

The extension flat roof will be finished in grey ply roofing membrane; the external walls will be built in London stock like the rear of the property. New windows and doors will be painted timber or powder coated steel double glazed elements.

Climate change

The new roof will drain to the surface water drainage system. It is probable that the owners may divert some of the run off to a new water butt for gardening purposes.

Maintenance & Management

The external materials proposed are hardwearing and robust and will require only minor regular maintenance.

DRAWINGS SUBMITTED :

| EXISTING DRAWINGS |  |  |
| --- | --- | --- |
| 130CS PA1.10 | LOCATION PLAN | 1:1250@a4 |
| 130CS PA1.21 | FIRST & GROUND FLOOR PLAN AS EXISTING | 1:50 @ A3 |
| 130CS PA1.22 | SECOND & THIRD FLOOR PLAN AS EXISTING | 1:50 @ A3 |
| 130CS PA1.23 | ROOF PLAN AS EXISTING | 1:50 @ A3 |
| 130CS PA1.31 | REAR ELEVATION AS EXISTING | 1:50 @ A3 |
| 130CS PA1.41 | SECTION THROUGH STAIRS AS EXISTING | 1:50 @ A3 |
| 130CS PA1.42 | SECTION TOWARDS 132 AS EXISTING | 1:50 @ A3 |
| PROPOSAL DRAWINGS |  |  |
| 130CS PA2.21 | FIRST & GROUND FLOOR PLAN AS PROPOSED | 1:50 @ A3 |
| 130CS PA2.22 | SECOND & THIRD FLOOR PLAN AS PROPOSED | 1:50 @ A3 |
| 130CS PA2.23 | ROOF PLAN AS PROPOSED | 1:50 @ A3 |
| 130CS PA2.31 | REAR ELEVATION AS PROPOSED | 1:50 @ A3 |
| 130CS PA2.41 | SECTION THROUGH STAIRS AS PROPOSED | 1:50 @ A3 |
| 130CS PA2.42 | SECTION TOWARDS 132 AS PROPOSED | 1:50 @ A3 |

ARP : October 2014

End of document