D O N A L D S H E A R E R C H A R T E R E D A R C H I T E C T S

Scholars’ House, Shottery Brook Office Park, Timothys Bridge Road, Stratford upon Avon, Warwickshire CV37 9NR

Tel 01789 294560 Fax 01789 294549 e-mail: donald@ds-architects.demon.co.uk

| Our Ref: 1099BA002 | Date: 29th August, 2013 |
| --- | --- |

Re:	The Land Adjoining 42 Falkland Road – Lifetimes Home Compliance

Supporting information confirming compliance / reasons where compliance cannot be achieved, to the following standards as set out in the requirements of Lifetime Homes;

- Criteria 1 in respect of disabled onsite parking cannot be complied with due to the constraints of the site and townscape considerations.  This is a development that involves the redevelopment of an existing inner city site.  There is however substantial on Street Parking available, which is controlled for residents only during control hours.  This on street parking includes designated Disabled Parking.  In addition to the substantial on street parking there is  excellent public transport with Kentish Town station and a main bus route within 150 Metres of the property together with the wider retail facilities of the Commercial centre of Kentish Town.  For these reason we submit that it is reasonable for criteria 1not to be applied to this development.

- The essence of Criteria 2 is met for the proposed flats, as the access between the pavement and the front door is both level and flat and on street parking includes designated Disabled Parking adjacent to the site.

- A Maximum gradient of 1:12 on slopes less than 5m is provided. A Maximum gradient of 1:15 on slopes between 5m & 10m is provided thus ensuring that the proposals comply with Criteria 3.

- The communal entrance to the flats is illuminated throughout the hours of darkness by adjacent street lights on Falkland Road.  The entrance doors will also have additional movement sensor lighting above the entrance doors recessed into the underside of the recessed soffits, which also offer some weather protection for the entrance to the building. The areas illuminated with movement sensor activated lighting with a 1 minute over run.  The main entrance is recessed to offer weather protection and has a level external landing, with level access over the threshold thus ensuring that the proposals comply with Criteria 4.

Both proposed units  – Disabled to Lifetime Homes – have a clear doorway opening of 950mm with an access corridor greater than 1000mm.

- The Communal Stairs allow for - 

Uniform rise not exceeding 170mm.

Uniform going not less than 250mm.

Handrails that extend 300mm beyond the top and bottom.

Handrails height 900mm from each nosing.

Step nosings distinguishable through contrasting brightness.

Risers which are not open

Therefore the proposals comply with Criteria 5. This scheme does not have a Communal lift.

- All internal hallways are a minimum width of 950mm wide.  All doorways have a minimum clear opening of 760mm ensuring that the proposals comply with Criteria 6.

- All proposed units - Disabled to Lifetime Homes – provide sufficient circulation space in the Bedrooms. Bathroom layouts meet the standards set out in Criterion 14. The kitchen has a clear turning circle of 1200mm between units.

- Entrance level living space is provided to each unit in accordance with criterion 8.

- Entrance level bed-space is provided to each unit in accordance with criterion 9.

- Criterion 10 is met as each flat has an accessible WC / Bathroom with potential showering facilities as illustrated on the plans. The Bathrooms in Flats 1, 2, 3, 4, 6, 7 & 8 have been designed to convert easily into shower rooms after removing the baths. The WC’s in Flats 5 & 9 have been designed to convert easily into shower rooms. The wheelchair users will be able to close the door from within the closet and achieve side transfer from a wheelchair to at least one side of the WC. There is at least 1100mm clear space from the front of the WC bowl and 700mm to one side of the WC. The centreline of the WC is more then 500mm from any side wall / fixed structure. The shower is provided within this converted shower room.  The WC’s / bathrooms will all be 1st fixed pre-plumbed to ensure conversion to shower rooms is easily achieved ensuring that the proposals comply with Criteria 10.

- All walls in the, bathrooms, en-suites and WC compartments will be capable of firm fixing and support for adaptions such as grab rails ensuring that the proposals comply with Criteria 1

- All Flats are single storey apart from Flat 9. The staircase in Flat 9 has a minimum clear width of 900mm and is easy going with potential for a stair lift in accordance with Criterion 12a. A potential 1000mm x 1500mm space for a through the floor lift is provided between Bedroom 1 and the Reception in compliance with Criterion 12b. A ‘knock out’ panel will be pre-formed within the floor.

- The structure above all floors will be designed as precast concrete floors, designed to be capable of supporting ceiling hoists should these become desirable by the occupants for bed to bathroom transfer in compliance with Criterion 13.

- All bathrooms are designed to allow the baths to be replaced by a shower to ensure a 1500 mobility radius is achieved to each unit ensuring that the proposals comply with Criteria 14.

- All windows will have a cill height no higher than 800mm above FFL.  Window opening controls will be located no higher than 1200mm above FFL ensuring that the proposals comply with Criteria 15.

- All service controls will be placed between 450 and 1200mm from FFL and at least 300mm from any internal room corner ensuring that the proposals comply with Criteria 16.

Yours sincerely,

Donald Shearer