| #### Delegated Report |  |  | #### Analysis sheet |  |  |  |  |  | #### Expiry Date: |  |  | # 12/03/2014 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | N/A / attached |  |  |  |  |  | Consultation Expiry Date: |  |  |  |  |
| #### Officer |  |  |  |  |  |  | #### Application Number(s) |  |  |  |  |  |  |
| Mandeep Chagger |  |  |  |  |  |  | 2014/0388/P |  |  |  |  |  |  |
| #### Application Address |  |  |  |  |  |  | ## Drawing Numbers |  |  |  |  |  |  |
| 6A North End London NW3 7HL |  |  |  |  |  |  | See ddn |  |  |  |  |  |  |
| #### PO 3/4 | #### Area Team Signature |  |  | #### C&UD |  |  | #### Authorised Officer Signature |  |  |  |  |  |  |
| #### Proposal(s) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Basement extension to existing dwelling including 2x lightwells to rear. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Recommendation(s): |  | ## Granted |  |  |  |  |  |  |  |  |  |  |  |
| Application Type: |  | ## Householder Application |  |  |  |  |  |  |  |  |  |  |  |
| ## Conditions or Reasons for Refusal: |  | ### Refer to Draft Decision Notice |  |  |  |  |  |  |  |  |  |  |  |
| ## Informatives: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Consultations |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ## Adjoining Occupiers: |  | No. notified |  |  | 02 | No. of responses No. electronic |  |  |  | 00 00 | No. of objections |  | 00 |
| ## Summary of consultation responses: |  | Site notice 24/01/2014 – 14/02/2014 Press advert 30/01/2014 – 20/02/2014 No letters were received from neighbouring. |  |  |  |  |  |  |  |  |  |  |  |
| ## CAAC/Local groups* comments: ## *Please Specify |  | Hampstead CAAC: Require BIA for neighbouring properties impact and groundwater. Not adequate information to assess these properties. OFFICER COMMENT: The submitted BIA was independently assessed and the results show any concerns raised have been adequately addressed. |  |  |  |  |  |  |  |  |  |  |  |

## Site Description

The property is part of a small block of three properties around a central courtyard. The existing property is positioned with two party walls to adjacent properties on perpendicular elevations. The property has two above ground floor levels, with vaulted ceilings to the first floor rooms. The site is location within the Hampstead Conservation Area, it is not a listed building.

## Relevant History

No relevant planning history.

## Relevant policies

National Planning Policy Framework 2012 London Plan 2011 LDF Core Strategy and Development Policies 2010 CS1 (Distribution of Growth) CS5 (Managing the Impact of Growth and Development) CS14 (Promoting High Quality Places and Conserving Our Heritage) DP16 (The transport implications of development) DP20 (Movement of goods and materials) DP21 (Development connecting to the highway network) DP22 (Promoting sustainable design and construction) DP23 (Water) DP24 (Securing High Quality Design) DP25 (Conserving Camden’s heritage) DP26 (Managing the Impact of Development on Occupiers and Neighbours) DP27 (Basements and lightwells) Camden Planning Guidance 2013 CPG1 Design CPG4 Basements and lightwells Hampstead Conservation Area Statement 2001

## Assessment

- Proposal 1.1 The application proposes a new basement level under the entire of the existing footprint of the property, extending under the properties courtyard, to the site boundaries. The layouts of the upper floors indicate that the changes include the addition of a new lightwell in the courtyard and amendments to the stairs, with new flight down to the lower ground floor level. 1.2 The main issues are: - Basement excavation - Design and impact on conservation area. - Neighbour amenity. - Basement excavation 2.1 The proposed development is to comprise a single storey basement that will underlie the current footprint of 6a North End and extend out to the west, under the property garden. 2.2 New lightwells will be formed at the eastern boundary of the garden, adjacent to the western elevation of the existing property. Development drawings indicate the basement formation level will be approximately 4m below ground level (mbgl). The above ground structures are to be retained. The existing foundations and party walls are to be underpinned 2.3 The applicant has submitted a basement impact assessment (BIA) in accordance with policy DP27. The site is located on Bagshot Beds. Given the site context, the BIA was independently assessed. The independent assessment advised that BIA had not identified that the site is within an aquifer. In order to meet the requirements of DP27 further information was required on the ground conditions, specifically, the presence and location of water-bearing layers beneath the site and information regarding the location and form of the foundations to the existing and neighbouring properties. A revised BIA was therefore sought. - The BIA was subsequently fully revised and has been reassessed by the independent assessor and now fully complies with the requirements of policy DP27. 3. Design and impact on conservation area. 3.1 The basement would include two lightwells positioned in the garden with a grille and access hatch. The low height of the lighwell surround would ensure that the lightwell would have a limited impact on the appearance of the host property and the surrounding area. The size and locations of the lightwells is considered acceptable. - Neighbour amenity - Policy DP26 aims to protect the quality of life of neighbours that might be affected by developments. - The proposed basement extension by reason of its location would not be likely to affect the residential amenities of the neighbouring properties any worse than the existing situation. Recommendation: Grant conditional permission