LONDON BOROUGH OF CAMDEN

ACCESS COMMENTS

From:		Michelle Horn

Tel. number:	020 7974 5124

Ref:		2014/4514/P

Date:		26/8/14

To:		Angela Ryan

Re:	11A Primrose Hill Road

Erection of a three storey plus basement single family dwelling house and associated landscaping works (Class C3)

As new build dwelling Part M of the Building Regulations will apply.  In addition planning policy DP6 will require the dwelling to be designed to comply with Lifetime homes standards

The applicant has submitted a Lifetimes Homes standard as part of their application however several of the requirements have not been achieved:

|  | LHS Standard | Comments |
| --- | --- | --- |
| 3 | Approach to all entrances Enable as far as practicable convenient movement along other approach routes to dwellings (in addition to the principal approach from a vehicle) for the widest range of people The approach to all entrances should preferably be level or gently sloping | This has not been achieved with a step at the entrance. This will need to be removed |
| 4 | Entrances Enable ease of use of all entrances for the widest range of people All entrances should: - Be illuminated - Have level access over the threshold - Have effective clear opening widths and nibs as specified In addition main entrances should also: - Have adequate weather protection - Have a level external landing | This has not been achieved with steps at both the front door and to the patio. These will need to be removed by lowering the ground floor level to be level. (there appears insufficient space to provide a ramp) All other requirements need to be achieved. . |
| 10 | Provide an accessible WC and potential showering facilities for: - any member of the household using the temporary entrance level bed space of Criterion 9, and: - visitors unable to use stairs. | The entrance level WC is insufficient in size. This will need to be revised. |
| 14 | Provide an accessible bathroom that has ease of access to its facilities from the outset and potential for simple adaptation to provide for different needs in the future. | The main bedroom en-suite does not appear large enough to accommodate the required transfer spaces. |