NBS SECTION Q37 - DESCRIPTION OF WORKS

Section Q37 deals with the design and installation of the Bauder Green Roof landscaping system, including the various related elements i.e. separation, protection, and drainage layers, substrates, Bauder supplied planting and accessories such as inspection chambers, trims etc.  It invokes clauses from related sections for waterproofing, insulation, landscaping and maintenance as necessary for a complete system.

It is intended for use on projects where the detailed design is completed by the specifier (architect or landscape architect) with technical assistance from the manufacturer as required and should be read in conjunction with any project specific drawings provided.

SCOPE OF WORKS

This section includes:

- Bauder Extensive green roof system components/ landscaping

- Related Bauder system accessories

This section does not include:

- Construction of the structural deck

- Bauder waterproofing system – refer NBS Section J41-110

### Q37	GREEN ROOFS

To be read with Preliminaries / General Conditions.

GENERAL

130   EXTENSIVE GREEN ROOF: Extension

  - Landscaping finish:  Pre-cultivated Sedum vegetation blanket

  - Substrate: New Plywood Deck

  - Slope: 2°

  - Waterproofing system: BAUDER TOTAL GREEN ROOF SYSTEM – as per J41-110

  - Drainage / protection layer: Bauder SDF Mat - 20 mm drainage / protection layer.  Installation as clause 770B.

  - Vegetation blanket: Bauder Xero Flor XF301 sedum blanket, applied in standard length rolls 2m x 1 m.  Installation as clause 800A

  - Landscaping depth: ca. 48 mm (excluding vegetation).

  - Vegetation: Mainly Sedum with some moss and grasses.

  - Accessories: -

-Bauder SS40 drainage and retention trim, fitted to all perimeters.  Installation as clause 820A.

-Purpose made chute outlet drainage grille, pre-fabricated from perforated stainless steel (fabricated and supplied by others).  Refer clause 830E.

-20 - 40 mm round washed pebble vegetation barrier provided around all protrusion, as clause 460.  Installation as clause 840.

-Bauder Xero Flor organic fertiliser, apply as clause 850A.

  - Additional requirements: As clauses 210, 710, 715A, 720, 910, 915A, 916, 920, 930.

PERFORMANCE

210	GENERAL DESIGN

  - Green roof and associated features: Complete the detailed design.

  - Proposals: Submit drawings, technical information, calculations and manufacturers literature.

PRODUCTS

460J31/440C440C	PEBBLE BALLAST

- Type: Washed, round pebbles.

- Size: Graded 20-40 mm and free from fines and sharps.

EXECUTION

710	INSTALLATION GENERALLY

  - Preparation: Clear all surfaces of debris.

  - Timing: After certification of waterproof membrane integrity.

  - Surface condition: Visually inspect waterproof membrane, report any damage.

  - Faults in waterproof membrane: Report.

  - Contamination: Do not use materials detrimental to healthy plant growth.

  - Storage: Do not overload.

  - Point loads: Avoid.

  - Outlets: Do not block.

  - Outlet grilles: Installed (these can be omitted where Bauder inspection chambers are used, if the grille cap height obstructs the closing of the chamber lid).

### 715A    GREEN ROOF RELATED REQUIREMENTS

- The following are vital to the accurate pricing, correct installation, and ultimately the long-term life of a green roof, and must, therefore, be included within the specification and tender documents: -

- Loadings: It is assumed that the architect or his advisors have satisfied themselves that the roof structure and deck are suitable to receive the dead load of the proposed green roof system and landscape both during construction and on completion of the works.

- Additional protection: A planned or contractual delay between the installation of the waterproofing and landscape will almost certainly necessitate additional/increased protection to the waterproofing.  This protection may be temporary or permanent.  The responsibility and cost of this possible extra protection should be clearly included within the tender documents.

- Detailed drawings: Correct detailing design and construction is essential to the long-term life of the green roof. It is essential, therefore, that detail drawings illustrating for the construction are included with the tender documents, in order to enable the contractor to tender accurately.

- Minimum upstand height requirements: The waterproofing should be taken up all abutment upstands, pipes, detailing protrusions etc. a minimum of 150mm above finished landscape surface level to comply with British Standards and current code of practice BS8217:2005.

- Provision for living products in hot weather conditions: During hot weather conditions, living products such as plants, turf, sedum blankets etc. must be laid on the day of delivery to site.   With regard to sedum blankets or turf, any rolls not installed should be laid out and kept watered prior to final installation.

- Watering / Irrigation: Adequate provision for watering the installed any form of planting must be in place on site before the product is installed.  Irrigation systems if fitted should be operational.  Initial watering should be by surface sprinklers to water in the fertilizer, where this is specified.  All watering should be carried out in strict accordance with the Bauder watering requirements and guidance document.

- Final Inspection: No landscaping work should be installed until Bauder have carried out a final inspection to the waterproofing and have passed this as suitable for guarantee. It is the responsibility of the roofing contractor to advise and organise this inspection with Bauder.  We cannot guarantee any waterproofing that has been landscaped without this inspection having been carried out and passed as acceptable.

- Damage risk form other trades: No landscaping should be installed while the roof area is subject to other site traffic.  Bauder will carry out an inspection of the completed roof 4-6 weeks following installation and any site related damage by others will be reported to the client.   Bauder accept no responsibility whatsoever for damage to the product or the installation caused by site work carried out by others after the landscaping has been installed.

- First year maintenance:  The contractor must price into his tender the cost of post installation maintenance for a minimum period of 1 year to ensure the hand over of a flourishing green roof.

720	ADVERSE WEATHER

- Unfinished work: Secure from damage and wind uplift.

- Conditions: Do not install or work with frozen materials.

LANDSCAPING INSTALLATION

##### 770B	DRAINAGE LAYER INSTALLATION

- Extent: Continuous over designated roof area

- Fitting: Loose laid over the waterproofing and butt jointed with 100 mm fleece overlap

- Upstands: Carefully cut to fit closely around penetrations and outlets.

800A	VEGETATION BLANKET INSTALLATION

Handling blankets:

  - Timing: Lay within 36 hours of lifting from growing position.

  - Method: laid manually – two-man operation

  - Excessive stacking: Not permitted.

  - Material loss (maximum): 3% of total surface area.

Growing medium condition: Thoroughly watered

Laying blankets:

Dry, damaged, frosty or waterlogged blankets: Do not lay.

    - Orientation: Diagonal or perpendicular to slope of roof.

    - Joints: Stagger. Butt together or slightly overlap to prevent gaps. Do not stretch blankets. All excess vegetation should be removed from the overlap and the opposite leading edge of the blanket to ensure that the joints butt together tightly (as per the Bauder installation guideline).

    - Edges: Finish with whole blankets.

    - Consolidation: N/A

Dressing: Bauder Xero Flor substrate.

    - Application: Brush in to fill joints.

Watering: Thorough, immediately after laying and dressing.

    - Roll size: 2m x 1m

820A	EDGE RETAINING PROFILE INSTALLATION

- Cutting: Neat, accurate and without spalling.

- Junctions: use pre-formed 90º corners where required and connector pieces

- Position: True to line and level. Smooth continuous lines.

- Fixing: The SS40 trim is to be secured in place by separate pieces of torch applied Bauder Plant-E capping sheet cut into strips 1000mm x 200mm, these bituminous flashings should be torched through the holes in the trim to the waterproofing surface and set at intervals of 400mm between each one metre long flashing piece, bonding onto the main capping sheet by a minimum 100mm.

- Precautionary note: when cutting metal, please ensure that appropriate tools and personal protection equipment are used.

830E	PURPOSE FABRICATED CHUTE OPENING GRILLE PLATE)

- Location: Install/place centrally over chute opening.

- Orientation: Align parallel with parapet upstand kerb/wall.

- Bedding: N/A

- Backfill: 20/40 mm washed pebbles

- Surround: min. 500 mm

Provision should be made by the contractor to supply a ‘once bent’ perforated stainless steel grille plate cover, placed centrally to all chute outlet openings.  The plate should cover the whole of the opening and be constructed/designed to ensure that no growing medium or vegetation barriers pebble can pass through once installed.  This grille plate should feature 5 mm uniformly spaced perforations, each at intervals of between 2 mm up to a maximum of 10 mm spacings.  The plate should extend beyond the chute opening by at least 75 mm each side and the angle fillet will need to be locally omitted so that the plate can sit back flush against the kerb.

Bauder should be contacted if there are any queries relating to this item.

# 840	VEGETATION BARRIER / DRAINAGE BARRIER

- A minimum 300 mm pebble vegetation barrier must be provided to all perimeters and roof penetrations and  protrusions i.e. surrounds to roof light or plant upstand kerbs, rainwater outlets, soil vent pipes, man-safe posts etc. as specified. If there are adjacent buildings within 6 metres and the barrier functions also as a fire-break, then the width of the vegetation barrier should be increased to 500mm.   On very large roof areas, a one metre wide pebble fire break should be incorporated every 40 metres.

- We recommend 20/40 mm rounded river washed pebbles.  Stones or aggregates with sharp edges must not be used i.e. flint.  Other materials may be used, but only after consultation and agreement with Bauder Ltd.

- At all open drainage perimeters AL80/100 drainage trim must be used to contain the vegetation barrier.

- For extensive green roofs where sedum blanket is specified, the vegetation barrier must cover the edges of the blanket by a minimum of 100 mm in order to protect the exposed edge of the blanket against wind uplift and substrate erosion.

850A	FERTILISER

- Xero Flor organic slow release fertiliser must be applied at a rate of 80g/m² onto the newly laid XF301 sedum blanket. This is available from Bauder Ltd. Care must be taken to distribute the fertiliser evenly, through use of an approved applicator.

- The vegetation blanket should then be thoroughly saturated by the use of sprinklers so as to promote rapid establishment. It is the responsibility of the roofing contractor to liaise with the main contractor/ building owner to provide water to ensure that the vegetation mat does not dry out within the first month.

### COMPLETION

### 910	INSPECTION

- Timing: Before handover.

- Give notice (minimum): 3 days.

915A	INITIAL WATERING

- It is the responsibility of the roofing contractor to liaise with the main contractor/ building owner to provide water to ensure that the vegetation mat does not dry out within the first month.  An adequate mains supply of sufficient pressure must be available and operational prior to the sedum blanket being delivered and installed. Irrigation systems (where fitted on roof slopes above 10 degrees) must be operational, but initial watering in of the fertiliser must be by surface mounted sprinklers. See the Bauder Watering Guide document for detailed information on watering requirements.

- Bauder Ltd accepts no responsibility whatsoever for the condition of installed sedum blankets that are not properly watered in accordance with our recommendations.

916	POST INSTALLATION MAINTENANCE

- The installing contractor should price into the tender, the cost of carrying out post-installation maintenance for a contract period to be agreed with the client’s representative.

- Soft landscaping: Following the final maintenance visit and application of slow release fertiliser at the end of the agreed contract period, the responsibility for the ongoing maintenance of the green roof planting becomes the responsibility of the building owner.

- Period of maintenance contract:  Insert requirement i.e. one year, two years etc.

- Scope of maintenance procedure: as per manufacturer’s recommendations.

920	COMPLETION

- General: Leave the works in a clean, tidy condition.

- Surfaces: Clean immediately before handover.

- Outlets: Clean and clear of obstructions.

- Completed green roof: Protect from adjacent or high level working.

930	DOCUMENTATION

- Timing: Submit at handover.

- Contents:

- Manufacturers guarantees and warranties.

- Procedures for maintenance of the green roof.

- Record drawings showing the location of planting and associated features.

- Number of copies: as required by client.