Q37 Green Roofs

To be read with Preliminaries/General conditions

GENERAL

130	EXTENSIVE GREEN ROOF Biodiverse Wildflower & Sedum Green Roof

- Roof type: ______________

- Substrate: ______________

- Slope: ______________

- Waterproofing: ______________

- Thermal insulation: ______________

- Protection: ______________

- Moisture control layers: ______________

- Growing medium: Lindum Seeded Turf Ltd approved extensive green roof substrate

- Depth: 75mm minimum

- Vegetation: Lindum Biodiverse Wildflower & Sedum Mat

- Accessories: ______________

PERFORMANCE

210	GENERAL DESIGN

- Green roof and associated features: Complete the detailed design

- Proposals: Submit drawings, technical information, calculations, and manufacturer’s literature

- Performance criteria:

- To satisfy aesthetic, weatherproofing, insulation and structural performance criteria

- To satisfy native planting criteria to sit within the surrounding landscape

255	MAXIMUM PERMITTED GREEN ROOF LOADS

- Dead loads:

- Green roof layers: refer to engineer’s specification

- Imposed loads:

- Activity: refer engineer’s specification

- Vegetation: refer engineer’s specification

- Allowance for additional loads during construction: refer engineer’s specification

- Service loads: refer engineer’s specification

- Requirement: Restrict site activities to ensure that design loads are not exceeded, or submit proposals for temporary supports

PRODUCTS

330	DRAINAGE LAYER

- Manufacturer: Supplied by Lindum Seeded Turf Ltd

Tel: 01904 448675 Email: lindum@turf.co.uk

- Product Reference: Roofdrain 20

- Material: Recycled High Density Polyethylene water reservoir core with moisture retentive protective fleece to underside and geotextile filter mat to upper side

- Depth: 22mm

- Infill: not required

390	EXTENSIVE GROWING MEDIUM

- Manufacturer: Lindum Seeded Turf Ltd approved supplier

Tel: 01904 448675 Email: lindum@turf.co.uk

- Product Reference: Lindum Extensive Green Roof Substrate

- Material: 100% recycled crushed brick and green waste compost

- Depth: 75mm (minimum after settlement)

- Ameliorant/conditioner: not required

- Declaration of analysis: Submit

- Parameters: To BS3882, annexes A-E

- VEGETATION BLANKET

- Manufacturer: Lindum Seeded Turf Ltd

Tel: 01904 448675 Email: lindum@turf.co.uk

- Product Reference: Lindum Biodiverse Wildflower & Sedum Mat

- Planting Mix: Lindum biodiverse green roof mix

- Thickness: 25mm

- Material: 450gsm felt with scrim

- Vegetation coverage (minimum): 75%

440	VEGETATION BARRIER

- Material: Washed Pea Shingle

- Depth: 100mm

- Width: 200mm (minimum)

EXECUTION

710	INSTALLATION GENERALLY

- Preparation: Clear all surface of debris

- Timing: After certification of waterproof membrane integrity

- Surface condition: Visually inspect waterproof membrane, report any damage

- Faults in waterproof membrane: Report

- Contamination: Do not use material detrimental to healthy plant growth

- Storage: Do not overload

- Point loads: Avoid

- Outlets: Do not block

720	ADVERSE WEATHER

- Unfinished work: Secure form damage and wind uplift

- Conditions: Do not install or work with frozen materials

770	DRAINAGE LAYER INSTALLATION

- Joints: Minimize

- Extent: Continuous over entire roof area

- Fitting: Follow manufacturer’s recommendation

- Upstands: Fit closely around penetrations and outlets

- GROWING MEDIUM INSTALLATION

- Handling: Minimize

- Conditions: Handle in the driest condition possible.  Do not handle when wet or frozen

- Layers:

- Depth (maximum): 150mm

- Sequence: Gently firm each layer before spreading the next

800	VEGETATION BLANKET INSTALLATION

- Handling blankets:

- Timing: Lay within 36 hours of lifting from growing position

- Excessive stacking: Not permitted

- Material loss (maximum): 3% of the total surface area

- Growing medium condition: Thoroughly watered and raked level

- Laying blankets:

- Dry, damaged, frosty or waterlogged blankets: Do no lay

- Orientation: Diagonal or perpendicular to slope of roof

- Joints: Stagger. Butt together or slightly overlay to prevent gaps.  Do not stretch blankets.

- Edges: Finish with whole blankets

- Consolidation: Firm as laying proceeds to ensure full contact with the growing medium.  Do not use rollers.

- Dressing: Submit proposals

- Application: Brush in to fill joints

- Watering: Thoroughly, immediately after laying and dressing

COMPLETION

910	INSPECTION

- Timing: Before handover

- Give notice (minimum): 3 days

920	COMPLETION

- General: Leave the works in a clean and tidy condition

- Surfaces: Clean immediately before handover

- Outlets: Clean and clear of obstructions

- Completed green roof: Protect from adjacent of high level working

930	DOCUMENTATION

- Timing: Submit at handover

- Contents:

- Growing medium declaration of analysis

- Manufacturer’s guarantees and warranties

- Procedures for good establishment of the green roof to include manufacturer’s recommended watering schedule through establishment period

- Procedures for maintenance of the green roof

- Record drawings showing the location of planting and associated features

- Number of copies: To be confirmed