Bristol

Exeter

Teddington

Bournemouth

DESIGN AND ACCESS STATEMENT

FOR

THE REFURBISHMENT OF THE

THE KHALILI LECTURE

THEATRE IN

THE PHILLIPS BUILDING

THORNHAUGH STREET

RUSSELL SQUARE

LONDON

WC1H 0XG

ON BEHALF OF

THE SCHOOL OF ORIENTAL AND AFRICAN STUDIES

UNIVERSITY OF LONDON

Kendall Kingscott Limited

Sandford House

1B Claremont Road

Teddington

Middlesex

TW11 8DH

| Prepared By: | John Alexander | Date: | March 2015 |
| --- | --- | --- | --- |
| Checked By: | Gavin Cowling | Job No: | 130305-600 |

1.00      Introduction

This Design and Access Statement is submitted in support of a listed building consent application, which is submitted by Kendall Kingscott Ltd on behalf of SOAS, University of London. The application relates to the Khalili Lecture Theatre on the Lower Ground Floor of the Phillips Building. The property is a grade II listed building.

This application relates to the refurbishment of the Khalili Lecture Theatre, which will be refurbished to improve accessibility, audio visual installations and general amenity including new ceiling, lighting, seating and wall and floor finishes.

2.00	Existing Layout and Finishes

The Khalili Lecture Theatre currently consists of a projection room, access corridors, a front stage area with a ‘screen wall’, a lobby/foyer area. Seating accommodates 145 people. The theatre has a disabled access ramp to the side of the theatre.

3.00	Proposed Alterations

3.01	Walls

As part of the works the existing wall finish will be stripped out and renewed with new to improve acoustic performance of the space.

A new partition wall to the Khalili Lecture Theatre Lobby is to be formed. The works will also include a new plasterboard wall lining to be fixed over the block work wall to elevations 2 and 4.

3.02	Ceiling

A new suspended ceiling will be fitted throughout the lecture theatre and the Corridors 1 and 2 to improve acoustic performance.

3.03	Flooring

To the front of the stage a raised timber platform/ decking is to be provided to floor level increasing the stage size, finished with timber plank flooring. This will improve accessibility for wheelchair users and comply with. This will improve accessibility per The rest of the lecture theatre is to have carpet tiles fitted throughout.

3.04	Doors

The proposed works include replacing doors DK.02, DK.04, DK.05 with fire resistant doors incorporating vision panels and door DK.01 with a fire resistant door without a vision panel as shown on the drawings.

3.05	Inclined Stair Lift

The works also include improving accessibility by installing 2no. inclined platform lifts to the side of the Khalili Lecture Theatre in order to transfer wheelchairs from the rear of the theatre to the front stage area replacing the existing ramp as the gradient does not comply with building regulation guidance.

3.06	Seating

New seating is to be provided to the lecture theatre replacing the existing 145no. seats with 146no. intermediate row chairs with writing ledges and 10no. row chairs without ledges.

4.00	Access

N/A

5.00	Parking and Vehicular Access

N/A

6.00	Landscaping

The proposed works do not incorporate any external landscaping works.