46 ALBERT STREET, CAMDEN TOWN, LONDON NW1 7NU:

HERITAGE STATEMENT

The existing building and context

The property is a four storey terraced property at the northern end of a terrace from nos 22 – 48 Albert Street for which planning and listed building consent has been granted for the new mansard storey ( 2014/0948/P & 2014/1045/L).

The building and the terrace dates from 1845 and is listed Grade 2 and lies within the Camden Town Conservation Area.

The terrace was built formally as a block; the centre ( nos 34 and 36 ) and the end properties ( nos 22 /24 and 46 /68 & 69 Mornington Street ) project forward slightly to form projecting centre and ends to the terrace. The property has rendered lower ground and ground floors with moulded render surrounds to first and second floor windows and the first floor has a balcony with railings. The front parapet of no 46 does not have its original cornice and the parapet is finished in plain render.

Alteration proposal to front parapet

The owner of the house is now constructing the new mansard storey granted planning and listed building consent last year. It is apparent from the construction that the height of the front parapet acts as a considerable block to light and views out from the new storey.

The property is part of the formal terrace of properties and the proposal is cognisant of the location and position in the terrace. Most of the properties on the terrace – bar two – have mansard stories and in most of these the front parapet has been taken down in front of the dormer windows and in some cases across the whole frontage.

The proposal mirrors and matches the parapet alteration on nos 22 and 24 at the other ‘bookend’ of the terrace.

The alteration is designed in line with other alterations to the parapet on the properties along the terrace on adjacent properties. The alteration to the parapet is to be in line with the new mansard floor windows

The parapet is retained at the sides abutting the adjoining parapet.

The proposals have been designed in the context of retaining historic fabric and the mansard is designed as a secondary element to the existing mass and scale of the building.

ARP : May  2015

End of document