- METHOD STATEMENT

- 31 Great James Street London

- WORKS INVOLVED

.

- Residential Sprinkler Solutions Limited

- Unit 6a, Copthall Farm

- Breakspear Road South

- Ickenham

- Middlesex

- UB10 8HB

| Client: Loncross | Copy No. |
| --- | --- |
| Site: 31 Great James Street | Location: London |
| Issued By: P Moody | Date: 6th April 2010 |

- CONTENTS

Section		Title

- 1.0	Description/Scope of Work

1.1	General

1.2	Programme

1.3	Sequence and Method of Work

- 2.0	Resources

2.1	Personnel

2.2	Supervision

2.3	Plant and Equipment

3.0	Risks

3.1	Works Risk Assessments

3.2	COSHH Assessments

3.3	Manual Handling Assessment

- 4.0	Control Measures

4.1	Permit to Work

4.3	Security

4.4	Environmental Controls

- 5.0	Personal Protective Equipment

6.0	Emergency Arrangements

6.1	First Aid

6.2	Accident Reporting

6.3	Emergency Procedures

- 7.0	Distribution

7.1	Distribution/Changes

7.2	Personnel Issue/Familiarisation

- 8.0	Monitoring and Compliance

9.0	Appendices

1.0	DESCRIPTION/SCOPE OF WORKS

  - General

  - New Domestic Sprinkler Installation

(i)	Design

- The design requirements for this section of work will be carried out

From drawings supplied.

(ii)	Prefabrication of Pipework

- The Pipe work will be prefabbed on site.

- 1.0	DESCRIPTION/SCOPE OF WORKS

1.2	Programme

The Programme is to commence on 6th January 2013

1.3

Sequence of Work

- (i)	Introduction

The sprinkler riser pipe will run from basement to the 3rd floor in the designated riser cupboard, each floor will have a tee / outlet and pipe work will be installed through the joists to the desired areas, a sprinkler head will be installed at each sprinkler outlet.

The pipe work will be CPCV and the sprinkler heads are concealed Viking VK457

Pipe work will be prefabbed on site and Installed to Residential Sprinkler Solutions design drawings which will meet BS9251 & TB1, the system will be pressure tested once complete.

Commissioning will take place once the system has been connected to new 50mm incoming mains.

- (ii)	Site Works

- Works will be in accordance with the Programme.

.

- Area will be left clean and tidy after each visit

- 2.0	RESOURCES

- 2.1	Personnel

Personnel, both Company employed and sub-contracted, working on the Contract will be suitably competent and adequately trained commensurate with their job specific requirements and responsibilities

- 2.3	Plant and Equipment

(i) Access to the works will be by steps, A frame Ladders, Towers, Podiums

3.0	RISKS

- 3.1	Works Risk Assessments

Risk Assessments apply to our works as below:-

-	Use of Hand Tools

-	Working at Height

- 3.2	COSHH Assessments

Operations carried out on site which may present a hazard to health are:

-	Cement (glue)

-	Boss White Jointing Compound

The COSHH Assessment Record and related Operation/Substance Safety and Hazard Data and Health Risk Assessments (in accordance with Control of Substances Hazardous to Health Regulations) are enclosed (Appendix D)

- 3.0	RISKS (continued)

- 3.3	Access and Egress

- Access to and egress from the Site will be via Security.

- 4.0	CONTROL MEASURES

- 4.1	Permit to Work

The contract specific Permit to Work System will be utilised in designated areas as required by the Principal Contractor and/or the Client.

The Company Permit to Work system (Appendix G) will be implemented in designated work areas.

- 4.2	Plant & Operator Training

All operatives will be suitably trained for safety awareness and for the use of plant and equipment used within the works activities.

Training Certificates will be held both on site and centrally as record of personnel training/competency.

- 4.3	Security

All personnel working on site and visitors to the site will adhere to Site Rules relating to security.

All personnel and visitors will receive the necessary induction and record entry onto and departure from the Site, both with Security and on the Company Attendance Record (Appendix J)

- 4.4	Environmental Controls

Generation of dust and noise is minimal in our installation processes and generally does not present an environmental hazard

- Materials used/installed do not present any significant environmental hazards.

5.0	PERSONAL PROTECTIVE EQUIPMENT (PPE)

PPE will be issued as identified in the Personnel Protective Equipment (PPE) Assessment (Appendix K), used in accordance with recommendations detailed in the Personal Protective Equipment Regulations and as identified in the appropriate Risk Assessments, refer Appendix C.

Typical PPE will include safety helmets, safety boots, and hi-vi vest. Goggles, Gloves.

Safety helmets and industrial footwear will always be worn during working operations.

- 6.0	EMERGENCY ARRANGEMENTS

- 6.1	Accident Reporting

All accidents, minor or major, dangerous occurrences or near misses must be reported to the Site Safety Personnel and the Company Project Engineer immediately. The Project Engineer will also report all accidents to the Quality and Safety Manager and record details of the accident in the appropriate Company Accident Book (Form BI  510).

In the event of a serious accident or dangerous occurrence, as identified in the Reporting of Injuries, Diseases and Dangerous Occurrences Regulations (RIDDOR), the Quality and Safety Manager will:

-	report any accidents or dangerous occurrences to the appropriate HSE office on the prescribed form (F2508, appropriate designation)

-	carry out any investigation deemed appropriate

-	Provide copies of all documentation to space pod and Residential Sprinkler Solutions as necessary.

- 6.2	Emergency Procedures

Emergency procedures areas defined at Induction and as identified in the Site Safety documentation.

In the event of Site Evacuation all Company personnel and visitors must report to the Company Site Supervisor at the designated assembly point.

- 6.3	First Aid

- 1st Aider – Jamie Grant (RSSL)

- 7.0	DISTRIBUTION/CHANGES

- 7.1	Changes/Issue

The Method Statement and related Appendices is a controlled document and will be issued as such to recipients/locations as below:

-	Contract File			(Copy no 1)

-	Site				(Copy no 2)

-	Principal Contractor / Client	(Copy no 3)

-	Other nominated recipients	(Copy number(s) to be allocated)

Control of the revision status and issue of the documentation will be by utilisation of the Drawing Register.

- Changes/additions will be identified by vertical line to the right of the amended text.

Evidence of issue will be either by utilisation of the Transmittal Form (Form 4.4E) or by signing the Health & Safety Confirmation Record (Appendix L).

Documentation issued will be controlled and documents retained in appropriate locations as required by our Management System, specifically QAP's 4.5 - Document and Data Control and QAP 4.9 - Process Control.

- 7.2	Personnel Issue/Familiarisation

All personnel working on Site will receive an induction into the contents of the Method Statement and related documentation and will be required to read and familiarise themselves with the contents of the documentation.

Evidence of the induction and familiarisation will be recorded by personnel signing the Health & Safety Confirmation Record. Copies of the signed document may be provided to the Principal Contractor if they require.

8.0	MONITORING AND COMPLIANCE

Control and monitoring arrangements are as detailed in our Management System, specifically QAPs 4.9 Process Control and 4.10 Inspection and Testing. Brief details of these procedures are contained in Section 4.0 of the Quality Assurance Manual.

Failure to comply with Site specific or Company requirements will be recorded onto the appropriate Non-Conformance Register.  Serious or repetitive breaches of requirements will result in issue of a Non-Conformance Report. These operations are detailed in our Management System, specifically QAP 4.13 Control of Non-conforming Product.

Issue of a Non-Conformance Report is deemed to be a written warning and may lead to dismissal from Site.

- 9.0	APPENDICES

- Project Risk Assessment Records and related Summaries

Project COSHH Assessment Records and related Assessments

Permit to Work

Company Attendance Record

Personal Protective Equipment (PPE) Assessment

Health & Safety Confirmation Record

| RISK ASSESSMENT SUMMARY (RA 001) (Part 1 of 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Contract Number: 142 Site: 31 GREAT JAMES STREET |  |  |  |  |  |
| Work Activity: General Manual Handling of Materials and Equipment |  |  |  |  |  |
| Persons Exposed: |  | Comments: (include reference to disabled, trainees etc) |  |  |  |
| Employees |  | No disabled or trainees |  |  |  |
| Other Workers | x |  |  |  |  |
| Public | x |  |  |  |  |
| Hazards Identified  If present/requiring control x if absent/not significant |  |  |  |  |  |
| Physical Injury |  | Physical Agents Manual Handling |  | Manual Handling |  |
| Mobile Plant |  | Ionising Radiation | x | Manual Handling Injury |  |
| Moving Parts of Machine | x | Lasers | x |  |  |
| Moving Materials |  | Ultraviolet Light | x | Miscellaneous |  |
| Falls from Heights |  | Cold Objects |  | Weather | x |
| Access Equipment |  | Hot Objects |  | Lone Working | x |
| Slips Trips Falls |  | Temperature | x | Confined Spaces | x |
| Excavations | x | Noise/Vibration | x | Restricted Access | x |
| Pressurised Systems | x | Hazardous Substances | x |  |  |
| Electrical | x | Micro Organisms | x | Other |  |
| Hot Work/Fire | x | Vermin/Weils Disease | x | Other |  |
| Explosion | x |  |  | Other |  |
| If Other (specify) |  |  |  |  |  |
| Specific Site Conditions - Ensure good housekeeping - Ensure access route is clear to enable safe progress with the load |  |  |  |  |  |

| Risk Evaluation |  |  |  |
| --- | --- | --- | --- |
| Score 1 2 3 Unlikely Possible Likely Minor Moderate Serious | Likelihood of Harm Score 1 - 3 | Severity Score 1 - 3 | Risk Number Scores Multiplied |
| Physical Injury | 3 | 2 | 6 |
| Physical Agents | 1 | 1 | 1 |
| Hazardous Substances | 1 | 1 | 1 |
| Manual Handling | 3 | 2 | 6 |
| Miscellaneous |  |  |  |
| State Key Risks (use Risk Evaluation Score for priorities) 5+ - Back strain, foot injury (heavy items), cuts and abrasions (sharp objects) |  |  |  |
| Control Measures - see Risk Assessment Summary Form 2 attached |  |  |  |
| Revision Status: 00 Date: |  |  |  |

- Form 4.9L/00

| RISK ASSESSMENT SUMMARY (RA 001) (Part 2 of 2) |  |
| --- | --- |
| Contract Number: 142 Site: 31 GREAT JAMES STREET |  |
| Work Activity: General Manual Handling of Materials/Equipment Significant Risks: Back Strain Injury, Foot Injury (Heavy Items). |  |
| Control Item | Details of Control Measures |
| Documents Procedures etc | General Company Policy on Manual Handling. |
| Information | Operatives are advised of risk of back strain and foot injuries arising from a wide variety of tasks to be carried out on site. |
| Instruction | Operatives required to avoid manual handling which they believe may cause them injury - beyond their capacity. To make use of lifting aids, hoists etc, wherever practical. To seek assistance from colleagues (team work) for heavy/awkward tasks. |
| Training | As part of introduction and regular refresher briefings - operatives advised and reminded of good lifting techniques, to use the legs not the back etc. To use mechanical aids wherever practical. Operatives many only use mechanical devices such as goods hoists after receiving formal training in their correct use, including weekly inspections. |
| Supervision | Constantly remind operatives of the need to use aids and lift correctly without bending the back. Review job tasks, storage arrangements, access, equipment, environment (see below) etc, to minimise unnecessary materials and equipment movement. |
| Access | Poor access arrangements may impose additional constraints on good manual handling - to be reviewed by Managers/Supervisors for each Contract. |
| Environment | Good lighting, firm footing and other items such as handrails are required to minimise risk of injury. |
| Equipment | Provision of mechanical aids suitable for the job and site. Hoists to be inspected weekly, examined every 6 months (entry on Statutory Register). |
| Emergencies | Standard site first aid, fire protection, evacuation, accident reporting and investigation. |
| Communications | Not applicable. |
| COSHH | Not applicable. |
| PPE | Back support belts may be used, but should not be taken to warrant increasing the weight an operative may safely lift considering his personal capacity. |
| Other Procedures | Buyers and Contracts Managers to review the materials ordered and where practical purchase in suitable sizes for handling (i.e. 25kg bags of a material rather than 50kg bags). Site design to establish storage areas close to work to minimise handling distances. HSE guidance available: Eye Bolts - PM16; Cable Laid Slings and Grommets - PM20; Lifting Gear Standards - PM 54. |

The above controls have been selected to protect the health and safety of operatives and others who may be affected by the work. The controls have been designed to protect against the risks recorded on Risk Assessment Summary Part 1. This general guidance should only be used when it is appropriate to specific site conditions.

- Form 4.9L/00

| RISK ASSESSMENT SUMMARY (RA 003) (Part 1 of 2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Contract Number: 142 Site: 31 GREAT JAMES STREET |  |  |  |  |  |
| Work Activity: Use of Hand Tools |  |  |  |  |  |
| Persons Exposed: |  | Comments: (include reference to disabled, trainees etc) |  |  |  |
| Employees |  | Poorly maintained hand tools may present risks to all personnel/others due to ejected parts |  |  |  |
| Other Workers |  |  |  |  |  |
| Public |  |  |  |  |  |
| Hazards Identified  If present/requiring control x if absent/not significant |  |  |  |  |  |
| Physical Injury Hazards |  |  |  | Physical Agents Manual HandlingManual Handling |  |
| Mobile Plant | x | Ionising Radiation | x | Manual Handling Injury |  |
| Moving Parts of Machine | x | Lasers | x |  |  |
| Moving Materials |  | Ultraviolet Light | x | Miscellaneous |  |
| Falls from Heights | x | Cold Objects | x | Weather |  |
| Access Equipment | x | Hot Objects | x | Lone Working | x |
| Slips Trips Falls |  | Temperature | x | Confined Spaces |  |
| Excavations | x | Noise/Vibration | x | Restricted Access |  |
| Pressurised Systems |  | Hazardous Substances | x |  |  |
| Electrical |  | Micro Organisms | x | Other | x |
| Hot Work/Fire | x | Vermin/Weils Disease | x | Other | x |
| Explosion | x |  |  | Other | x |
| If Other (specify) |  |  |  |  |  |
| Specific Site Conditions - No specific site elements which present any additional risks to works |  |  |  |  |  |

| Risk Evaluation |  |  |  |
| --- | --- | --- | --- |
| Score 1 2 3 Unlikely Possible Likely Minor Moderate Serious | Likelihood of Harm Score 1 - 3 | Severity Score 1 - 3 | Risk Number Scores Multiplied |
| Physical Injury | 3 | 2 | 6 |
| Physical Agents | 2 | 2 | 4 |
| Hazardous Substances | 2 | 1 | 2 |
| Manual Handling | 3 | 2 | 6 |
| Miscellaneous |  |  |  |
| State Key Risks (use Risk Evaluation Score for priorities) 5+ - Minor cuts & abrasions, debris in eyes, injury from damaged tools |  |  |  |
| Control Measures - see Risk Assessment Summary Form 2 attached |  |  |  |
| Revision Status: 00 Date: |  |  |  |

- Form 4.9L/00

| RISK ASSESSMENT SUMMARY (RA 003) (Part 2 of 2) |  |
| --- | --- |
| Contract Number: 142 Site: 31 GREAT JAMES STREET |  |
| Work Activity: Use OF Hand Tools Significant Risks: Eye Injury, Other Minor Physical Injury |  |
| Control Item | Details of Control Measures |
| Documents Procedures etc | Operatives are responsible for ensuring that their own tools are in good condition and are the correct tools for the job. |
| Information | Operatives are advised of risk of eye injuries and other minor injuries which can arise from poor maintenance or incorrect use of hand tools and extension leads. |
| Instruction | Operatives are required to inspect their tools before each task and ensure that they are in good condition. |
| Training | Aspects of tool standards are incorporated and general training and specific training for specific tasks requiring a higher level of competency. |
| Supervision | Remind operatives occasionally of the need to check their own tools. Intervene if operative is identified using a poorly maintained or inappropriate hand tool. |
| Access | Not applicable. |
| Environment | Not applicable. |
| Equipment | Hammers: Head secure to shaft, shaft smooth (no splits). Head in good condition - no chips, not round edged. Handles not bound with any material. |
|  | Chisels: Used with eye protection. Kept in good condition - sharp, without mushroom heads, heads free from oil and grease. |
|  | Screwdrivers: Not carried in pockets, correct size used with work, workpiece not held in free hand. |
|  | Electric Drills: Checked for electrical integrity regularly, operating on 110V. Correct drill for material selected, kept sharp. Check before drilling to ensure avoiding services. |
| Emergencies | Standard site first aid, fire protection, evacuation, accident reporting and investigation. |
| Communications | Not applicable. |
| COSHH | Not applicable. |
| PPE | Eye protection required when using cold chisels, electric drills etc. |
| Other Procedures | When using tools on or adjacent to electrical equipment, ensure that equipment is isolated and locked off. |
| Maintenance | Some tools, such as chisels require periodic maintenance to use them safely. |

The above controls have been selected to protect the health and safety of operatives and others who may be affected by the work. The controls have been designed to protect against the risks recorded on Risk Assessment Summary Part 1. This general guidance should only be used when it is appropriate to specific site conditions.

- Form 4.9L/00

| HEALTH AND SAFETY CONFIRMATION RECORD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Contract Number: 142 Contract Title: 31 GREAT JAMES STREET |  |  |  |  |
| I confirm receipt and/or view of the Method Statement and related Risk and COSHH Assessments for the Contract. (Revision status and copy number as below). I confirm that the contents and requirements have been explained and that I have read and understand the Method Statement and the Risk Assessments and that I will work in accordance with the requirements detailed. |  |  |  |  |
| Rev Status | Copy Nbr | Date | Signed | Print Name |

Appendix L                                                                                       	Form 4.9N/01

| PERSONAL PROTECTIVE EQUIPMENT (PPE) ASSESSMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contract Number: 142 Contract Title/Location: 31 GREAT JAMES STREET |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ASSESSMENT CARRIED OUT BY: SIGNATURE: DATE: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RISKS | PARTS OF THE BODY AT RISK |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CRANIUM | EARS | EYES | RESP. TRACT | FACE | WHOLE HEAD | HANDS | ARMS (parts) | FEET | LEGS (parts) | SKIN | TRUNK/ ABDOMEN | WHOLE BODY |
| FALLS FROM HEIGHT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BLOWS,IMPACT,CUTS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STABS,CUTS GRAZES |  |  |  |  |  |  |  |  |  |  |  |  |  |
| VIBRATION |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SLIPS,TRIPS,FALLS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HEAT/FIRE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| COLD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IMMERSION |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NON-IONISING RAD. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ELECTRICAL |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOISE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IONISING RADIATION |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DUST/FIBRES |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FUMES |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SPLASHES/SPURTS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GASES/VAPOURS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OTHER FACTORS WHICH MAY BE RELEVANT IN USE OF PPE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IS A COMBINATION OF PPE (IF APPROPRIATE) A FACTOR? |  |  |  |  | YES/NO |  | IS VISIBILITY A FACTOR IN CHOICE OF PPE? |  |  |  |  | YES/NO |  |
| IS PHYSICAL EFFORT A FACTOR IN CHOICE OF PPE? |  |  |  |  | YES/NO |  | IS COMMUNICATION A FACTOR IN CHOICE OF PPE? |  |  |  |  | YES/NO |  |
| ANY OTHER FACTORS IN CHOICE OF PPE? (SPECIFY) |  |  |  |  | YES/NO |  | NOT APPLICABLE ON CONTRACT |  |  |  |  |  |  |

- Appendix K

| ATTENDANCE RECORD |  |  |  |
| --- | --- | --- | --- |
| Contract Number: 142 Contract Title/Location: 31 GREAT JAMES STREET |  |  |  |
| Date: |  | Name (Signature) | Print Name |
| Time in | Time out |  |  |

Appendix J

PERMIT TO WORK (PTW) PROCEDURE Page 1 of 3

Contract Number: 142 Contract Title/Location: 31 GREAT JAMES STREET

Method of Issue/Control

1. The Contracts Manager and/or Project Engineer, where appropriate in association with the Client and / or Main Contractor will identify areas within the contract to be controlled by a Permit to Work.

2. All such areas will be clearly identified on appropriate documentation / drawings and to appropriate personnel, both supervisory and operatives.

3. Permits to Work (Appendix PTW1) may only be issued by either the Contracts Manager or the Project Engineer (hereafter called the originator).

4. To generate a Permit to Work, the originator will; i) Allocate the next sequential PTW from the Permit to Work Register (PTW2) ii) Generate the Permit to Work (identified with the PTW number) iii) Record details of the Permit to Work in the Register

5. The Permit to Work will detail the following: i) Personnel requiring entry ii) Responsible Person iii) Location/Area of Work iv) Details of work to be carried out v) Equipment/PPE to be used vi) Period of Access - date, time of entry and time of exit

6. The Permit to Work will be signed by the recipient (person in charge of works to be carried out) and the originator.

7. Both entries will be dated and timed accurately.

8. Upon completion and signing of the Permit to Work, a copy will be made and issued to the responsible person.

9. After issue, the originator will enter necessary details on the Permit to Work Register. N.B. Permits to Work will be issued on a daily basis.

10. Upon completion of works, the responsible person will report to the Contracts Manager or Project Engineer to inform of completion.

11. The Contracts Manager or Project Engineer will: i) Cancel the original Permit to Work ii) Attach the copy surrendered by the responsible person iii) Record cancellation details on the Permit to Work Log

12. The Permit to Work Register and related Permits to Work, both live and surrendered, will be retained in a suitable location on site.

Appendix G

PERMIT TO WORK (PTW) PROCEDURE Page 2 of 3

Contract Number: 142 Contract Title/Location:31 GREAT JAMES STREET

Permit To Work PTW Number:

Personnel requiring entry:

Responsible person:

Location/Area of works:

Details of work to be carried out:

Equipment/PPE to be used:

Period of Access: Date: From: am/pm To: am/pm

ISSUE OF PERMIT P.T.W. received by: Signed: Print Name: Person in charge of works to be carried out and I confirm that one man working will not be permitted at any time. Date: Time: am/pm

P.T.W. issued by: Signed: Print Name: Date: Time: am/pm

CANCELLATION OF PERMIT Permit cancelled by: Signed: Print Name: Date: Time: am/pm

Appendix G

| PERMIT TO WORK (PTW) PROCEDURE Page 3 of 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contract Number: 142 Contract Title/Location: 31 GREAT JAMES STREET |  |  |  |  |  |  |  |
| PERMIT TO WORK REGISTER |  |  |  |  |  |  |  |
| Date Issued | Ptw Nbr | Area of Work | Issued to | Work Period | Nbr of Persons | Issued by | Date & Time Cancelled |
|  |  |  |  | From: am/pm To: am/pm |  |  |  |
|  |  |  |  | From: am/pm To: am/pm |  |  |  |
|  |  |  |  | From: am/pm To: am/pm |  |  |  |
|  |  |  |  | From: am/pm To: am/pm |  |  |  |
|  |  |  |  | From: am/pm To: am/pm |  |  |  |
|  |  |  |  | From: am/pm To: am/pm |  |  |  |
|  |  |  |  | From: am/pm To: am/pm |  |  |  |

Appendix G

MANUAL HANDLING ASSESSMENT Page 1 of 2

Contract Number: 142 Contract Title/Location: 31 GREAT JAMES STREET

METHOD OF ASSESSMENT To comply with the Manual Handling Regulations, the company will: 1. Identify the manual handling operations to be undertaken by their employees. 2. Make an initial appraisal of all operations to determine if there is a risk of injury to employees. 3. It is anticipated that the tasks where manual handling will be the subject of a specific Method Statement will be:- - fabricated pipes and related components - pumps - ductile iron pipework - large bore steel pipe and accessories. When details of these products and methods of installation / routes to installation locations are finalised, a full assessment will be carried out. This will be by utilisation of the Assessment Record . Upon completion of the assessment, a suitable Method Statement will be generated and inserted into the Project Method Statement. 4. The Assessment will review the following: - the task - the load - the working environment - the individual capacity of personnel - any other factors 5. After the assessment has been carried out, the risks will be analysed to endeavour to reduce / minimise them.

Appendix E

| MANUAL HANDLING ASSESSMENT Page 2 of 2 |  |  |
| --- | --- | --- |
| Contract Number: 142 Contract Title/Location 31 GREAT JAMES STREET |  |  |
| ASSESSMENT RECORD |  |  |
| Factors | Operations | Y/N |
| 1. The Task | Do they involve: |  |
|  | - holding or manipulating loads at a distance from trunk? | N |
|  | - unsatisfactory body movements or posture, especially: |  |
|  | - twisting? | Y |
|  | - stooping? | Y |
|  | - reaching upwards? | Y |
|  | - significant movement of loads, especially: |  |
|  | - lifting or lowering distances? | N |
|  | - carrying distance? | Y |
|  | - pushing or pulling of loads? | N |
|  | - risk of sudden movement of loads? | N |
|  | - frequent or prolonged physical effort? | Y |
|  | - insufficient rest or recovery periods? | N |
|  | - a rate of work imposed by a process? | N |
| 2. The loads | Are they: |  |
|  | - heavy? | Y |
|  | - bulky or unwieldy? | Y |
|  | - difficult to grasp? | N |
|  | - unstable or with contents likely to shift? | N |
|  | - sharp, hot or otherwise potentially damaging? | Y |
| 3. The Working Environment | Are there: |  |
|  | - space constraints preventing good posture? | Y |
|  | - uneven, slippery or unstable floors? | Y |
|  | - variation in level of floors or work surfaces? | Y |
|  | - extremes of temperature or humidity/ | N |
|  | - conditions causing ventilation problems or gusts of wind? | N |
|  | - poor lighting conditions? | Y |

Appendix E

| PROJECT COSHH ASSESSMENT RECORD |  |  |  |
| --- | --- | --- | --- |
| Contract Number: 142 Contract Title/Location: 31 GREAT JAMES STREET |  |  |  |
| COSHH Assessment Reference | Product and/or Trade Name | Rev: | Applicable yes/no |
| C 001 | Boss White Jointing Compound | 00 |  |
| KEY:  = yes X = no Page 1 of 1 |  |  |  |
| CARRIED OUT BY: SIGNED: TITLE: DATE: |  |  |  |

Appendix D

| PROJECT RISK ASSESSMENT RECORD |  |  |  |
| --- | --- | --- | --- |
| Contract Number Contract Title/Location: 31 GREAT JAMES STREET |  |  |  |
| Risk Assessment Reference | Nature of Work/Risk | Rev: | Applicable yes/no |
| RA 001 | Manual Handling | 00 |  |
| RA 003 | Use of Hand Tools | 00 |  |
| RA 004 | Working at Height | 00 |  |
| KEY:  = yes X = no |  | PAGE 1 OF 1 |  |
| FOR RISK RATING - SEE RISK ASSESSMENTS (SHEET 1) |  |  |  |
| CARRIED OUT BY: SIGNED: TITLE: DATE: |  |  |  |

Appendix