| Ref. | HAZARD | Persons who may be harmed | Risk Assessment (No Controls) |  |  | CONTROL MEASURES – Apply Hierarchy of Control in identifying control measures: 1. Elimination 2. Substitution 3. Engineering Controls 4. Signage / warnings and / or administrative controls 5. Personal Protective Equipment (PPE) | Risk Assessment (With Controls) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | P | S | R |  | P | S | R |
| 1 | Demolition dust | Workforce & Public | 4 | 4 | H | Demolition dust will be controlled by the below methods- - Solid hoarding erected to the perimeter of the site - All demolition to be completed behind scaffold wrapped in Monarflex. - Dust generated to be suppressed by water locally. | 4 | 1 | L |
| 2 | Dust/Fumes from Delivery vehicles | Workforce & Public | 2 | 2 | L | Dust/Fumes from Delivery vehicles will be controlled by the below methods- - Low emission vehicles to be used were possible - Delivery vehicles to be kept clean including wheels - Dust generated to be suppressed by water locally. - All loads entering and leaving the site must be covered | 2 | 1 | L |
| 3 | Dust from excavations | Workforce & Public | 4 | 3 | H | Excavation dust will be controlled by the below methods- - Dust generated to be suppressed by water locally. - Stocked piled materials from excavations to be covered until removed from site | 4 | 1 | L |
| 4 | Dust/fumes-from cutting/grinding | Workforce & Public | 4 | 4 | H | Dust and fumes from cutting will be controlled by the following- - Cutting equipment to use water suppressant and local exhaust ventilation systems. | 4 | 1 | L |

Additional Controls Agreed: - Appropriate PPE must be provided for the specific task. - Task specific Risk Assessments to be provided and approved by Gilbert-Ash for each work activity relating to above - LEV systems to be approved via Gilbert-Ash - Site monitoring will be undertaken in line with MOL BPG and logs kept of any complaints from the public.