## HAND ARM VIBRATION POLICY

Hand Arm Vibration Policy Statement

Demo One Limited is committed to meeting the requirements of the Health and Safety Executive and Management of Health and Safety at Work Regulations in identifying and eliminating or reducing exposure to vibration that can cause Hand Arm Vibration

1. Hand Arm Vibration

Hand Arm Vibration (HAV) is vibration transmitted from work processes into worker’s hands and arms. It can be caused by the operation and use of hand held power tools such as road breakers and hammer drills, or by holding materials being processed by machines such as pedestal grinders. Regular and frequent exposure to high levels of vibration can lead to permanent injury. This is most likely when contact with a vibrating tool or process is a regular part of a person’s job. Regular exposure to HAV can cause a range of permanent injuries to hands and arms that result in loss of sense or touch, severe pain and numbness, pins and needles, a loss of grip strength and pain in the wrists.

2. What is required under the regulations.

The requirements of the Management at Work Regulations are that an assessment of the risk to employees from using equipment that can cause Hand Arm Vibration is carried out, and managing that risk. (see attached checklist)

3. Action by Demo One Ltd.

Demo One Ltd. to ensure the following:-

-  That the right equipment for the job is provided and that it is maintained.

-  That operatives are using the equipment correctly, and for the right job.

-  That job rotation is introduced to reduce exposure to vibration.

-  That poor posture is avoided by designing the job properly.

-  That tools are maintained to the manufacturer’s specifications. For example:

-replace vibration mounts before they are worn out.

-ensure rotating parts are checked for balance.

-keep tools sharp.

-  That advice from the equipment manufacturer is sought on safe use of equipment.

-  That health surveillance of employees is provided where risks cannot be completely eliminated.

-  Employees should be advised that they must report problems with their hands and arms.

As part of the Induction and ‘tool box’ talk process Employees must receive information and training on Health Risks and the safe use of the equipment including being advised that health surveillance will be provided where risks cannot be completely eliminated.

That training to include: -

- Instruction on the cause of hand arm vibration and the proper use of any control measures it is

proposed be implemented.

- Instruction on how to use tools to reduce grip force, strain etc.

- Instruction to employees to advise and report as to any problems with equipment.

- Instruction to employees to maintain blood circulation by preventing hands and fingers from

becoming cold, exercising fingers and not smoking.

- Instruction not to stand on cold floors.

- Instruction to operatives to report any tingling or throbbing in their hands.

4. Risk Assessment - Alternative Method.

Look for alternative ways of working which eliminate the vibrating equipment altogether.

5. Can the job be done in a safer way?

If vibration cannot be eliminated from the work, can you do the job in a safer way by limiting exposure or protecting the worker.

6. Health Checks.

If employees are likely to be exposed to vibration they should have hand inspections carried out by the Company Safety Officer, if problems are identified they should be referred to their GP.

The checklist attached is to be completed for each site to aid in identifying and managing equipment that cause HAV

Signed ………………………………….	Date ……5-01-16…………….

Mr David McGrath – Chairman

| # HAND ARM VIBRATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name of Site: |  |  |  | Date: |
|  | ## YES | ## NO | ## INFORMATION/ACTION |  |
| ## Work Equipment |  |  |  |  |
| Have you identified hand held equipment used on site that produces vibration? |  |  |  |  |
| Can you identify an alternative way of carrying out this work? |  |  |  |  |
| Have tools using least vibration been selected? |  |  |  |  |
| Have attempts been made to reduce exposure at source. |  |  |  |  |
| Has equipment been tagged or marked with vibration data. |  |  |  |  |
| Have risk assessments been made on each piece of plant to reduce risks and identify control measures. |  |  |  |  |
| Is there a Planned Maintenance Schedule in place. |  |  |  |  |
| ## Workers |  |  |  |  |
| Have those workers exposed to vibration been identified? |  |  |  |  |
| Have those workers been provided with relevant information about their exposure to vibration? |  |  |  |  |
| Have those workers been provided with information about the symptoms of Hand Held Vibration and who to report concerns to? |  |  |  |  |
| Is job rotation required? |  |  |  |  |
| Do you carry out visual inspections of workers hands? |  |  |  |  |
| ## Controls |  |  |  |  |
| Are the controls identified in your risk assessment in place? |  |  |  |  |
| Is vibration exposure monitored? |  |  |  |  |
| Is equipment being maintained in accordance with the schedule? |  |  |  |  |
| ## Records |  |  |  |  |
| Are written records available of: |  |  |  |  |
| Risk Assessments |  |  |  |  |
| Hand Inspections |  |  |  |  |
| Information and training given |  |  |  |  |

| HAND ARM VIBRATION SAFETY GUIDE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Hand Vibration | Exposure Action | Exposure Limit | Noise Output |
|  |  |  | Value | Value |  |
|  |  |  | (2.5m/sec2 A(8)) | (5m/s2 A(8)) |  |
|  |  | (m/s2) | (mins) | (mins/hrs) | (dB) range-1m |
| HAV Rating | BREAKERS / DRILLS |  |  |  |  |
|  | AIR TOOLS |  |  |  |  |
|  | Medium duty breaker (28kg) | 9.9 | 31 | 122 | 107 |
|  | Medium duty breaker(V/Damped) | 3.6 | 231 | >8hrs | 107 |
|  | Heavy duty breaker (32kg) | 8.6 | 41 | 162 | 110 |
|  | Heavy duty breaker (V/Damped) | 4.6 | 142 | >8hrs | 110 |
|  | HYDRAULIC BREAKERS |  |  |  |  |
|  | Medium duty gun (25kg) | 10.5 | 27 | 109 | 93 |
|  | Medium duty gun (V/Damped) | 5.5 | 99 | 397 | 93 |
|  | Heavy duty gun (31kg) | 22 | 6 | 25 | 93 |
|  | Heavy duty gun (V/Damped) | 7.5 | 53 | 213 | 93 |
|  | ELECTRIC BREAKERS |  |  |  |  |
|  | Heavy duty breaker (30kg) | 8 | 47 | 188 | 108 |
|  | H-Duty Demo hammer (11kg) | 14 | 15 | 61 | 101 |
|  | H-Duty Demo hammer-Anti/Vib | 8 | 47 | 188 | 98 |
|  | M-Duty Demo hammer (7kg) | 9 | 37 | 148 | 102 |
|  | M-Duty Demo hammer-Anti/Vib | 7 | 61 | 290 | 98 |
|  | L-Duty Demo hammer (5kg) | 14 | 15 | 61 | 101 |
|  | L-Duty Demo hammer-Anti/Vib | 6 | 83 | 333 | 97 |
|  | 2-STROKE BREAKERS |  |  |  |  |
|  | Atlas Copco Kango | 10 | 30 | 120 | 103 |
|  | ROTARY/HAMMER DRILLS |  |  |  |  |
|  | 9kg hammer | 14 | 15 | 61 | 103 |
|  | 6kg hammer | 7 | 61 | 290 | 106 |
|  | 4kg hammer | 8 | 47 | 188 | 103 |
|  | 3kg hammer | 10 | 30 | 120 | 101 |
|  | ROTARY DRILLS |  |  |  |  |
|  | 4kg drill | 2.5 | 480 | >8hrs | 99 |
|  | SAWING / CUTTING / GRINDING |  |  |  |  |
|  | CUT OFF SAW |  |  |  |  |
|  | 2-Stroke 12" | 7.6 | 52 | 208 | 107 |
|  | 2-Stroke 12" Anti Vibration | 4.6 | 142 | >8hrs | 106 |
|  | GRINDERS |  |  |  |  |
|  | 4/5" Mini Grinder | 2.5 | 480 | >8hrs | 85 |
|  | 7/9" Grinder | 3 | 333 | >8hrs | 104 |
|  | CIRCULAR SAWS |  |  |  |  |
|  | 6"-9" Saw | 2.5 | 480 | >8hrs | 106 |
|  | WALL CHASERS |  |  |  |  |
|  | Single blade | 3 | 333 | >8hrs | 111 |
|  | Twin blade | 4 | 188 | >8hrs | 111 |
|  | FLOOR TREATMENT |  |  |  |  |
|  | 14" Petrol saw | 7.5 | 53 | 213 | 105 |
|  | 18" Petrol saw | 4.5 | 148 | >8hrs | 100 |
|  | Key RED |  | Over 10m/S2 |  |  |
|  | AMBER |  | 5-10m/s2 |  |  |
|  | GREEN |  | Up to 5m/s2 |  |  |

|  |  |  | DEMO ONE LIMITED HAVS RECORD SHEET |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | OPERATIVE NAME: |  |  |  |  |  | OPERATIVES SIGNATURE: |  |  |  |  |  |  |  |  |  | W/K ENDING |  |  |  |  |
|  | MANAGERS NAME: |  |  |  |  |  | MANAGERS SIGNATURE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ITEM OF PLANT |  | MAX USAGE |  | MAX USAGE |  |  | USAGE PER DAY IN MINUTES |  |  |  |  |  |  |  |  |  |  |  | WEEKLY |  |
|  |  |  | PERIOD IN 8HRS |  | PERIOD IN 1 HR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | (MINUTES) |  | (MINUTES) |  |  | MON |  | TUE |  | WED |  | THURS |  |  | FRI | SAT | SUN | TOTAL PER TOOL |  |
|  |  |  |  |  | TOTALS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | TOTAL PER WEEK |  |  |  |  |  |  |  |