Construction Environment Management Plan ( CEMP);

Works at 4 Gloucester Crescent, Camden

The CEMP addresses issues associated with:

- Noise and Vibration

- Air Quality

- Visual Impact

- Ecology

- Water Pollution

- Waste

- Contaminated Land

- Construction Traffic Management

- Protection of Existing Services

- Community Responsibility

It should be noted that this is a retrospective application being made after the client started work in ignorance of the extent of the Listed Building control of the work.

The work extent is limited to the remaining work

- Introduction

This Construction Environmental Management Plan (CEMP) is for the construction phase of

the development, and sets out the intended methods of effectively managing potential

environmental impacts arising from the construction.

The responsibility for implementation of the CEMP lies with the Principal Contractor

In order to ensure that the plan remains relevant it will be the responsibility of

The Principal Contractor to take ownership of the CEMP and ensure its relevance to activities being

undertaken on site in light of any changes from the initial scope of the plan, this requires its

regular revision and updating as necessary.

- CEMP Overview

This CEMP identifies the project management structure roles and responsibilities with regard

to managing and reporting on the environmental impact of the construction phase.

The overall environmental objectives that will be applied to the project are:

- All practicable steps shall be taken to minimise the environmental effects of

construction works.

- All activities shall be conducted in accordance with the CEMP, relevant legislation,

Codes of Practices, Guidelines, and any local environmental procedures.

- Environmental licenses, permits and consents and other statutory requirements are to

be obtained prior to works commencing, and fully complied with.

- All staff (including sub-contractors) shall be aware of the environmental issues

relevant to the Project through the provision of site specific information on the

environmental impacts of construction and the mitigation measures to be applied

during inductions, briefings and tool box talks and other relevant .

- Regularly reviewing of the environmental requirements of the project and ensuring

that environmental controls remain adequate throughout the duration of the project.

3;   Roles and Responsibilities

This section describes the environmental roles and responsibilities of key members of the

project team and provides contact details for the relevant personnel. The Principal Contractor

shall be responsible for all actions

Site Manager

To lead by example on  all areas of environmental management.

Ensure that appropriate resources are in place to effectively implement the CEMP and

deliver all legal requirements.

Review the CEMP throughout the construction process to ensure it remains relevant

and effective in identifying and managing environmental risks.

Report to and agree in writing with the LPA any amendments to the CEMP.

Ensure that all legal requirements are identified and met.

Implement the use of an accurate Site Waste Management Plan (SWMP) and ensure

its applicability to the site operations.

Ensure that the site is safe and that hazards are identified and secured.

Monitor performance of the project against statutory requirements, objectives and

targets.

Ensure that all legal requirements are identified and met.

Ensure the accurate reporting of resource usage e.g. energy and water.

Manage investigation and resolution of complaints

Ensure correct procedures are followed in case of an environmental incident.

Site Staff & Sub-Contractors

Compliance with direction given in the Site Induction

Proactively approach environmental issues whilst on site

Site staff should ensure they are fully aware of the environmental procedures in place

and if they have any questions they should be directed towards the Site Manager.

Ensure all construction activities are carried out in line with the procedures detailed in

the CEMP.

Report any environmental incident to the Site Manager

Contact Details

During the works contact can be made with the construction site

on the following number: TBA

. Phone: [TBA]

. Email: [TBA]

The induction given to all site personnel shall include a general overview of site specific

environmental issues, as well as details of how these issues shall be managed. All appropriate

site personnel shall undertake environmental awareness training and if supplementary training

is needed over the course of the works it will be provided as necessary.

Detailed information shall be communicated to personnel by means of regular Environmental

Briefings and Toolbox Talks covering topics relating to specific site activities.

4. Environmental Management

All identified risks are addressed within this document.

The construction of the facility is anticipated to take approximately 2 months

Demolition has been undertaken prior to this application and all materials removed

Construction Environmental Management Plan

The BRE guidance makes the following recommendations: “Before the start of any project,

appropriate training on how to control pollution emissions should be given to all personnel

expected to be present on site. This training should include;

The benefits of reducing pollution to health and environment,

The benefits of minimising disruption from complaints and enforcement actions,

Methods to minimise the generation of pollution,

Actions plans on what should be done if emissions breach any limits that have

been set for the particular site,

All site personnel, site operatives and sub-contractors will be made aware of the risks and

mitigation methodology for any potential environmental impacts relevant to their specific

activities through the Site Induction and regular briefings and toolbox talks.

Consultation with the public

The Pollution control Guide: Part 1 – Pre-project planning and effective management makes recommendations regarding the handling of public relations. These recommendations will be implemented at the site; Notice boards on the site perimeter fencing will display telephone and email contacts for enquiries and receipt of complaints, and the name of the persons who should be contacted

Dust

The site is located on Gloucester Crescent a Listed Building with sensitive neighbours, in residential surroundings.

The site has a range of residential properties which are noted as sensitive receptors.

It is also recognised that in periods of high winds the receptors affected by wind blown dust

could be areas further away than those mentioned above if suitable dust mitigation measures

are not implemented.

The generation of dust outside of the site boundary is a potential source of statutory nuisance

and can lead to complaints being received. As well as receiving complaints dust can also have

an impact on human health and local ecology. Properties within 100m of the main construction

area who may experience occasional increase in local soiling during activities being carried out

in extremely dry and windy weather. These are deemed to be short-term episodes with a likely

result in increased soiling of property surfaces, and are not normally associated with the general

risk to health. Whilst this statement outlines the potential impact from dust on residents the

mitigation outlined in this section will focus on removing the potential from nuisance dust emissions.

Potential dust sources during the construction phases of the development works

Site clearance

Cutting, grinding and drilling operations

Accidental spillage and loss of load from vehicles carrying loose materials

Location of sensitive receptors (including residential and commercial properties)

Potential impacts

There are no UK statutory or recommended levels of dust deposition that constitute a statutory

nuisance; whether or not a nuisance exists is determined in the first instance by the

professional judgement of the environmental health officer.

Dust contains particles in the range of sizes. The finer fraction of dust particle range is more likely to become airborne and to travel beyond the boundary of a site.

Particulate dust emissions from construction works associated with the development have the potential to impact upon local residents, and on-site workers during the works. Dust which is raised by onsite operations may be carried to nearby residential areas, if present in sufficient

quantities. Concerns have been raised by the inhalation of dust particles and the possible

health effects this may have. This has the potential to impact upon local residents and on-site

workers.

Good environmental practices will as described below will be used to control these dust

emissions, and mitigate against any nuisance problems arising.

Any complaints in relation to fugitive dust will be logged and appropriate action taken by site

Management.

Mitigation measures will be implemented to ensure effective control of dust emissions from the

construction works. These are industry best practices. The best practices are proven, well

established, techniques to mitigate nuisance dust emissions.

The mitigation measures proposed in this plan are based on guidance provided by the

following documents;

The BRE Guidance Document, controlling particles, vapour and noise pollution

from construction sites.

London Best Practice Guidance, The control of dust and emissions from

construction and demolition.

General mitigation techniques for the various construction activities will include:

All construction traffic shall park in designated permit area.

All vehicles carrying loose material shall be covered

Loose Materials

To be damped down enclosed or covered as appropriate.

Location of looseaway from any sensitive receptors wherever possible

Dust

Suppression

All cutting and drilling will be undertaken indoors with dust extraction via controlled collectors

Road edges and pathways shall be swept by hand and damped down as necessary

Monitoring and mitigation activity to be increased during significantly dry and windy

Where necessary, use of enclosures to be considered to ensure reduction in dust

migration

All cutting and grinding operations to be conducted in ways to reduce risk of dust

migration (wet cutting techniques etc.)

There will be a complete prohibition on the burning of any materials on the site during

construction.

Mitigation equipment will be readily available on site from the commencement of the works.

These will include sheeting, and damping equipment.

Where processes are intrinsically dusty and alternative processes exist which are less dusty,

these will be implemented. Prefabricated components and construction will be used wherever

possible to minimise the need for on-site construction which may cause the emission of dust.

The main construction process will involve the construction internal partitions.

There is minimal potential for the emission of dust in enclosed areas during the construction stage

but mitigation methods are necessary to reduce these.

Off site fabrication and construction will be used wherever possible. If this is not possible, the

fabrication processes will be undertaken away from sensitive receptor areas.

Dust suppression methods will be implemented by using fixed enclosures or equipment with

water sprays, local exhaust ventilation or particle extraction/minimisation systems.

Mixing of concrete will be undertaken in shielded areas away from sensitive receptors.

Dry sweeping will be avoided and damp sweeping using a fine mist will be used. Washing and

damping down will be implemented as required.

Materials handling (throughout the construction period)

A wide range of materials will be handled during the construction phases of the development

and the handling of these materials has the potential to create dust emissions. Generally the

use of dry or powdery material on site will be minimised. The following precautions will be

implemented to minimise dust emissions arising from materials handling;

Material drop heights will be minimised,

Damping down will be used to reduce dust emissions. in dry, hot weather damping

down frequency will be increased,

Lorries will be covered.

Materials delivered to site will be left wrapped until needed,

Methods and equipment will be in place in case of spillages. The site will be

regularly inspected for spillages and wet handling methods for cleaning up spillages

such as cement powder will be used.

Site, Access and Public roads will be monitored and cleaned throughout the construction period

The access road to the main construction site will be cleaned daily during the working week and

more frequently if necessary using a mechanical road sweeper. Edges of the access road and

footpaths will be cleaned with a hand broom and controlled damping.

Damping down techniques used to minimise the re-suspension of dust into the air can also

cause the build up of mud and dirt on roads which is picked up by vehicle wheels. Therefore

wheel washing techniques to prevent the transportation of mud and dirt off site.

Vehicles leaving the construction areas will be inspected at the wheelwash points and

thoroughly cleaned as required.

4.3 Noise

Like dust, the generation of noise beyond of the site boundary is a potential source of statutory

nuisance and can lead to complaints. Such complaints, if upheld, can lead to legal action

which causes resultant delays and costs to the project. The site is directly adjacent to various

residential areas making construction noise a particularly sensitive issue to local residents

A basic element in the mitigation of nuisance due to noise is adherence to acceptable working

hours. All works shall be undertaken in compliance with the working hours specified in

"Code of Practice: Control of Pollution & Noise from Demolition &

Construction Sites"

ie:

. Monday to Friday 8am - 6pm

. Saturday 8.30am - 1pm

. No Sunday, Bank holiday or Public holiday working

Should work be required outside of these hours prior permission must be sought from the local

authority including details of any noise that may result from the activities.

Good relations with people living and working in the vicinity of site operations are of

paramount importance. Early establishment and maintenance of these relations throughout

the carrying out of site operations will contribute towards allaying people’s fears. Good

relations can be developed by keeping people informed of progress and by treating complaints

fairly and expeditiously.

Noise can also interfere with working efficiency of site workers by inducing stress, by

disturbing concentration and by increasing accident risk. Effects of noise on persons on site

are similar to the effects on nearby residents, and the benefits of good control measures will

apply equally on and off site

Where any noise complaints are received, these will be thoroughly investigated by the site

management .

Site Investigation and Assessment

Noise Control Measures

A number of control measures will be implemented at the site to minimise noise. These

measures are developed from current best practice and the BRE Pollution control Guide: Part

1 – Pre-project planning and effective management. In particular the following control

measures will be applied;

All construction plant and equipment shall comply with EU noise emission limits.

All vehicles and mechanical plant used for the purpose of the works shall be fitted

with effective exhaust silencers.

Selection of inherently quiet plant.

All major compressors, generators etc. shall be ‘sound reduced’ models fitted with

properly lined and sealed acoustic covers which shall be kept closed whenever the

machines are in use and all ancillary pneumatic percussive tools shall be fitted with

silencers of the type recommended by the manufacturers.

Machines in intermittent use shall be shut down in the intervening periods between

works or throttled down to a minimum.

Materials shall be delivered during normal site working hours.

All ancillary plant such as generators, compressors and pumps shall be positioned

so as to cause minimum noise disturbance, i.e. furthest from receptors or if

necessary, behind acoustic enclosures and /or shielding. Where possible, loading

and unloading will also be carried out away from such areas,

Modification of existing plant and equipment. Noise from existing plant and

equipment can often be modified at source if necessary or improved sound

reduction methods can be applied,

The hours of operation of all plant and vehicles will be limited to the normal site

working hours and any use of equipment outside of these hours will be avoided.

No plant or machinery will be left running unnecessarily.

Materials will be handled as carefully as possible when loading lorries and skips to

minimise noise,

Whilst reversing alarms do present audible impact their necessity is a reflection of the high risk

associated with reversing vehicles. They must be distinct to ensure they are audible above

background noise. Despite their need, mitigation can be introduced to prevent nuisance to

residents. The site layout will introduce and maintain one-way vehicle movements with

unloading areas and lay-by’s to prevent the need for vehicles to reverse. Any reversing areas

needed or introduced will be located away from sensitive receptors. Pre-established reversing

areas will also allow additional acoustic barriers to be introduced to provide further mitigation.

All practicable additional measures will be taken to minimise nuisance from reversing alarms

4.4 Vibration

The works should not pose any significant risks in relation to vibration.

Best practice techniques shall be utilised at all times to minimise vibration from construction

activities.

Any complaints relating to vibration disturbance will be fully investigated by the site

management team.

4.5 Light Disturbance

The major mitigating factor is the project working hours. (0800-1800, Monday to Friday), this

will prevent the more severe element of intrusion, i.e. sleep deprivation, being realised.

4.6 Waste Management

The site will comply with the Site Waste Management Plans Regulations 2008.

4.7 Water

Internal work only, riask from leakages to be monitored as work progresses

4.8 Transportation and Traffic Management

The site has no internal road structure and off road management will be to Highways Authorities

standard guidance

4.9 Ecological Management

There is no ecological value to the site , internal works only

Dust minimisation methods shall be employed

4.13 Energy Consumption/Monitoring

“Switch it Off” schemes and other energy saving campaigns shall be implemented on site to

encourage all personnel to consider their carbon footprint. Use of car sharing and buses shall

be encouraged as outlined in the CTMP. Posters shall be clearly displayed within the site

offices to ensure all personnel are aware of the CTMP requirements. This shall also be

covered within the site induction and regular toolbox talks held relating to the subject.

4.14 Water usage

Within site accommodation taps shall be switched off when not in use and all staff shall be

made aware of water saving techniques. Every effort to ensure reduction in water use shall be

implemented where available.

4.15 Visual Amenity

The following mitigation measures shall be deployed to

minimise the impacts;

Front yard shall be cleaned at the end of every work shift. No materials are to be stored externally

5. Pollution Control Measures

Weekly combined Health, Safety and Environmental Inspection will be carried out to ensure

procedures are followed.

Site Transportation

Parking Permits to provided established for off-site parking.

No out of working hours deliveries will be accepted.

6. The site Gangers and Foremen will be trained in pollution incident mitigation and clean-up.

7.  Contaminated Land

The site investigation has shown there to be no contamination

8. Emergency Contact Numbers

Contact

Name TBA

Daytime Contact TBA

Email Contact TBA

Out of Hours

Contact

TBA

Site Office TBC

Emergency Services 999 / 112

Local Council

Cambden.