DUST

| Project: 259-263 Kentish Town Rd | Contractor: Demo One & DDC Ltd |  | Client: GMS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Document ref no: | Assessed by: Matt Hamshere |  | Date: 25.4.16 |  |  |  |
| Task/operation and location (including any tools/equipment in use) Demolition and general site construction including cutting, grinding, sanding etc |  |  |  |  |  |  |
| Hazards |  |  |  | # Risk ratings |  |  |
|  |  |  |  | Without controls |  | With controls |
| - Flying particles of brick / stone / silica dust . Other: |  |  |  | Medium |  | Low |
| Harm - Eye injuries from flying particles. - Respiratory problems. - Dermatitis – hands and face. Other: |  |  |  |  |  |  |
| Persons in danger - Workers involved in demolition. - Construction workers throughout the project. - Third parties – if dust allowed to spread past site confines Other: |  | Number of persons in danger |  |  |  |  |
| Controls - Training in dust suppression methods should be given to all operatives. - Site boundary to be shrouded by independent scaffolding and enclosed with monoflex or debris netting - Relevant personal protective equipment should be issued and worn. - Only operatives trained in the mounting of cutting discs under the Abrasive Wheel Regulations should undertake work using such equipment. - Where reasonably practicable, any abrasive wheels used should be provided with a guard that will contain every part of the wheel following a fracture. - Method statements should be produced detailing work activities and Personal Protective Equipment (PPE) requirements. - A COSHH assessment should be produced if appropriate. - Brick / Stone should be cut wet if possible, otherwise extraction by sanding machines should be used to remove the dust at source. - Workers must be given relevant information from any COSHH assessment. - Personal hygiene practice should be recommended to all workers as a means of avoiding dermatitis – barrier creams can also act as a preventative. - Suitable washing facilities should be provided. Other: |  |  |  |  |  |  |
| Personal protective equipment - Hearing protection. - Gloves. - Safety shoes/boots. - Goggles. - Appropriate dust masks to filter out harmful respiratory constituents. Other: |  |  |  |  |  |  |
| Additional assessments - COSHH. - Manual handling - Noise - Personal Protective Equipment (PPE). - Use of abrasive wheels. Other: |  |  |  |  |  |  |
| Method statement required? Yes |  |  |  |  |  |  |
| Is the task adequately controlled? Yes |  |  |  |  |  |  |
| Specific legislation - Manual Handling Regulations 1992 (SI 1992 No. 2793). - Abrasive Wheels Regulations 1970 (SI 1970 No. 353) (as amended). - Provision and Use of Work Equipment Regulations 1998 (SI 1998 No. 2306) (PUWER). - Control of Pollution Act 1974 - Environmental protection Act 1990 Other: |  |  |  |  |  |  |
| HSE and other guidance |  |  |  |  |  |  |
| Information, instruction and training - All workers should be briefed on the possible hazards and control measures to be implemented. - Instruction in correct use of PPE should be provided. - Toolbox talks should be given to maintain a safe system of work. Other: |  |  |  |  |  |  |
| Emergency procedures - Suitable first-aid facilities as required generally for the site must be available. - The first-aider on site should be alerted and he or she should be familiar with the treatment required for this work. Other: |  |  |  |  |  |  |
| Monitoring procedures - Supervisors should monitor the wearing of appropriate PPE as detailed in the method statement and site rules. - Supervisors should monitor the all activities as detailed in the written method statements. Other: |  |  |  |  |  |  |
| Other items |  |  |  |  |  |  |
| Signature |  |  |  |  | Date for review |  |