| # LDC (Proposed) Report |  | # Application number | # 2015/7260/P |
| --- | --- | --- | --- |
| ## Officer |  | ## Expiry date |  |
| ## Anna Roe |  | ## 21/03/2016 |  |
| ## Application Address |  | ## Authorised Officer Signature |  |
| 34 Ingham Road, London, NW6 1DE |  |  |  |
| ## Conservation Area |  | ## Article 4 |  |
| ## N/A |  | ## N/A |  |
| ## Proposal |  |  |  |
| Erection of rear dormer roof extension and "pod" roof extension above part two storey rear addition. |  |  |  |
| ## Recommendation: | ## Approve Certificate |  |  |

| The proposal includes both the enlargement of the existing rear dormer extension on the main roof slope and a “pod” roof extension to the two storey outrigger at the rear of the property. The outrigger extension would join the main building at the eaves of the existing roof, creating L-shaped addition. The agent asserts that the proposed works are permitted by virtue of Class B of Part 1 of Schedule 2 to the GPDO. This permits the enlargement of a dwelling house consisting of an addition or alteration to its roof. Taking into account the Technical Guidance (April 2014, DCLG), the legislation (2015 GPDO) and recent appeal decisions, the Council considers that Class B of Part 1, Schedule 2 of the GPDO is engaged. |  |  |
| --- | --- | --- |
| Class B The enlargement of a dwellinghouse consisting of an addition or alteration to its roof |  |  |
| If yes to any of the questions below the proposal is not permitted development |  | Yes/no |
| B.1(a) | Permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use). | No |
| B.1(b) | As a result of the works, would any part of the dwellinghouse exceed the height of the highest part of the existing roof? | No |
| B.1(c) | As a result of the works, would any part of the dwellinghouse extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway? | No |
| B.1(d) | As a result of the works, would the cubic content of the resulting roof space exceed the cubic content of the original roof space by more than— (i) 40 cubic metres in the case of a terrace house, or (ii) 50 cubic metres in any other case? | No |
| B.1(e) | Would it consist of or include— (i) the construction or provision of a veranda, balcony or raised platform, or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe? | No |
| B.1(f) | Is the dwellinghouse on article 1(5) land? | No |
| If no to any of the questions below the proposal is not permitted development. |  |  |
| B.2(a) | Would the materials used in any exterior work be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse? | Yes |
| B.2(b) | Other than in the case of a hip-to-gable enlargement, or an enlargement which joins the original roof to the roof of a rear or side extension would the edge of the enlargement closest to the eaves of the original roof be less than 20 centimetres from the eaves of the original roof, so far as practicable? | Yes |
| B.2(c) | Would any windows inserted on a wall or roof slope forming a side elevation be obscured-glazed and non-opening unless the opening part is higher than 1.7 metres above the floor of the room in which the window is installed? | N/a |
| With regard to Condition B.2.(b) the proposal joins the original roof to the roof of a rear extension. There is an existing rear dormer which itself resulted from an earlier extension to the property however the original roof remains at eaves level, satisfying the requirements of part B.2.(b). Summary The proposal would comply with Class B of Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015. |  |  |