# SIMON LEVY ASSOCIATES

## Chartered Building Surveyors

Link House, 49 Theobald Street,

Borehamwood Hertfordshire, WD6 4RT

Telephone: 0208 207 6100    Fax: 0208 207 6313

Email: sl@simonlevy.net

Principal: Simon Levy FRICS MAE

Regulated by the RICS

DESIGN & ACCESS STATEMENT

IN RESPECT OF:

Ground Floor Flat

4 Chamberlain Street

London

NW1 8BX

PREPARED FOR:

MR J Bucknell

PREPARED BY:

# SIMON LEVY ASSOCIATES

LINK HOUSE, 49 THEOBALD STREET

BOREHAMWOOD, HERTS. WD6 4RT

TEL: 0208 207 6100   FAX: 0208 207 6313

DATE:

24/08/2015

- PROPOSAL

  - Addition of a secondary glazing system set in an aluminium or plastic frame shall be installed and fixed to the interior joinery surfaces without cutting away of the internal joinery. All components are to replicate the original sectional profiles and are to use matching materials.

  - The rear addition glazed door is single glazed and may permit excessive heat loss. In order to mitigate this and accommodate Environmental Health’s requirements defined in the Housing Act 2004 schedule, the door will be replaced with a suitable painted timber component fitted with hermetically sealed double glazing that is like for like with previous door. Work to be carried out by appropriate craftsman.

  - Strip out the lighting and power circuits complete back to the incoming mains supply, including all wiring, lamp holders, switches, sockets and spurs and switch gear. Provide a new domestic electrical installation to the entire house. Works to include a complete replacement of the electrical installation, light fittings, power sockets and any wiring to the gas boiler or an electric immersion heater

  - To strip out rear bathroom and supply new bathroom interior

  - Overhaul, repair or renew as necessary the gas pipework, fixtures and fittings for the gas supply to the premises and install a whole house gas fired central heating system to the flat to satisfy the design and installation requirements of British Standard 5449:1990 and in accordance with Part L of the current Building Regulations.

  - Overhaul double hung sliding sash windows in both the Living Room and Kitchen.

  - Strip existing roof covering and flashings from the back addition roof above the bathroom and remove from site. Examine roof and decking exposed for signs of rot or deterioration and remove defective decking, replace with new materials as required.

  - New Kitchen to be re-constructed in ground floor rear. Installation of new kitchen fixings to be connected to existing hook-ups.

  - To provide 6 new sockets in the kitchen area and 5 new sockets in the front room in order to make it compliant with BS7671.

  - Construct and secure a new railing to the rear garden between the garden level and the basement light well. The guarding will be at least 1100mm high.

  - Provide and fix a handrail to the outer string of the external staircase between the main entrance steps at the front on the property and the basement light well.

  - Install new door in the rear of the basement flat to match the rear door of the ground floor flat

  - To restore Lime Plastered walls like-for-like by an appropriate craftsman

  - To restore Cornices like-for-like by an appropriate craftsman

  - Non-historic glazed timber door to First Floor Flat will have glazing replaced with fire resitant glazing and finished with intumescent glazing. Cold smoke seals also to be installed

  - Apply intumescent glazing and cold smoke seals to door leading to bathroom area

  - Compensatory fire detection system to be installed in the form of:

Mixed system: Grade A: LD2 coverage in the common areas and a heat detector in each flat in the room/lobby opening on to the escape route (interlinked) - Grade D:LD3 coverage in each flat (non-interlinked smoke alarm in the room/lobby opening onto the escape route) to protect the sleeping occupants of the flat.

  - Sheeps wool insulation to be installed between joists of basement ceiling

  - To carry out plasterwork to basement ceiling using Lime Plaster by an appropriate craftsman.

  - Envirograf fire resistant coating to be applied over lath-and-plaster ceilings

  - Install Xcluder rodent mesh protection around service pipes, ducts, skirtings and any gaps around openings. This allows Lime Plaster to be applied over.

2.00	SITE LOCATION

2.01	The existing house is a terraced property that has been converted into flats on Chamberlain Street. The street is accessible from off of Regents Park Road. The property is the ground floor flat.

3.00	PURPOSE

3.01	To comply with the requirements of Camden Council: Environmental Health.