Ms Catherine Bond	Direct Dial: 020 7973 3785

London Borough of Camden

Development Control Planning Services	Our ref: L00517795

Camden Town Hall

Argyle Street

LONDON

WC1H 8ND	21 July 2016

Dear Ms Bond

Arrangements for Handling Heritage Applications Direction 2015 &

T&CP (Development Management Procedure) (England) Order 2015

LAUDERDALE HOUSE , HIGHGATE HILL , LONDON, N6 5HG

Application No 2016/2482/L

Thank you for your letter of 7 July 2016 notifying Historic England of the application for listed building consent relating to the above site.

I have now had the opportunity to visit the site and see the timbers in question, having met the foreman and architect whilst there.

Historic England Advice

The recent works to the building have uncovered three late seventeenth-century timber posts. Despite being attached at top and bottom to more modern beams, it is entirely possible that these posts are in their original positions. The posts are all of hand-sawn oak, and two of the three have an ovolo moulding along one side. These moulded sides correspond to form either edge of a doorway. The lintel of the this doorway is no longer extant, apart from a single tenon left in a mortice, and a corresponding vacant mortice on the other side.

The central post of the three was proposed to be moved in order to avoid any damage when this part of the building becomes used as circulation space. In my opinion relocating this post would make the legibility of the original doorway somewhat obscure and could pose a difficult to future attempts to understand the building's history.

While the post has been damaged through fire in the past and is badly charred in some areas, I do not think passing knocks will present any serious threat to its condition. The most fire-eaten part of the post centres around a cavity where a mortice once existed. The biggest threat to condition is likely to be visitors fiddling with this rather more fragile bit of fabric. I suggest that this is a management issue, and that if it is found to be a problem signage or some other solution could be devised at a later date.

In the mean time the best response to these very interesting timbers is, I suggest, a light touch approach which keeps them in situ and properly intelligible. Moving the post in question may in fact be riskier than exposing it to visitor footfall.

I now understand that the proposals have been amended to retain the post in its original position. While I have not had sight of the amended drawings, Historic England would encourage this approach and recommend its approval.

Recommendation

Historic England recommends approval of the retention in situ of the recently discovered posts, and that the proposals are determined in accordance with national and local legislation and policy, and on the basis of your own specialist conservation advice.

Please note that this response related to historic building matters only. If there are any archaeological implications to the proposals it is recommended that you contact the Greater London Archaeological Advisory Service for further advice (Tel: 020 7973 3735).

Yours sincerely

Matthew Cooper

Inspector of Historic Buildings and Areas

E-mail: matthew.cooper@HistoricEngland.org.uk