- External Glass Fin System

- Drawing references:… ………………..

Manufacturer and reference: Levolux 9000 Vertical Glass Fin system supplied and installed by Levolux Ltd. Tel: 020 8863 9111, E: info@levolux.com .

Description: Fixed external laminated glass louvre system. The glass comprises of one sheet of 8mm clear heat strengthened glass with a single ply coloured interlayer and a 8mm heat strengthened glass sheet. Glass to have polished edges all round. The glass fins are to be clamped each end using bespoke clamps. Clamps are bolted to bespoke steel brackets which are fixed to a suitable structure.

Finish:  All steel and aluminium components to be polyester powder coated to standard solid RAL colour.

Dimensions: As shown on drawings.

Fixing: Hexagonal head Stainless Steel fixings.