NBS SECTION Q37 - DESCRIPTION OF WORKS

Section Q37 deals with the design and installation of the Bauder Green Roof landscaping system, including the various related elements i.e. separation, protection, and drainage layers, substrates, Bauder supplied planting and accessories such as inspection chambers, trims etc.  It invokes clauses from related sections for waterproofing, insulation, landscaping and maintenance as necessary for a complete system.

It is intended for use on projects where the detailed design is completed by the specifier (architect or landscape architect) with technical assistance from the manufacturer as required and should be read in conjunction with any project specific drawings provided.

SCOPE OF WORKS

This section includes:

- Bauder Extensive green roof system components/ landscaping.

- Related Bauder system accessories

This section does not include:

- Construction of the structural deck

- Bauder waterproofing system – refer NBS Section J41-110A & B.

- Irrigation– refer NBS section S__.

### Q37	GREEN ROOFS

To be read with Preliminaries / General Conditions.

GENERAL

130A EXTENSIVE GREEN ROOF: Barrell Roof

  - Landscaping finish:  Pre-cultivated Sedum vegetation blanket

  - Substrate: New Plywood Deck

  - Slope: 10°

  - Waterproofing system: BAUDER TOTAL GREEN ROOF SYSTEM - as per J41-110A

  - Drainage / protection layer: N/A

  - Vegetation blanket: Bauder Xero Flor XF301 sedum blanket (FAA fire rated), applied in standard length rolls 2m x 1 m.  Installation as clause 800C.

  - Landscaping depth: ca. 28mm (excluding vegetation).

  - Vegetation: Mainly Sedum with some moss and grasses.

  - Accessories: -

-	Project specific drip line irrigation system (designed and supplied by others), as clause 463B. Installation of irrigation pipe work to the waterproofing as clause 825.

-	Bauder AL40 Sedum blanket edge trim, fitted around all perimeters and protrusions.  Installation as clause 820A.

-	       Bauder sedum blanket retention strips.  Installation as clause 821.

- 	Bauder Xero Flor organic fertiliser, apply as clause 850A.

  - Additional requirements: As clauses 210, 710, 715A, 720, 910, 915A, 916, 920, 930.

130B EXTENSIVE GREEN ROOF: Existing Flat Roof

  - Landscaping finish:  Pre-cultivated Sedum vegetation blanket

  - Substrate: Existing Concrete Deck

  - Slope: 1°

  - Waterproofing system: BAUDER TOTAL GREEN ROOF SYSTEM - as per J41-110B

  - Drainage / protection layer: Bauder SDF Mat - 20mm drainage / protection layer.  Installation as clause 770B.

  - Vegetation blanket: Bauder Xero Flor XF301 sedum blanket (FAA fire rated), applied in standard length rolls 2m x 1 m.  Installation as clause 800B.

  - Landscaping depth: ca. 48mm (excluding vegetation).

  - Vegetation: Mainly Sedum with some moss and grasses.

  - Accessories: -

-	Bauder AL40 Sedum blanket edge trim, fitted around all perimeters and protrusions.  Installation as clause 820A.

- 	Bauder Xero Flor organic fertiliser, apply as clause 850A.

  - Additional requirements: As clauses 210, 710, 715A, 720, 910, 915A, 916, 920, 930.

PERFORMANCE

210	GENERAL DESIGN

  - Green roof and associated features: Complete the detailed design.

  - Proposals: Submit drawings, technical information, calculations and manufacturers literature.

PRODUCTS

- 463B	IRRIGATION

- Supplier: Access Irrigation Ltd.  Crick, Northampton NN6 7XS

Tel: 01788 823811, Fax: 01788 824256, E-mail: sales@access-irrigation.co.uk

- Product reference: Permadrip Pro - drip line irrigation system with anti-syphon design to resist clogging and pressure regulation.

- Material: Perforated UV resistant plastic  - Colour: Dark Brown

- Height: 10mm

- Operating range: 0.8 -4.3bar.

- Wall thickness: 1.2mm

- Nozzle output: 1.6L/h

- Spacing: 150mm.

- Pipe connection:  For Bauder Extensive Green Roofs, all irrigation systems using Permadrip Pro must use PoziLock compression fittings for both the dripline and water feed pipework.  Barbed connectors are not permitted.

- Location: To be installed above the XF301 sedum blanket.

- Design: The design, water pressure, positioning and spacing of the drip line is critical to the performance and effectiveness of the system.  Therefore, the irrigation scheme for each project should be designed accordingly to ensure it is fit for purpose. Irrigation is a specialist subjects and Access Irrigation Ltd provide a design service and can assist with information pertaining to any individual project upon request.

EXECUTION

710	INSTALLATION GENERALLY

  - Preparation: Clear all surfaces of debris.

  - Timing: After certification of waterproof membrane integrity.

  - Surface condition: Visually inspect waterproof membrane, report any damage.

  - Faults in waterproof membrane: Report.

  - Contamination: Do not use materials detrimental to healthy plant growth.

  - Storage: Do not overload.

  - Point loads: Avoid.

  - Outlets: Do not block.

  - Outlet grilles: Installed (these can be omitted where Bauder inspection chambers are used, if the grille cap height obstructs the closing of the chamber lid).

### 715A    GREEN ROOF RELATED REQUIREMENTS

- The following are vital to the accurate pricing, correct installation, and ultimately the long-term life of a green roof, and must, therefore, be included within the specification and tender documents: -

- Loadings: It is assumed that the architect or his advisors have satisfied themselves that the roof structure and deck are suitable to receive the dead load of the proposed green roof system and landscape both during construction and on completion of the works.

- Additional protection: A planned or contractual delay between the installation of the waterproofing and landscape will almost certainly necessitate additional/increased protection to the waterproofing.  This protection may be temporary or permanent.  The responsibility and cost of this possible extra protection should be clearly included within the tender documents.

- Detailed drawings: Correct detailing design and construction is essential to the long-term life of the green roof. It is essential, therefore, that detail drawings illustrating for the construction are included with the tender documents, in order to enable the contractor to tender accurately.

- Minimum upstand height requirements: The waterproofing should be taken up all abutment upstands, pipes, detailing protrusions etc. a minimum of 150mm above finished landscape surface level to comply with British Standards and current code of practice BS8217:2005.

- Provision for living products in hot weather conditions: During hot weather conditions, living products such as plants, turf, sedum blankets etc. must be laid on the day of delivery to site.   With regard to sedum blankets or turf, any rolls not installed should be laid out and kept watered prior to final installation.

- Watering / Irrigation: Adequate provision for watering the installed any form of planting must be in place on site before the product is installed.  Irrigation systems if fitted should be operational.  Initial watering should be by surface sprinklers to water in the fertiliser, where this is specified.  All watering should be carried out in strict accordance with the Bauder watering requirements and guidance document.

- Final Inspection: No landscaping work should be installed until Bauder have carried out a final inspection to the waterproofing and have passed this as suitable for guarantee. It is the responsibility of the roofing contractor to advise and organise this inspection with Bauder.  We cannot guarantee any waterproofing that has been landscaped without this inspection having been carried out and passed as acceptable.

- Damage risk form other trades: No landscaping should be installed while the roof area is subject to other site traffic.  Bauder will carry out an inspection of the completed roof 4-6 weeks following installation and any site related damage by others will be reported to the client.   Bauder accept no responsibility whatsoever for damage to the product or the installation caused by site work carried out by others after the landscaping has been installed.

- First year maintenance:  The contractor must price into his tender the cost of post installation maintenance for a minimum period of 1 year to ensure the handover of a flourishing green roof.

720	ADVERSE WEATHER

- Unfinished work: Secure from damage and wind uplift.

- Conditions: Do not install or work with frozen materials.

LANDSCAPING INSTALLATION

##### 770B	DRAINAGE LAYER INSTALLATION

- Extent: Continuous over designated roof area

- Fitting: Loose laid over the waterproofing and butt jointed with 100mm fleece overlap

- Upstands: Carefully cut to fit closely around penetrations and outlets.

800B	VEGETATION BLANKET INSTALLATION

  - Handling blankets:

  - Timing: Lay within 36 hours of lifting from growing position.

  - Method: laid manually – two-man operation

  - Excessive stacking: Not permitted.

  - Material loss (maximum): 3% of total surface area.

  - Growing medium condition: Thoroughly watered

  - Laying blankets:

Dry, damaged, frosty or waterlogged blankets: Do not lay.

    - Orientation: Diagonal or perpendicular to slope of roof.

    - Joints: Stagger. Butt together or slightly overlap to prevent gaps. Do not stretch blankets. All excess vegetation should be removed from the overlap and the opposite leading edge of the blanket to ensure that the joints butt together tightly (as per the Bauder installation guideline).

    - Edges: Finish with whole blankets.

    - Consolidation: N/A

    - Dressing: Bauder Xero Flor substrate.

    - Application: Brush in to fill joints.

- Watering: Thoroughly water using surface sprinklers immediately after installation and substrate dressing, ensuring that the blankets and substrate are fully saturated before moving on to the next area.

    - Roll size: 2m x 1m

800C	VEGETATION BLANKET INSTALLATION

Handling blankets:

  - Timing: Lay within 36 hours of lifting from growing position.

  - Method: laid manually – two-man operation

  - Excessive stacking: Not permitted.

  - Material loss (maximum): 3% of total surface area.

  - Growing medium condition: Thoroughly watered

  - Laying blankets:

Dry, damaged, frosty or waterlogged blankets: Do not lay.  Blankets to laid over the spikes of the already installed Bauder retention strips and trod down to ensure that the spikes penetrate the moisture retention fleece to the underside of the blanket.

- Orientation: Diagonal or perpendicular to slope of roof.

- Joints: Stagger. Butt together or slightly overlap to prevent gaps. Do not stretch blankets.  All excess vegetation should be removed from the overlap and the opposite leading edge of the blanket to ensure that the joints butt together tightly (as per the Bauder installation guideline).

- Edges: Finish with whole blankets.

- Consolidation: N/A

- Dressing: Bauder Xero Flor substrate.

- Application: Brush in to fill joints.

- Watering: Thoroughly water using surface sprinklers immediately after installation and substrate dressing, ensuring that the blankets and substrate are fully saturated before moving on to the next area.

- Roll size: 2m x 1m.

820A	EDGE RETAINING PROFILE INSTALLATION

- Cutting: Neat, accurate and without spalling.

- Junctions: Use connector pieces and pre-formed 90º corners where required.

- Position: True to line and level. Smooth continuous lines.

- Fixing: The AL40 Sedum blanket edge trim is to be secured in place by separate pieces of torch applied Bauder Plant-E capping sheet cut into strips 500mm x 200mm, these bituminous flashings should be torched through the holes in the trim to the waterproofing surface and set at intervals of 400mm between each flashing piece, bonding onto the main capping sheet by a minimum 100mm.

- Location: To secure and contain the exposed edges of XF301 sedum blanket at abutments and open perimeters.

- Precautionary note: When cutting metal, please ensure that appropriate tools and personal protection equipment are used.

# 821	BAUDER SEDUM BLANKET RETENTION STRIP

- Bauder sedum blanket retention strips, 800mm long are to be laid on to the capping sheet and held in place by means of separate flashing of capping sheet, minimum 800mm x 200mm, lapped onto the main capping sheet by a minimum 100mm.

- These are to be installed at 2m centres down the roof slope, staggered with alternate rows at 1M centres.  They should be positioned so that they grip the vegetation blanket 200mm down from the top leading edge.  In case of any queries please contact Bauder Ltd so advice may be given.

- Note:-  Care must be taken when handling this item due to its extremely sharp nature.

- Only remove the protective plastic guard immediately before installing each run of blanket.  Please read separate safety data sheet.

- Precautionary note: when cutting metal, please ensure that appropriate tools and personal protection equipment are used.

825	INSTALLATION OF IRRIGATION PIPEWORK

- Pipe work should be installed and connected in accordance with the irrigation supplier’s installation guidelines and set as per the plan provided.  Pipework should be secured at intervals to the XF301 using wire or cable ties, as required. Over time the sedum planting will cover the pipework, visually hiding it.

850A  FERTILISER

- Bauder Organic Fertiliser slow release must be applied at a rate of 80g/m² onto the installed planting.

- This product is to be supplied by Bauder Ltd.

- Care must be taken to distribute the fertiliser evenly, through use of an approved applicator.

- The vegetation / vegetation blankets should then be thoroughly saturated by the use of sprinklers so as to promote rapid establishment. It is the responsibility of the roofing contractor to liaise with the main contractor/ building owner to provide water to ensure that the growing medium/ blanket does not dry out within the first month – refer document ‘Watering Requirement Guidelines for Extensive and Bio-diverse green roof installations’.

COMPLETION

- 910	INSPECTION

- Timing: Before handover.

- Give notice (minimum): 3 days.

915A	ESTABLISHMENT WATERING REQUIREMENTS –

- XF301 requires surface watering for the first 4 weeks following installation (to ensure that the sedum blanket remains moist to the touch) It is the responsibility of the roofing contractor to liaise with the main contractor/ building owner to provide water to ensure that the vegetation mat does not dry out during this period.

- An adequate mains supply of sufficient pressure must be available and operational prior to the sedum blanket being delivered and installed. Irrigation systems (where fitted on roof slopes above 10 degrees) must be operational, but initial watering in of the fertiliser must be by surface mounted sprinklers.

- See the Bauder Watering Guide document for detailed information on watering requirements.

- Bauder Ltd accepts no responsibilities whatsoever for the condition of installed sedum blankets that are not properly watered in accordance with our recommendations.

916	POST INSTALLATION MAINTENANCE

- The installing contractor should price into the tender, the cost of carrying out post-installation maintenance for a contract period to be agreed with the client’s representative. Following completion of the landscaping installation and handover, the responsibility for future on-going maintenance of the green roof planting becomes the responsibility of the building owner or the Main Contractor, where this element forms part of the contract.

- Maintenance services: Bauder Ltd offers a professional maintenance service using experienced green roof technicians and would be pleased to provide an estimate for carrying out on-going maintenance.   Please contact our green roof maintenance team on Tel: 01473 257671.  Alternatively, the work can be contracted to experienced landscape contractors of your choice.

- Period of maintenance contract:  Insert requirement i.e. one year, two years etc.

- Scope of maintenance procedure: as per manufacturer’s recommendations.

920	COMPLETION

- General: Leave the works in a clean, tidy condition.

- Surfaces: Clean immediately before handover.

- Outlets: Clean and clear of obstructions.

- Completed green roof: Protect from adjacent or high level working.

930	DOCUMENTATION

- Timing: Submit at handover.

- Contents:

- Manufacturer’s guarantees and warranties.

- Procedures for maintenance of the green roof.

- Record drawings showing the location of planting and associated features.

- Number of copies: as required by client.

Bauder reserves the right to amend information and product specifications without prior notice. All reasonable care has been taken to ensure that the information is current and correct at the time of issue.  Please note that any future regulation changes could result in this specification requiring an update.  The specifier is responsible for ensuring that this specification information is still current prior to issue, as Bauder Ltd can accept no liability for any resulting errors or omissions.