KASIA WHITFIELD DESIGN

# DESIGN AND ACCESS STATEMENT

FLAT D, 26 DALEHAM GARDENS, LONDON NW3 5DA

PROPOSED DEVELOPMENT OF THE ROOF SPACE, ENLARGEMENT OF FLAT D

- Introduction

The Design and Access Statement (DAS) accompanies an application for a small enlargement of the roof space for the top floor flat.

This statement has been prepared in accordance with CABE guidelines on the preparation of Design and Access Statements, 2006.

As this proposal is for minor alterations to an existing house, some aspects such as the social and economic context are of limited applicability.

2. Design Principles and Concepts

The site is located in the Fitzjohn’s Netherhall Estate Conservation Area, on the eastern side of Daleham Gardens. It is a substantial detached dwelling house with 4 levels of accommodation. The construction of the house is of traditional materials, mainly in red brick, with timber windows and two, parallel pitched roofs (one of which has a hip-end with a dormer, facing the street) plus a smaller flat roofed infill joining the two.

The building is subdivided into four residential flats, one on each floor.

Top floor flat – Flat D, is entirely within the roof space.

The rear garden is shared use.

The site has an area of approximately 657m2.

The proposal seeks planning permission for a small enlargement of the top floor flat.

The proposal will provide additional living accommodation for an enjoyment of the residents of this flat.

The proposal will have no effect on neighbouring properties.

3. Layout

The proposal involves relocation of the existing kitchen, enlargement of the existing bathroom and addition of another bathroom as well as converting the old kitchen into a bedroom. The location of the kitchen will be moved to the proposed enlarged part of the infill flat roof. The proposal will create approximately 13sq.m. additional nett floor space.

The two large pitched roofs will remain unaltered and only the flat roofed area linking the two pitched roofs will be enlarged.

As the flat-roofed area is set back from the front and rear elevations, the development will be hardly visible from the street level.

4. Form & Scale

Rear and front elevation as well as section drawings are included with the application.

The proposed extension will not be visible to any of the neighbours and will not take away any light from the neighbouring properties.

The side elevations will not be affected by the proposed changes.

5. Access

The access to the property will remain unaltered.