PH/5962 FLAT 1 RUSSELL SQUARE MANSIONS, 122 SOUTHAMPTON ROW

IRONMONGERY SCHEDULE

| DOORS | HANDLE | LOCK | HINGE | ACCESSORIES |
| --- | --- | --- | --- | --- |
| D01 Front door | Oliver Knight Alan CDK Centre door knob, in Polished chrome – Q.ty 1 | Banham M5000 Cylinder Mortice Deadlock to BS3621 – Q.ty 2no for top and bottom installation, to be keyed alike + Banham BS2510 Latchbolt. All in Polished Chrome. | Banham Square hinge, 101 x 76mm, Ref. TRDFH207PNPSQ in Polished Chrome – Q.ty 3 + Banham H103 Hinge Bolt – Q.ty 2 | SDS London Stepped Edge Letterplate 282 x 89 mm in Polished chrome, Ref 03030 + Inner tidy flap |
| D02 Drawing room door | Oliver Knight Aglovale Mortice Knob, Ref. AGLOVALE1 in Brushed Brass – Q.ty 2 per leaf | SDS London Mortice Box Latch 76 mm Light Spring, Ref 78011, 57mm backset, Polished brass unlacquered | SDS London Strong Suite Brass Hinge DPBW 100 x 76mm, 32021, 11mm knuckle, Polished brass unlacquered – Q.ty 3 for each door | SDS London Hooded door stop 35021 in Antique Brass Unlacquered - Q.ty 2 |
| D03 Bedroom door D14 Study door | Oliver Knight Frederick Lever on Earl rose in brushed brass – Q.ty 1 pair for each door | SDS London Mortice Box Latch 76 mm Heavy Spring, Ref 78019, 57mm backset, Polished brass unlacquered | SDS London Strong Suite Brass Hinge DPBW 100 x 76mm, 32021, 11mm knuckle, Polished brass unlacquered - – Q.ty 3 for each door |  |
| D13 Lobby door | Oliver Knight Frederick Lever on Earl rose on entrance hall side, and on Reynfred plate + Ricaud turn and release on Garrett rose on lobby side, all in brushed brass – Q.ty 1 pair for each door | SDS London Bathroom Lock 76mm Follower 8mm Heavy Spring, Ref 78149, 57mm backset, Imitation Bronze Unlacquered | SDS London Strong Suite Brass Hinge DPBW 100 x 76mm, 32021, 11mm knuckle, Imitation Bronze Unlacquered - – Q.ty 3 for each door |  |
| D12 Pair of antique Art Deco doors to Closet | Oliver Knight Vasey pull handles (400mm) in Bronze – Q.ty 1 pair | SDS London Roller Catch 51mm, Ref 69029, Imitation Bronze Unlacquered – Q.ty 1 | SDS London Soss Hinge, in solid drawn brass, size to be confirmed once thickness of existing doors has been assessed – Q.ty 4 per leaf | SDS London Tyre Door Stop 40mm, Ref 91571 in Bronze Stainless - Q.ty 1 |
| Bedroom Wardrobe doors | Oliver Knight Vasey pull handles (550mm) in Bronze – Q.ty 2 pairs | SDS London Roller Catch 51mm, Ref 69029, Imitation Bronze Unlacquered – Q.ty 1 | Kitchen hinges with integrated soft closers to be Contractor’s choice – Q.ty 4 per leaf |  |
| Bedroom Wardrobe drawers | Oliver Knight Gallanhead drawer pull, 50mm, in Bronze – Q.ty 1 per drawer |  |  | Ball bearing, full extension drawer runners, with integrated soft closing mechanism to be contractors choice – Q.ty 1 pair per drawer |
| D17 Door to service cupboard |  |  | Kitchen hinges with integrated soft closers to be Contractor’s choice – Q.ty 4 per leaf |  |
| Dining room cupboard doors | Purpose made, laser cut quadrant handle in Bronze as per ADAM drawing | SDS London Roller Catch 51mm, Ref 69029, Imitation Bronze Unlacquered – Q.ty 1 | SDS London Soss 208 Hinge, 70 x 16mm, in solid drawn brass – Q.ty 4 per leaf for bottom leaves, 3 per leaf for top leaves |  |
| Vanity unit in shower room | Oliver Knight Bausan CH, 120mm length, in nickel | SDS London Twin Ball Catch 50 mm, Ref 69025, Satin Chrome plated – Q.ty 1 | SDS London Soss 208 Hinge, 70 x 16mm, nickel plated– Q.ty 3 per leaf |  |
| WINDOWS AND FRENCH DOOR | HANDLE | LOCK | HINGE | ACCESSORIES |
| D18 French door to terrace | Oliver Knight Frederick Lever on Reynfred plate with cut out for Yale Doormaster euro cylinder in brushed brass – Q.ty 1 | Yale Doormaster Professional Multipoint Lock 35mm Back Set for Timber, Ref 78876 | Simonswerk hinge to Simon Ayres suggestion. | SDS London Hooded door stop 35021 in Antique Brass Unlacquered - Q.ty 2 |
| Casement windows W01, W02 and W08 | SDS London Reeded Mortice Plate Fastener, Ref. 30087, in Antique Brass Unlacquered – Q.ty 1 per leaf + Reeded Casement Stay 254 mm, Ref 30086, in Antique Brass Unlacquered - Q.ty 1 per leaf + Casement Stay Locking Pin and key, Ref 30082, in Antique Brass Unlacquered - Q.ty 1 per leaf |  | SDS London Broad Suite Butt Hinge 100 x 75 mm, Ref. 32034, Solid Drawn Brass – Q.ty 2 per leaf |  |
| Secondary glazing to W02 | SDS London Reeded Hook Plate Fastener, Ref. 30088, in Antique Brass Unlacquered – Q.ty 1 set |  | SDS London Broad Suite Butt Hinge 100 x 75 mm, Ref. 32034, Solid Drawn Brass – Q.ty 2 per leaf |  |
| Sash windows W03 and W06 |  | SDS London Sash Window Brighton Set – Locking, Ref. 91161, in Polished brass lacquered – Q.ty 1 per window |  | NOTE: Sash pulley and cord to be Contractor’s choice |
| MISCELLANEOUS |  |  |  |  |
| Bedroom wardrobes | SDS London Quality Rod Socket 32 mm in Polished Brass Unlacquered, Ref 14048 (2no. per wardrobe) + 32 mm Round Solid Brass Rail 2000mm Ref 14975 (1no. per wardrobe) |  |  |  |
| Closet | SDS London Quality Rod Socket 32 mm in Polished Brass Unlacquered, Ref 14048 (4no.) + 32 mm Round Solid Brass Rail 2000mm Ref 14975 (2no.) |  |  |  |
| Shower room cupboards | Hafele Magnetic pressure catch 245.62.311 + 246.03.790 counterplate – Q.ty 1 per door |  |  |  |
| Study bookcases | SDS London Flat Bookcase Strip 1829mm, Ref. 49002, Polished Brass Unlacquered – Q.ty 4 per bookcase bay + SDS London Shelf Stud for Flat Strip, Ref. 49004, Polished Brass Unlacquered – Q.ty 4 per shelf |  |  |  |