### OCCUPATIONAL THERAPY

ADAPTATION

@

9 SPEDAN CLOSE

BRANCH HILL ESTATE

NW3 7XF

SPECIFICATION OF WORKS – 26/07/2016

TO SUPPLY & FIT BATHROOM WET AREA FLOOR SHOWER IN BATHROOM

- GENERAL

- SCHEDULE OF WORKS

C.  PLANS

Housing Repairs Service

Renewals Division

020 7974 3913

- GENERAL

1. Scope of Works

The work is to fit a graded wet area floor shower in the bathroom as specification. This is a listed building therefore work is subject to planning approval.

2. Period of the works

The work must be completed within a two-week period of start on site. A continuity of work must be maintained in order to minimise disruption and disturbance of the occupants. The flat & building will be occupied throughout the period of the works and facilities must be kept useable.

3. Safety of occupants of the block

The contractor shall ensure that all adequate and necessary precautions are taken to prevent accidents and inconvenience to other occupants of the block. At the end of each working day the contractor must ensure that the site is left safe and secured, with all communal areas clean & tidy.

4. Start date

The works should commence ASAP within receipt of an official order, and should be completed within two weeks of start date.

5. Details to be agreed on site

The position of the shower seat, shower control, and riser bar are to be agreed with the occupational therapist on site, unless indicated otherwise. All measurements to be checked on site.

6. Contact Information

Client/tenant: AS OT REQUEST

Occupational therapist: AMY MISSION

Surveyor:	Stephen Williams

CONTRACT PLANNER – ANTONY REILEY – 0207 974 8603

- SCHEDULE OF WORKS

BATHROOM WET FLOOR SHOWER

| ITEM | DESCRIPTION OF WORKS | UNIT | QUANT | RATE | TOTAL |
| --- | --- | --- | --- | --- | --- |
| 1. | NOTE: ASBESTOS SURVEY REQUIRED COMMUNAL HEATING |  |  |  |  |
| 2. | Protection Allow for the protection of furniture/fittings, floor finishes and windows etc. as work proceeds and reinstate on completion. | Item |  |  |  |
| 3. | Demolition (The bathroom doorway must be sealed with plastic sheeting so that the demolition works can be carried out creating the minimum of dust and thus protecting the landing area.) | Item |  |  |  |
| 4. | Isolate supplies to BATH and Remove | Item |  |  |  |
| 5. | Remove plinth and all existing wall tiles within shower area | Item |  |  |  |
| 6. | Uplift existing floor coverings, remove floor, form recess and fit new shower former. Remove all debris from site. | Item |  |  |  |
| 7. | REMOVE RADIATOR | ITEM |  |  |  |
|  | ##### Bath Adaptations |  |  |  |  |
| 8. | S&F new double convector radiator LOCATED AS PLAN | ITEM |  |  |  |
| 9. | Make good disturbed plaster where walls/items/tiles etc removed | Item |  |  |  |
| 10. | Relocate window winder outside shower area | item |  |  |  |
| 11. | Fill void in floor where bath removed | item |  |  |  |
| 12. | Fit Altro strip in the shower area, to take tiles to above and vinyl below. Tile in the new shower room, full height on walls around shower area with 150mm x 150mm ceramic wall tiles, waterproof grouted, and finished at exposed edges. Ensure contrast of tiles and shower controls/grab rails. | Item |  |  |  |
| 13. | Supply & fit Mira “Advance” thermostatic electric shower unit. Set at maximum 43 degrees Celsius. Run new mains water connection, and electrical supply, pipe work and cable, to include all necessary fittings, inline speed fit ‘Scale master’, isolating valves and double check valves, to comply with current regulations. Paint all exposed pipes on completion. The head must be detachable on a 1000mm extended riser bar, and with a 1500mm flexible hose, handset, and soap dish. The control and riser bar will be fitted as per plan or DIRECTED BY OCCUPATIONAL THERAPIST. | Item |  |  |  |
| 14. | Supply and fit L- SHAPED shower curtain rail, 100mm inside of the edge of the shower former, as shown on the drawing, and fit 2no full length weighted shower curtain just touching the floor. | Item |  |  |  |
| 15. | Supply and fix 2 no shower grab rails, AS plan / DIRECTED BY OCCUPATIONAL THERAPIST? | Item |  |  |  |
| 16. | Supply and fit a GRAVITY WASTE IF POSSIBLE OR Trays Direct ‘PGTF’ Waste and WHALE Pump, to include all necessary connections to new shower area and existing waste pipe. Fit a waste trap that will allow access from the top for cleaning and rodding. | Item |  |  |  |
| 17. | Box in pipes / pump as required inc access panels | ITEM |  |  |  |
| 18. | Supply and install a 1200MM x 1200mm IMPEY easy fit shower former | Item |  |  |  |
| 19. | Provide new electrical supply to shower, concealed in the shower area and surface mounted in mini trunking elsewhere, to include fused spur RCD and suitably rated MCB, all electrical work to fully comply with current 17th Edition Regulations. Provide Earth Bonding in bathroom as necessary. | Item |  |  |  |
| 20. | S&F Window shower curtain kit | ITEM |  |  |  |
| 21. |  |  |  |  |  |
| 22. |  |  |  |  |  |
| 23. |  |  |  |  |  |
| 24. |  |  |  |  |  |
| 25. | Prepare the floor, latex level, with any falls to be in direction of shower tray, and supply and fit slip resistant ‘Altro AQUARIOUS’ slip resistant vinyl floor covering complete with 100mm coving, in accordance with manufacturer’s instructions. Client should be offered a choice of standard colours. Fit a screw down aluminium threshold strip at the doorway. |  |  |  |  |
| 29. | MAKE GOOD DECORATIONS DISTURBED | Item |  |  |  |
| 30. | Remove all rubbish and leave affected area clean and tidy. | Item |  |  |  |
| 31. | Test and ensure that services are in good working order | Item |  |  |  |
| 32. | Explain to client the operation of the new installation and hand over manufactures information and/or instructions to client on completion. | Item |  |  |  |
| 33. | Carry out works to Building Regulations, and on completion submit a NICEIC electrical certificate (1 copy to Surveyor / 1 copy to tenant). | Item |  |  |  |

PRODUCTS

MIRA ADVANCE FLEX EXTRA ELECTRIC SHOWER

ALTRO AQUARIOUS SLIP RESISTANT VINYL FLOORING (RESIDENT TO CHOOSE COLOUR)