Mr Aaron D’Austin

Residential Building Solutions

Aaron@rbslondon.co.uk

21 Clifton Avenue, Stanmore, Middlesex, HA7 2HR

TEL: 07807 234 683

-----------------------

Site of 21 Canfield Place, London, NW6 3BT

-----------------------------

Building Materials:

Floor:

75mm Sand/cement screed min 80mm insulation (Celotex GA3000) on 100mm oversite concrete slab on 1200g polythene DPM dressed into existing DPC on 50mm sand blinding on minimum 1599mm well compacted hard-core.  Duropolymer skirting’s to be used.

Fittings and Services:

All electrical points, sockets, communication points, heating systems, water/gas/electricity

meter, will be placed above the predicted flood level of above 300mm.

For pipe works closed cell insulation below flood level to be used

Drainage services - Non return valves will be used in the drainage system

Doors:

All external doors to be UPVC flood doors kitemarked to BSI PAS 1188-1:2014 creating and lock and seal system to protect from flood water. All cills to be waterproof.

Windows:

All new windows will be installed 1m above ground level and to be UPVC flood windows kitemarked to BSI PAS 1188-1:2014 creating and lock and seal system to protect from flood water. All cills to be waterproof.