25th September 2019

Camden

Development Management

Camden Town Hall Extension

Argyle Street

London

WC1H 8EQ.

Dear Team,

SITE: PIZZERIA EXPRESS, 85-87 PARKWAY, LONDON, NW1 7PP

DESIGN AND ACCESS STATEMENT

Use

Restaurant

Amount

Not applicable.  Our application is for signage / advertisements, alterations to the shopfront and the inclusion of screens to the outside of the premises and we are proposing no other development to the site with this application.

Layout

Please refer to drawing nos: 596135-1, 2, 3, 4 & 5.

Scale

Not applicable.  Our application relates only to the installation of signage / advertisements, alterations to the shopfront and the inclusion of screens to the outside of the premises.  We are not proposing any new buildings as part of this application.

Landscaping

Not applicable.  Our application relates only to the installation of signage / advertisements, alterations to the shopfront and the inclusion of screens to the outside of the premises.

No new landscaping is to take place or any existing landscaping impacted upon as part of this application.

Appearance

Please refer to drawing nos. drawing nos: 596135-1,2,3,4 & 5

2 SETS OF EXTERNALLY ILLUMINTED SIGN WRITTEN TEXT – Black timber fascia panel.  The text is gold leaf applied to the timber panel on the main text and sign written in white on the address. Externally illuminated by a black steel trough fitted with low energy warm white LEDs.

1 INTERNALLY ILLUMINATED PROJECTION SIGN – Manufactured with 1.2mm descaled stainless steel faces and returns which are finished satin black.  Large circle is fret cut out of the faces and fitted with push through 5mm opal 030 acrylic. Fret cut logo is 2mm black aluminium which is inlaid into the engraved opal acrylic circles.  The numerals and underline at the bottom are also fret cut out of the faces and with fitted push through 8mm 030 opal acrylic which is faced with opaque gold leaf.  Fitted with small white low energy warm white LEDs for halo illumination of the graphics.  Fixing arms are covered by a folded steel black shroud.

INTERNALLY ILLUMINATED MENU – Menu fixed to the wall.  Descaled stainless steel folded and welded box finished satin black.  Front display backed in laminated 6mm glass.  Internally illuminated by low energy white LEDs fitted in the top recess.

Our design team has looked at all aspects of upgrading the signage at this site and has always been aware to produce a design concept that compliments the listed building and surrounding area.  We feel the subtle change in the signage will have no detrimental impact on the building and surrounding units.  All existing fixtures and fittings where possible will be used.

Access

Not applicable.