25th September 2019

Camden

Development Management

Camden Town Hall Extension

Argyle Street

London

WC1H 8EQ.

Dear Team,

SITE: PIZZERIA EXPRESS, 85-87 PARKWAY, LONDON, NW1 7PP

HERITAGE STATEMENT

.

The site is located on the south east side of Parkway and comprises of 2 mid-terraces, 3 storey properties with a restaurant occupied by Pizza Express at ground floor level.  The site is also within the Camden Town Conservation Area.

Pizza Express has occupied the premises for a number of years and there have been many changes to signage during this period.

No 85 is not listed but no. 87 Parkway is a Grade II Listed Building.

The works to the building are as follows:-

Appearance

Please refer to drawing nos. drawing nos: 596135-1,2,3,4 & 5.

2 SET OF EXTERNALLY ILLUMINTED SIGN WRITTEN TEXT – Black timber fascia panel.  The text is gold leaf applied to the timber panel on the main text and sign written in white on the address. Externally illuminated by a black steel trough fitted with low energy warm white LEDs.

1 INTERNALLY ILLUMINATED PROJECTION SIGN – Manufactured with 1.2mm descaled stainless steel faces and returns which are finished satin black.  Large circle is fret cut out of the faces and fitted with push through 5mm opal 030 acrylic. Fret cut logo is 2mm black aluminium which is inlaid into the engraved opal acrylic circles.  The numerals and underline at the bottom are also fret cut out of the faces and with fitted push through 8mm 030 opal acrylic which is faced with opaque gold leaf.  Fitted with small white low energy warm white LEDs for halo illumination of the graphics.  Fixing arms are covered by a folded steel black shroud.

INTERNALLY ILLUMINATED MENU – Menu fixed to the wall.  Descaled stainless steel folded and welded box finished satin black.  Front display backed in laminated 6mm glass.  Internally illuminated by low energy white LEDs fitted in the top recess.

Our design team has looked at all aspects of upgrading the signage at this site and has always been aware to produce a design concept that compliments the listed building and surrounding area.  We feel the subtle change in the signage will have no detrimental impact on the building and surrounding units.  All existing fixtures and fittings where possible will be used.

We are of the opinion that the heritage assets are secure in respect of the works to the site.  There will be less signage than existing in muted heritage colours made with superior finishes.  All heritage assets are protected.

Access

Not applicable.