## STANDARD METHOD OF WORK

| # Method of Work: Super-heated Steam Cleaning |  | Ref No: SMW037 |
| --- | --- | --- |
| #### CLEANING METHOD To carry out cleaning of brick, stonework or general masonry using the Super-heated Steam System as per the Method Statement and Recommendations. All work to be undertaken in accordance with the requirements of BS8221-1 : Cleaning of Natural Stone - Install appropriate barriers sheeting to external windows and openings to minimise water ingress to the building and shift with progress of work. - Position Torik steam cleaner to an agreed location and provide bund tray below fuel tank to collect any diesel leakages. - Heat water through specialist plant to approximately 150 degrees. - Apply to surface of stone and brickwork via specialist nozzle to remove all previously softened dirt deposits. - Rotate labour at hourly intervals. - Refuel Torik steam cleaner using 5lt jerry can with flexible spout. Use spill kit immediately if required. No fuel to be stored on site. - Wash down with clean cold water. COSHH AND PRODUCT DATA DETAILS ENCLOSED: |  |  |
| Originators Name: Brian Strydom | ## Last Review Date:06/12/2019 |  |