Adam Buczakowski

Winthrope House

White City Estate

Shepherd’s Bush

London

W12 7QB

Mobile: 07517 917145

12 March 2020

Nutmeg Properties Limited

16 Finchley Road

St Johns Wood

London

NW8 6EB

Re: 	Works at 20 Belsize Square, London NW3 4HT

Following inspection of the above named site we advise as follows :

- Scaffold erected at property in order to carry out works

- Gained access to loft space to prepare area and sheet up room

- Measure out dimensions in accordance with the architect’s drawings

- Proceed to remove slates on roof to open up

- Cut back roofing felt and batons where required

- Prepare supports inside loft space ready to cut out timbers

- Cut out timbers as required

- Re-trim around rooflight opening – install double trimmers

- Offer up new roof light, position and fix

- Fit flashing kit

- Cut slates as appropriate on roof and re-fix around rooflight in conjunction with the flashing kit

- Re-fix internal trim in loft space

- Remove supports and clear area