| # LDC (Proposed) Report |  | # Application number | # 2020/3409/P |
| --- | --- | --- | --- |
| ## Officer |  | ## Expiry date |  |
| ## Raymond Yeung |  | ## 25/09/2020 |  |
| ## Application Address |  | ## Authorised Officer Signature |  |
| 108 South Hill Park London NW3 2SN |  |  |  |
| ## Conservation Area |  | ## Article 4 |  |
| ## South Hill Park |  | ## Basements |  |
| ## Proposal |  |  |  |
| Erection of an outbuilding incidental to the dwellinghouse (Use Class C3). |  |  |  |
| ## Recommendation: | ## Grant Certificate |  |  |

The property is a 3 storey terraced property, it is not listed but is within South Hill Park conservation area.

Amended plans were submitted to replace the initial proposed drawings as initially it included new steps and an outdoor balcony area outside the front door, this was considered not permitted development and hence amended plans were submitted to remove the new steps and to retain the existing ones unaltered, also to remove the outdoor balcony area following relocation of the front door to the outbuilding.

The householder technical guidance document that accompanies the GPDO clarifies

how height is measured when assessing development:

“Height” - references to height (for example, the heights of the eaves on a house

extension) is the height measured from ground level. (Note- ground level is the surface of the ground immediately adjacent to the building in question, and would not include any addition laid on top of the ground such as decking. Where ground level is not uniform (for example if the ground is sloping), then the ground level is the highest part of the surface of the ground next to the building.)

The proposed plans demonstrate that the parts of the building is of 4.5 metres, however the building would be no more than 2.5 metres when measured from the highest point of the existing ground level closest to the building, in accordance with the requirements of Class E.1 (e).

Assessment against Class E of Part 1 of Schedule 2 to the Town and Country Planning (General Permitted Development) (England) Order 2015, to whether it is permitted development or require planning permission is demonstrated below.

| Class E The provision within the curtilage of the dwellinghouse of— a) any building or enclosure, swimming or other pool required for a purpose incidental to the enjoyment of the dwellinghouse as such, or the maintenance, improvement or other alteration of such a building or enclosure; or b) a container used for domestic heating purposes for the storage of oil or liquid petroleum gas |  |  |
| --- | --- | --- |
| If ‘yes’ to any of the questions below the proposal is not permitted development |  | Yes/no |
| E.1 (a) | As a result of the works, will the total area of ground covered by buildings, enclosures and containers within the curtilage of the dwellinghouse (other than the original dwellinghouse) exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse)? Existing Outbuildings 5 sq.m Proposed outbuilding 75 sq.m Rear garden area 530 sq.m = Coverage from outbuildings within the rear curtilage following proposal Approx.. 15% | NO |
| E.1 (b) | Would any part of the building, enclosure, pool or container be situated on land forward of a wall forming the principal elevation of the original dwellinghouse? | NO |
| E.1 (c) | Would the building have more than one storey? | NO |
| E.1 (d) | Would the height of the building, enclosure or container exceed— i) 4 metres in the case of a building with a dual-pitched roof, ii) 2.5 metres in the case of a building, enclosure or container within 2 metres of the boundary of the curtilage of the dwellinghouse, or iii) 3 metres in any other case? | NO |
| E.1 (e) | Would the height of the eaves of the building exceed 2.5 metres? | NO-Measured from the highest part of ground adjacent to host dwelling |
| E.1 (f) | Would the building, enclosure, pool or container be situated within the curtilage of a listed building? | NO |
| E.1 (g) | Would it include the construction or provision of a veranda, balcony or raised platform? | NO |
| E.1 (h) | Does it relate to a dwelling or a microwave antenna? | DWELLING |
| E.1 (i) | Would the capacity of the container exceed 3,500 litres? | NO |
| Is the property in a conservation area? If yes to the question below then the proposal is not permitted development |  |  |
| E.3 | Would any part of the building, enclosure, pool or container be situated on land between a wall forming a side elevation of the dwellinghouse and the boundary of the curtilage of the dwellinghouse? | NO |

* The land referred to as article 2(3) land is the land described in Part 1 of Schedule 1 to Town and Country Planning (General Permitted Development) (England) Order 2015/596

It is demonstrated from the above, that the outbuilding is permitted development and therefore would not be required to be applied for planning permission.

Conclusion

Recommendation – Grant Certificate of Lawfulness

The garden outbuilding building is permitted under Class E of Part 1 of Schedule 2 to the Town and Country Planning (General Permitted Development) (England) Order 2015.