|  | S |
| --- | --- |

ENVIRONMENTAL HEALTH

SUPPORTING COMMUNITIES

| To: | David Fowler, Planning Officer, Development Management, 5 Pancras Square N1C |
| --- | --- |
| From: | Nick Priddle BSc (Hons) MSc AIEMA, MIOA (Technical Officer Contaminated Land & Noise Officer) |
| Date: | 14/10/20 |
| Address: | Clockwork Factory Apartments 13 Blackburn Road London NW6 1RZ |
| Proposal: | Demolition of existing building and construction of three buildings between 1 and 9 storeys (plus basement) in height comprising 53 residential dwellings, 4,797sqm of commercial floorspace, publically accessible space, landscaping and resident's facilities including cycle and refuse facilities.  Existing Land UseB1 Business, B8 Warehousing  Proposed Land UseB1 Business, B1a Business - Office, C3 Dwelling House |
| Reference: | 2020/2940/P |
| Key Points: | No objection subject to the three conditions recommended below |

# ENVIRONEMTAL HEALTH OBERVATIONS

PART 1

The following were reviewed in preparation of the comments below:

- Ground Investigation & BIA Report (Ref J19295) by GEA Ltd, dated 20 May 2020.

- Planning Statement

- DAS

Summary

The proposal includes basement provisions some 4mbgl and landscaping within a central courtyard for amenity use.  The central courtyard will be located above the basement and include raised planters for trees and ornamental planting, which effectively severs pollution linkages between soil contaminants and plants and human health.

I’m in general agreement that there is a low risk of land contamination at the application site due to historical land use activities at and neighbouring the site and vis versa in terms of the site impacting neighbouring site’s adjacent, as mentioned in the Ground Investigation report.

Radon

There is however concern about the risk from radon gas due to the basement This concern focuses on the human health risk to future occupants of the basement whilst acknowledging comments in Section 2.3 of the ground investigation report, which explains it is likely the site is within an area where <1% of homes are affected by radon gas and therefore no radon protection is required.  Concern is based on the Radon guidance BRE 211 (2015), which notes that all basements are at increased risk of elevated levels of radon regardless of geographic location, because more walls are in contact with the ground as well as the floor, and reduced natural ventilation below ground level increases the risk of elevated radon levels.  In addition, the Management of Health and Safety at Work Regulations (1999) require the assessment of health and safety risks and both the Health and Safety Executive (HSE) and Public Health England (PHE) state that this should include the measurement of radon for occupied below ground workplaces (occupied for more than 1 hour per week/52 hours of the year), irrespective of whether a site is situated in a radon affected area. This is the responsibility of the Employer. For residential developments Public Health England advise that consideration should be given to testing for radon if the basement includes rooms regularly used.

The guidance advocates radon testing to establish whether radon protection is necessary.

UKradon recently confirmed it is feasible to monitor radon levels in the field, but there is no established way of interpreting the results.  Radon levels in soil gas are typically 1000’s Bq m-3 but cannot be used to predict the likely radon level in a building on the same site.  To put field levels into perspective the radon Action and Target levels in homes are 200bq m3 and 100bq m3 -  Public Health England, advices if the action level is exceeded then levels should be reduce to the target level or below.  By contract if an employer has a workplace level above 300bq m3 (Ionising Radiations Regulations 2017) they are required to limit employee exposure to radon, which usually means building mitigation work.

Measuring radon is inexpensive but can take 3 months to complete.  It appears the earliest opportunity for measuring radon levels in basements may be during the fit out stage.   As a precautionary approach it may be prudent to consider making provisions for radon reduction at the design stage.   This may prove to be a cheaper option compared to altering the building fabric latter.  A condition has been recommended below to address the risk from radon.

Asbestos

It is proposed to demolish the two application buildings.  Based on their age it is possible that asbestos containing materials (ACMs) may have been used in construction and/or maintenance. If an up-to-date asbestos register is not available, it is recommended that an asbestos survey is undertaken so that any ACMs present can be managed accordingly to protect neighbours/occupiers/construction workers.  A condition has been recommended below to address this.

Recommendations

The Ground Investigation Report included the following recommendations to reduce the risk to human health:

- Before intrusive site works commence including ground works, piling and basement construction a Detailed UXO assessment should be undertaken as recommended by the Preliminary UXO assessment.

- A hydrocarbon odour was detected from arisings associated with the Made Ground during the installation of BH2 & BH5. Furthermore elevated levels of TPH & PAHs were identified within the Made Ground formation at BH1 & BH6, which may pose a risk to buried services incorporating plastic pipes for potable water supply.  Confirmation should obtained from Thames Water approving ground conditions and materials used for potable water supplies on site.

- Recommendations included a watching brief for areas of unexpected contamination during ground works especially beneath parts of the site to be excavated or not investigated during the BIA.  Once ground works have been completed a statement should be provided confirming or otherwise the discovery of any contaminants on site.

- Evidence should be provided that excavated waste material was disposed in accordance with the waste producers “Duty of Care Obligations.”

A condition has been recommended below so the LPA receive a Verification Statement demonstrating the above has been undertaken.

In principal the proposed development is considered acceptable in terms of environmental health risk from land contamination and contaminants, subject to the three conditions below:

- Small scale ground gas and vapour condition:

1a.	No development shall commence (other than demolition above ground level) until an appropriate radon gas and vapour investigation (incorporating detailed assessment of the risks to all receptors that may be affected) is undertaken and a radon gas and vapour assessment report (GVAR) [where necessary incorporating a Remediation Strategy (RS)], is submitted to, and approved in writing by, the local planning authority.

1b	No occupation or use of the development shall commence until the approved RS is implemented and a Verification Report (VR) is submitted to, and approved in writing by, the local planning authority.

Where remedial measures are implemented to protect end-users of the development they shall be maintained.

Reason: To ensure the risks from radon to the future users are acceptable.

- Asbestos

The developer must either submit evidence that site buildings were built post 2000 or provide an intrusive pre-demolition and refurbishment asbestos survey in accordance with HSG264 supported by and appropriate mitigation scheme to control risks to occupiers.  The scheme must be written by a suitably qualified person and submitted to the Local Planning Authority (LPA) for approval before commencement.   The scheme as submitted shall demonstrably identify potential sources of asbestos contamination and detail removal or mitigation appropriate for the proposed end use. Detailed working methods are not required but the scheme of mitigation shall be independently verified to the satisfaction of the LPA prior to occupation.

Reason To protect occupiers of the development from the possible contamination arising in connection with the buildings on the site in accordance with policies A1 (Managing impact of development), C1 (Health) and CC5 (Waste) of the London Borough of Camden Local Plan 2017

- Land Contamination

Prior to occupation of the development or stage in the development as may be agreed in writing by the Local Planning Authority (LPA) a Verification Statement shall be submitted to and approved in writing by the LPA demonstrating the recommendations in the Ground Investigation and Basement Impact Assessment Report (J19295) have bene undertaken for Detailed UXO assessment, to confirm the risk to buried potable water supplies is acceptable, confirming the discovery of unexpected contamination and confirming excavated waste was disposed is in accordance with the producers “duty of care” obligations.

Reason: To ensure the risks from land contamination to the future users of the land and neighbouring land are minimised, together with those to controlled waters, property and ecological systems, and to ensure that the development can be carried out safely without unacceptable risks to workers, neighbours and other offsite receptors, in accordance with policies G1, D1, A1, and DM1 of the London Borough of Camden Local Plan 2017.