| # LDC (Proposed) Report |  | # Application number | # 2020/5166/P |
| --- | --- | --- | --- |
| ## Officer |  | ## Expiry date |  |
| ## Raymond Yeung |  | ## 05/01/2021 |  |
| ## Application Address |  | ## Authorised Officer Signature |  |
| 7 Hilltop Road London NW6 2QA |  |  |  |
| ## Conservation Area |  | ## Article 4 |  |
| ## N/A |  | ## Basements |  |
| ## Proposal |  |  |  |
| Erection of a front porch to the dwellinghouse (Use Class C3). |  |  |  |
| ## Recommendation: | ## Grant Certificate |  |  |

The proposal is for;

Window fan light above the proposed front porch, this would be considered under Class A.

The creation of a porch to the front of the house, this would be considered under Class D.

The design of the porch and new fan light above to create the upper enclosure are to be read together and are permitted under different classes.

It should be noted that there are  also proposed steps of a height of approximately 500mm) leading up to the front porch, this is therefore considered to constitute as development and not permitted development and therefore require planning permission which they have not obtained yet to date.

Site description

The property is a detached dwelling, a Georgian house on two floors and benefits from an existing open porch enclosure to the front, it is not within the conservation area nor is the property listed.

| Class A The enlargement, improvement or other alteration of a dwellinghouse |  |  |
| --- | --- | --- |
| If yes to any of the questions below the proposal is not permitted development |  | Yes/no |
| A.1 (a) | Permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P, PA or Q of Part 3 of this Schedule (changes of use) | No |
| Comments: |  |  |
| A.1 (b) | As a result of the works, will the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse)? | No |
| A.1 (c) | Will the height of the part of the dwellinghouse enlarged, improved or altered exceed the height of the highest part of the roof of the existing dwellinghouse? | No |
| A.1 (d) | Will the height of the eaves of the part of the dwellinghouse enlarged, improved or altered exceed the height of the eaves of the existing dwellinghouse? | No |
| A.1 (e) | Will the enlarged part of the dwellinghouse extend beyond a wall which: - forms the principal elevation of the original dwellinghouse; or - fronts a highway and forms a side elevation of the original dwellinghouse? | No |
| A.1 (f) (subject to A.1 (g)) | Will the enlarged part of the dwellinghouse have a single storey and: - extend beyond the rear wall of the original dwellinghouse by more than 4 metres in the case of a detached dwellinghouse, or 3 metres in the case of any other dwellinghouse, or - exceed 4 metres in height? | No |
| A.1 (g) (until 30th May 2019) | For a dwellinghouse not on article 2(3) land* nor on a site of special scientific interest, will the enlarged part of the dwellinghouse have more than one storey and— - extend beyond the rear wall of the original dwellinghouse by more than 8 metres in the case of a detached dwellinghouse, or 6 metres in the case of any other dwellinghouse; or - exceed 4 metres in height? | N/A |
| A.1 (h) | Will the enlarged part of the dwellinghouse have more than a single storey and - extend beyond the rear wall of the original dwellinghouse by more than 3 metres, or - be within 7 metres of any boundary of the curtilage of the dwellinghouse opposite the rear wall the dwellinghouse? | No |
| A.1 (i) | Will the enlarged part of the dwellinghouse be within 2 metres of the boundary of the curtilage of the dwellinghouse, and the height of the eaves of the enlarged part exceed 3 metres? | No |
| A.1 (j) | Will the enlarged part of the dwellinghouse extend beyond a wall forming a side elevation of the original dwellinghouse, and either - exceed 4 metres in height, - have more than one storey, or - have a width greater than half the width of the original dwellinghouse? | No |
| A.1(ja) | Will any total enlargement (being the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceed the limits set out in A.1(e) to A.1(j)? | No |
| A.1(k) | Would it consist of or include either: - the construction or provision of a veranda, balcony or raised platform, - the installation, alteration or replacement of a microwave antenna, - the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or - an alteration to any part of the roof of the dwellinghouse? | No |
| Is the property in a conservation area (article 2(3) land)? If yes to any of the questions below then the proposal is not permitted development. |  |  |
| A.2(a) | Would it consist of or include the cladding of any part of the exterior of the dwellinghouse with stone, artificial stone, pebble dash, render, timber, plastic or tiles? | No |
| A.2(b) | Would the enlarged part of the dwellinghouse extend beyond a wall forming a side elevation of the original dwellinghouse? | No |
| A.2(c) | Would the enlarged part of the dwellinghouse have more than a single storey and extend beyond the rear wall of the original dwellinghouse? | No |
| A.2(d) | Would any total enlargement (being the enlarged part together with any existing enlargement of the original dwellinghouse to which it will be joined) exceed the limits set out in sub-paragraphs A.2(b) and A.2(c)? | No |
| Conditions. If no to any of the below then the proposal is not permitted development |  |  |
| A.3(a) | Would the materials used in any exterior work (other than materials used in the construction of a conservatory) be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse? | Yes |
| A.3(b) | Would any upper-floor window located in a wall or roof slope forming a side elevation of the dwellinghouse be— - obscure-glazed, and - non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed? | N/A |
| A.3(c) | Where the enlarged part of the dwellinghouse has more than a single storey, or forms an upper storey on an existing enlargement of the original dwellinghouse, would the roof pitch of the enlarged part, so far as practicable, be the same as the roof pitch of the original dwellinghouse? | No |

| Class D The erection or construction of a porch outside any external door of a dwellinghouse If yes to any of the questions below the proposal is not permitted development Yes/no |  |  |
| --- | --- | --- |
| D.1(a) Would the ground area (measured externally) of the structure exceed 3 square metres? | No |  |
| D.1(b) Would any part of the structure be more than 3 metres above ground level? | No |  |
| D.1(c) Would any part of the structure be within 2 metres of any boundary of the curtilage of the dwellinghouse with a highway? | No |  |

Assessment: Approval