| # LDC (Proposed) Report |  | # Application number | # 2020/4867/P |
| --- | --- | --- | --- |
| ## Officer |  | ## Expiry date |  |
| ## Raymond Yeung |  | ## 15/12/2020 |  |
| ## Application Address |  | ## Authorised Officer Signature |  |
| 41 Belsize Road London NW6 4RX |  |  |  |
| ## Conservation Area |  | ## Article 4 |  |
| ## N/A |  | ## Basements |  |
| ## Proposal |  |  |  |
| Erection of a rear roof dormer extension (Class C3). |  |  |  |
| ## Recommendation: | ## Grant approval |  |  |

The property is an end of terrace 1960s three storey property. The property has a specifically designed rear elevation which has the rear facade set within a frame of projecting eaves and party walls to create an alcove design; the applicant is planning to infill these areas as part of a separate application.

The property is not listed nor is it within a conservation area.

The proposal is for a full width roof dormer, projecting out as far as the eaves beyond the rear elevation wall.

| Class B The enlargement of a dwellinghouse consisting of an addition or alteration to its roof |  |  |
| --- | --- | --- |
| If yes to any of the questions below the proposal is not permitted development |  |  |
| As a result of the works, would any part of the dwellinghouse exceed the height of the highest part of the existing roof? |  | No |
| As a result of the works, would any part of the dwellinghouse extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway? |  | No |
| As a result of the works, would the cubic content of the resulting roof space exceed the cubic content of the original roof space by more than— (i) 40 cubic metres in the case of a terrace house, or (ii) 50 cubic metres in any other case? | 4.7m2 (cross section) x 6.1m (width) = 28.67m3 | No |
| would it consist of or include— - the construction or provision of a veranda, balcony or raised platform, or - the installation, alteration or replacement of a chimney, flue or soil and vent pipe? |  | No |
| Is the dwellinghouse on article 1(5) land? |  | No |
| Would the materials used in any exterior work be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse? |  | Yes |
| Other than in the case of a hip-to-gable enlargement, would the edge of the enlargement closest to the eaves of the original roof be less than 20 centimetres from the eaves of the original roof, so far as practicable? |  | Yes |
| Would any windows inserted on a wall or roof slope forming a side elevation be obscured-glazed and non-opening unless the opening part is higher than 1.7 metres above the floor of the room in which the window is installed? |  | Yes |

Assessment: Complies with criteria for Class B, thus is permitted development