| # LDC (Proposed) Report |  | # Application number | # 2020/5777/P |
| --- | --- | --- | --- |
| ## Officer |  | ## Expiry date |  |
| ## Raymond Yeung |  | ## 05/02/2021 |  |
| ## Application Address |  | ## Authorised Officer Signature |  |
| 27 Ulysses Road London NW6 1ED |  |  |  |
| ## Conservation Area |  | ## Article 4 |  |
| ## N/A |  | ## Basements |  |
| ## Proposal |  |  |  |
| Erection of a rear roof dormer and installation of two front rooflights to the dwellinghouse (Class C3). |  |  |  |
| ## Recommendation: | ## Grant approval |  |  |

The property is a mid-terrace 2/3 storey house which benefits from an original 2 storey rear outrigger. The property is not listed and not within a conservation area.

The application is for a rear dormer which is assessed under Class B and 2 front roof lights which are assessed under Class C.

| Class B The enlargement of a dwellinghouse consisting of an addition or alteration to its roof |  |  |
| --- | --- | --- |
| If yes to any of the questions below the proposal is not permitted development |  |  |
| As a result of the works, would any part of the dwellinghouse exceed the height of the highest part of the existing roof? |  | No |
| As a result of the works, would any part of the dwellinghouse extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway? |  | No |
| As a result of the works, would the cubic content of the resulting roof space exceed the cubic content of the original roof space by more than— (i) 40 cubic metres in the case of a terrace house, or (ii) 50 cubic metres in any other case? | Approx: 2.7m height x3.5m depth x 5.3m width =50m3 / 2 = 25m3 | No |
| would it consist of or include— - the construction or provision of a veranda, balcony or raised platform, or - the installation, alteration or replacement of a chimney, flue or soil and vent pipe? |  | No |
| Is the dwellinghouse on article 1(5) land? |  | No |
| Would the materials used in any exterior work be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse? |  | Yes |
| Other than in the case of a hip-to-gable enlargement, would the edge of the enlargement closest to the eaves of the original roof be less than 20 centimetres from the eaves of the original roof, so far as practicable? |  | Yes |
| Would any windows inserted on a wall or roof slope forming a side elevation be obscured-glazed and non-opening unless the opening part is higher than 1.7 metres above the floor of the room in which the window is installed? |  | N/A |

| Class C – other alterations to the roof of a dwellinghouse. |  |  |
| --- | --- | --- |
| C.1 | Development is not permitted by Class C if— |  |
| C.1 (a) | permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); | No |
| C.1 (b) | the alteration would protrude more than 0.15 metres beyond the plane of the slope of the original roof when measured from the perpendicular with the external surface of the original roof; | No |
| C.1 (c) | it would result in the highest part of the alteration being higher than the highest part of the original roof; or | No |
| C.1 (d) | it would consist of or include— |  |
| C.1 (d i) | the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or | No |
| C.1 (d ii) | the installation, alteration or replacement of solar photovoltaics or solar thermal equipment. | No |
| Conditions: |  |  |
| C.2 | Development is permitted by Class C subject to the condition that any window located on a roof slope forming a side elevation of the dwellinghouse must be— |  |
| C.2 (a) | obscure-glazed; and |  |
| C.2 (b) | non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. | No |

Assessment: Accords with criteria of Classes B and C and thus both are permitted development.