# LONDON BOROUGH OF CAMDEN

# MEMORANDUM

| Date: | Tuesday 15th October 2019 |
| --- | --- |
| To: | Insurance Section |
| Ref: | 18TR000049 |
| From: | Jake Camilleri |
| Our Ref: | 17214 - 23351 |

## ALLEGED TREE ROOT DAMAGE REPORT

| Address of Property: | 20 Churchill Road, NW5 1AN |
| --- | --- |
| Department/s Responsible for Tree/s: | Camden Borough London, 18 Churchill Road |
| Date of inspection and Officer: | 15th October 2019 |
| Maintenance History: |  |
| Camden Tree SEQ 81 (T1) Platanus x hispanica Completion Date Item 06/03/2015 Crown reduction 40% & Reshape - Large Specific branch/limb removal (150mm) General Prune (GP.3 ) Re-reduction - Large Remove self sowns in pit 19/03/2018 General Prune (GP.3) - Medium Camden Tree SEQ 80 (T2) Platanus x hispanica Completion Date Item 06/03/2015 Crown reduction 40% & Reshape - Large Specific branch/limb removal (150mm) General Prune (GP.3 ) Re-reduction - Large Remove self sowns in pit 19/03/2018 General Prune (GP.3) - Medium Camden Tree SEQ 79 (T3) Platanus x hispanica Completion Date Item 06/03/2015 Crown reduction 40% & Reshape - Large Specific branch/limb removal (150mm) General Prune (GP.3 ) Re-reduction - Large Remove self sowns in pit 19/03/2018 General Prune (GP.3) - Medium Camden Tree SEQ 78 (T4) Platanus x hispanica Completion Date Item 06/03/2015 Crown reduction 40% & Reshape - Large Specific branch/limb removal (150mm) General Prune (GP.3 ) Re-reduction - Large Remove self sowns in pit 19/03/2018 General Prune (GP.3) - Medium Camden Tree SEQ 77 (T5) Platanus x hispanica Completion Date Item 06/03/2015 Crown reduction 40% & Reshape - Large Specific branch/limb removal (150mm) General Prune (GP.3 ) Re-reduction - Large Remove self sowns in pit 19/03/2018 General Prune (GP.3) - Medium Camden Tree SEQ 76 (T6) Platanus x hispanica Completion Date Item 06/03/2015 Crown reduction 40% & Reshape - Large Specific branch/limb removal (150mm) General Prune (GP.3 ) Re-reduction - Large Remove self sowns in pit 19/03/2018 General Prune (GP.3) – Medium Camden Tree SEQ 75 (T7) Platanus x hispanica Completion Date Item 06/03/2015 Crown reduction 40% & Reshape - Large Specific branch/limb removal (100mm) General Prune (GP.3 ) Re-reduction - Large Remove self sowns in pit 19/03/2018 General Prune (GP.3) - Medium Camden Tree SEQ 74 (T8) Platanus x hispanica Completion Date Item 06/03/2015 Crown reduction 40% & Reshape - Large Specific branch/limb removal (100mm) General Prune (GP.3 ) Re-reduction - Large Remove self sowns in pit Inspect Possible fungal bracket at 7m going towards the west Provide specified climbing inspection report - Large 19/03/2018 General Prune (GP.3) – Medium Camden Tree SEQ 73 (T9) Acer pseudoplatanus Item 06/03/2015 General Prune (GP.1) – Medium Specific branch/limb removal (100mm) Remove self sowns in pit 19/03/2018 General Prune (GP.1) - Medium |  |
| Cyclical inspection and maintenance programme. Trees, T1,T2,T3,T4,T5,T6,T7 and T8 are all inspected once every three years and re-reduced once every three years. T9 is inspected is inspected once every three years with work being raised appropriately. ST1, ST2,ST3 are not officially listed on our maps, They are small, self-sown trees which have arisen via succession. |  |

| Claims Ref: 19TR000002 | Address: St. Georges Terrace |
| --- | --- |

| Tree No. | Ownership | Species | Height (m) approximate | Crown Spread (m) | DBH (cm) | Vitality | Water Demand | Distance to Boundary (m) | Distance to Property (m) | CAVAT Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | London Borough Camden | Platanus x hispanica | 5 | 4 | 144cm | Good | Moderate | 10 | 12 | £12531.41 |
| T2 | London Borough Camden | Platanus x hispanica | 6 | 4 | 84cm | Good | Moderate | 12 | 14 | £4375.64 |
| T3 | London Borough Camden | Platanus x hispanica | 6 | 3 | 157cm | Good | Moderate | 10 | 12 | £16182.09 |
| T4 | London Borough Camden | Platanus x hispanica | 6 | 4 | 176cm | Good | Moderate | 9 | 11 | £16182.09 |
| T5 | London Borough Camden | Platanus x hispanica | 5 | 6 | 56cm | Good | Moderate | 9 | 11 | £10356.53 |
| T6 | London Borough Camden | Platanus x hispanica | 5 | 5 | 49cm | Good | Moderate | 12 | 14 | £16182.09 |
| T7 | London Borough Camden | Platanus x hispanica | 5 | 5 | 180cm | Good | Moderate | 14 | 16 | £23302.21 |
| T8 | London Borough Camden | Platanus x hispanica | 5 | 6 | 214cm | Good | Moderate | 16 | 18 | NA |
| T9 | London Borough Camden | Acer pseudoplatanus | 6 | 4 | 110cm | Good | Moderate | 17 | 19 | £3452.18 |
| T10 | NA | Fraxinus excelsior | 7 | 4 | NA | Good | Moderate | 16 | 18 | NA |
| H1 | 18 Churchill Road | Privet Spp. | 2 | NA | NA | Good | NA | 5 | 6 | NA |
| C1 | 18 Churchill Road | Wisteria sinensis | 3.5 | 4 | NA | Good | NA | 5 | 5 | NA |
| ST1 | London Borough Camden | Acer spp. | 0.5 | NA | NA | Good | Moderate | 9 | 11 | NA |
| ST2 | London Borough Camden | Prunus padus | NA | NA | NA | NA | Moderate | 9 | 11 | NA |
| ST3 | London Borough Camden | Fraxinus excelsior | 1.5m | N/A | 5 | Good | Moderate | 12 | 14 | NA |

TG.1

| T11 | NA | Acer pseudoplatanus | 15m | 6m | 80cm | Good | Moderate | 16 | 18 | NA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In the original Arb report the area ‘TG1.’  Was listed as ‘Mixed Species Group’. This area holds a sub-station, which is surrounded by some land and a fence. It has some mature trees and small shrubs none of which appear to have cyclical pruning regimes in place. The area was not directly accessible due to the large fence, so some details are estimated or missing altogether. I specifically looked at the biggest tree in that area, T11.

Data taken from Confirm database and/ or onsite inspection

TREE/VEGETATION ISSUES: - Trees T1, T2, T3 ,T4,T5,T6,T7 and T8 are all rated as good in health. No trees had any significant health or structural defects at point of inspection. Each one of these trees is inspected and re-reduced once every three years. - T9 had no significant health or structural defects at point of inspection, this tree is inspected once every three years with work being raised appropriately. - T10 was in good health at the time of inspection with no major structural defects noted. - H1 was in good health at time of inspection with no major structural defects noted. - C1 was in good health at time of inspection with no major structural defects noted. - ST1 and ST3 was in fair health at time of inspection with no structural defects. - ST2 had been felled at time of inspected. - T11 (TG1) is an area approximately 10m x 10m with some mature trees and small shrubs. It cannot be ruled out that this area is not having an effect on soil moisture levels.

STRUCTURE ISSUES: Data gathered from Structural engineers report dated: 24th January 2019 Ref:QG1T1074537 - Externally, we noted severe crack damage to the left hand splay of the two storey bay with crack widths up to 30 mm. - The left hand splay was also distorted. - A separation crack was viewed between the left hand central pier of the bay and the sash window frame of 10 mm. - Cracking was also noted to the lower ground floor window cill and onto the brickwork below to all sections of the bay. - The brick built bin store had pulled away from the wall and was close to collapse. - The front retaining wall opposite the bay was out of plumb. - Internally, cracking was noted to the painted timber panel below the left hand splay of - The bay 5-6 mm in width and the left hand window frame was distorted with a gap - Between the glazing and the timber – lounge of the lower ground floor flat. - The external entrance door to the lower ground floor flat was difficult to open The damage would be described as Category 4-5 using the BRE 251 Classification.

TECHNICAL INVESTIGATIONS: Data gathered from Structural engineers report dated: 24th January 2019 Ref:QG1T1074537 Trial Hole 1 - A trial hole was excavated adjacent to the front bay and that revealed a shallow Concrete footing with the underside 200 mm below ground level seated on stiff brown slightly sandy clay. - A trial hole extended by hand auger revealed this material to continue to a depth of 3000 mm where the trial hole was terminated. Laboratory Analysis - Analysis of soil samples taken from Trial Hole 1 revealed clay of very high plasticity that would be subject to volumetric change with changes in moisture content. - Evidence of desiccation was noted below foundation level. - Roots found below the foundation level were identified as Plane and shrubs. Trial Hole 2 - A trial hole was excavated adjacent to the front wall close to the entrance to the lower ground floor flat and that revealed a brick footing with the underside 150 mm below ground level seated on stiff brown slightly sandy clay. - A trial hole extended by hand auger revealed this material to continue to a depth of 3000 mm where the trial hole was terminated. Laboratory Analysis - Analysis of soil samples taken from Trial Hole 2 revealed clay of high to very high plasticity that would be subject to volumetric change with changes in moisture content. - Evidence of desiccation was noted below foundation level. - One root found below foundation level was identified as plane. Drainage Survey - The underground drainage system was surveyed and no defects identified.

COMMENTS: - Plane tree roots and ‘shrub’ roots have been found under the foundations of 20 Churchill Road. - The closest plane trees are 11 meters from the foundation of 20 Churchill Road. - The Plane trees are managed on a triennial re-reduction regime. - The closest shrubs are 5 meters from the foundation of the house. The maintenance record of the wisteria and hedge is unclear. Though they have had maintenance. - Around the base of the Plane trees is a lot of unmanaged vegetation. - The substation is approx. 18 – 20 meters from 20 Churchill Road, there appears to be no ongoing maintenance regime of these trees.

RECOMMENDATIONS: - Increase the re-reduction regimes of T4 and T5 from triennial to biennial - Strip all vegetation surrounding the tree pits from T1, T2, T3, T4, T5, T6,T7, T8 and T9 - Remove ST1 and ST3 and poison stumps - Poison the stump of ST2 to make sure it does not grow back or sprout suckers from elsewhere - Review the pruning regime for TG1 (T11) and do not allow for vegetation levels to dramatically increase.

T11

T10

T1	   T2

T3                                                                                                T4

T5                                                                                         T6

T7                                                                                           T8

C1 & H1                                                                                               ST3

ST2                                                                                          ST1