Herritage Statement

Flat 5, Fitzroy Lodge, N6 6LH

__________________________________________________________________

Official List Entry

Heritage Category: Listed Building, Grade II
List Entry Number: 1378989
Date first listed: 10-May-1974
Statutory Address: FITZROY LODGE, THE GROVE
The building or site itself may lie within the boundary of more than one authority.

County: Greater London Authority

District: Camden (London Borough) National Grid Reference: TQ 28221 87458
Details: CAMDEN TQ2887SW THE GROVE 798-1/5/1834 (East side) 10/05/74 Fitzroy Lodge (Formerly Listed as: HARINGEY THE GROVE Fitzroy Lodge)

GV II

Listing NGR: TQ2822187458

Extract from the Conservation Area Appraisal:

Villa. Early C19 in 2-part composition. Stuccoed brick, with hipped slate roof on deep projecting eaves with tall stacks. Main part a square composition 3 windows wide and 2 storeys high, with ground floor windows set within relieving arches. 2 window return to left similarly treated. Continuous sill band at first floor. All windows are sashes with glazing bars; mid-C20 glazed door. To right a projecting wing, 3 bays deep and with one window return, 2 storeys with high attic which has been altered. C20 extension and rooftop conservatory not of special interest. INTERIOR not inspected. Formerly listed in the London Borough of Haringey, was transferred to LB Camden on 1.4.94.

Proposal description and impact on historical fabric of the listed building

Flat 5 is formed from the entire top floor (2nd) of the Southerly section of Fitzroy Lodge and sits within the eaves of the building.

The Southerly section is made up of two parts, the original, Georgian part fronting The Grove and a 1930s addition that sits behind. The bulk of proposed alterations, including the reconfiguration of kitchen, bathroom and wc to create additional shower room, utility area and storage, sits within the 1930s addition. The only alterations within the Georgian section are the stepping of a wall between the bedrooms, to allow for clothes storage in both, and this wall is not thought to be original. There is a short, wider section of this wall (at its Northern end) that is of solid construction and may be original but this is not being altered. Additionally, the doorway to proposed bedroom 2 is being moved marginally to the right (West) to allow for the aforementioned bedroom storage. This wall is also of modern studwork construction and clearly has no historical relevance.

The new bathroom, shower room, kitchen and utility area all utilise existing service runs, including the waste stack that runs internally.